IEEE P802.11
Wireless LANs

| Enhancing BSS Transition Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Kenan Xu | Nortel Networks | 3500 Carling Avenue, Ottawa | +1 613-763-9308 | kenanxu at nortel.com |

Abstract

The current 802.11v D3.02 standard draft fails to regulate all technical aspects required by an effective network-initiated BSS transition process. Amendment proposals (on IEEE 802.11v) are made to enable a streamlined network-initiated BSS transition process when the current AP cannot accept more resource requests, while neighboring APs have abundant unsed radio resource. The resulting network-initiated load shifting process is more effective and robust than an STA-initiated BSS transition process.

This document presents the actual text changes neeed to 802.11v D3.02 in order to enable this enhanced capability.

7.3.1.9 Status Code field

Add a new Status Code to Table 7-23

:

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| <ANA> | Requested TCLAS processing is not supported by the AP |
| <ANA> | The AP has insufficient TCLAS processing resources to satisfy the request |
| <ANA> | The TS has not been created because the request cannot be honored; however, the HC suggests the STA to transition to other BSSs to set up the TS. |
| (<ANA>+1) 31— 65535 | Reserved |

7.4.2.2 ADDTS Response frame format

Add the BSS Transition Candidate List Entries element to Table 7-47:

Table 7-47—ADDTS Response frame body

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | Action |
| 3 | Dialog Token |
| 4 | Status Code |
| 5 | TS Delay |
| 6 | TSPEC |
| 7–n | TCLAS(optional) |
| n + 1 | TCLAS Processing (optional) |
| n + 2 | Schedule |
| n + 3 | BSS Transition Candidate List Entries |

Change this paragraph in clause 7.4.2.2 as shown:

The Dialog Token, TS Delay, TSPEC, TCLAS, and TCLAS Processing, BSS Transition Candidate List Entries fields in this frame are contained in an MLME-ADDTS.response primitive that causes the frame to be sent. The TS Delay information element is present in an ADDTS Response frame only if the status code is set to 47. The BSS Transition Candidate List Entries is present in an ADDTS Response frame only if the status code is set to <ANA>.

7.4.11.9 BSS Transition Management Request frame format

— The Preferred Candidate List Included (bit 0) field indicates whether the BSS transition candidate list

included in this frame is a preferred candidate list or a list of known BSS transition candidates. See

11.20.6.3.

— The Abridged (bit 1) field indicates to the recipient of the frame the intended treatment of all BSSIDs

not listed in the BSS Transition Candidate List. See 11.20.6.3.

— The Disassociation Imminent (bit 2) field indicates whether the STA will be disassociated from the

current AP. See 11.20.6.3.

— The Power Down Included (bit 3) field indicates that the Power Down Duration field is included, the

AP is shutting down and the STA will be disassociated.

10.3.24.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDTS.confirm(

ResultCode,

DialogToken,

TSDelay,

TSPEC,

Schedule,

BSSTransitionCandidateList,

TCLAS,

TCLASProcessing,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETE RS, REJECTED_WITH_SU GGESTED_CHANGES, REJECTED_FOR_DEL AY_PERIOD, TIMEOUT, TRANSMISSION_FAILURE, REJECTED_WITH_SUGGESTED _BSS_TRANSITION, REQUESTED_TCLAS_ NOT_SUPPORTED, TCLAS_RESOURCES_ EXHAUSTED | Indicates the results of the corresponding MLME-ADDTS.request primitive. |
| DialogToken | Integer | As defined in the corresponding MLMEADDTS.request primitive | Specifies a number unique to the QoS Action primitives and frames used in adding (or modifying) the TS. |
| TSDelay | Integer | ≥0 | When the result code is REJECTED_FOR_DELAY_ PERIOD, provides the amount of time a STA should wait before attempting another ADDTS request frame. |
| TSPEC | As defined in frame format | As defined in frame format | Specifies the TS information, traffic characteristics, and QoS requirements of the TS. |
| Schedule | As defined in frame format | As defined in frame format | Specifies the schedule information, service start time, SI, and the specification interval. |
| BSSTransitionCandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.37 | Contains the description of candidate BSS transition APs and their capabilities in section 7.3.2.37. |
| TCLAS (0 or more) | As defined in frame format | As defined in frame format | Specifies the rules and parameters by which an MSDU may be classified to the specified TS. |
| TCLASProcessing | As defined in frame format | As defined in frame format | Specifies how the TCLAS elements are to be processed when there are multiple TCLAS elements. |
| VendorSpecificInfo | A set of information Elements | As defined in 7.3.2.26 | Zero or more information elements. |

For the ResultCode value of SUCCESS, the TSPEC and the optional TCLAS parameters describe the

characteristics of the TS that has been created (or modified); and the specified (nonzero) parameters [with

the exception of Service Start Time, Medium Time, and any possibly unspecified minimum set of

parameters (see 9.9.3.2) in the TSPEC in ADDTS Request frame] exactly match those of the matching

MLME-ADDTS.request primitive.

For other values of ResultCode, no new TS has been created. In the case of REJECTED_WITH_

SUGGESTED_CHANGES, the TSPEC represents an alternative proposal by the HC. A TS is not created

with this definition. If the suggested changes are acceptable to the non-AP STA, it is the responsibility of the

non-AP STA to set up the TS with the suggested changes.

If this is the result of a modification of an existing TS, the status of that TS remains unchanged.

For the ResultCode value of REJECTED_WITH_SUGGESTED_BSS_TRANSITION, the BSSTransitionCandidateList is supplied to facilitate the non-AP STA transition to other APs. The non-AP STA can retry the TS setup process with the newly associated AP once the transition is complete.

10.3.24.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDTS.response (

ResultCode,

DialogToken,

Non-APSTAAddress,

TSDelay,

TSPEC,

Schedule,

BSSTransitionCandidateList

TCLAS,

TCLASProcessing,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | ResultCode Enumeration SUCCESS, INVALID_PARAMETERS, REJECTED_WITH_SUGGESTED_ CHANGES, REJECTED_FOR_DELAY_PERIOD, TIMEOUT, TRANSMISSION_FAILURE, REQUESTED_TCLAS_NOT_SUPPORTED, TCLAS_RESOURCES_EXHAUSTED | Indicates the results of the corresponding MLME-ADDTS.indication primitive. |
| DialogToken | Integer | As defined in the corresponding MLMEADDTS.indication | DialogToken of the matching MLME-ADDTS.indication primitive. |
| Non- APSTAAddress | MACAddress |  | Contains the non-AP STA address of the matching MLME-ADDTS.indication primitive. |
| TSDelay | Integer | ≥0 | When the result code is REJECTED_FOR_DELAY_ PERIOD, provides the amount of time a STA should wait before attempting another ADDTS request. |
| TSPEC | As defined in frame format | As defined in frame format | Specifies the QoS parameters of the TS. |
| Schedule | As defined in frame format | As defined in frame format | Specifies the schedule information, service start time, SI, and the specification interval. |
| BSSTransitionCandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.37 | Contains the description of candidate BSS transition APs and their capabilities in section 7.3.2.37. |
| TCLAS (0 or more) | As defined in frame format | As defined in frame format | Specifies the rules and parameters by which an MSDU may be classified to the specified TS. |
| TCLASProcessing | As defined in frame format | As defined in frame format | Specifies how the TCLAS elements are to be processed when there are multiple TCLAS elements. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

The DialogToken and non-APSTAAddress parameters contain the values from the matching MLMEADDTS.

indication primitive.

If the result code is SUCCESS, the TSPEC and (optional) TCLAS parameters contain the values from the

matching MLME-ADDTS-indication.

If the result code is REJECTED_WITH_SUGGESTED_CHANGES, the TSPEC and TCLAS parameters

represent an alternative proposed TS. The TS, however, is not created. The TSID and direction values within

the TSPEC are as in the matching MLME-ADDTS.indication primitive. The difference may lie in the QoS

(e.g., minimum data rate, mean data rate, and delay bound) values, as a result of admission control

performed at the SME of the HC on the TS requested to be added (or modified) by the non-AP STA. If

sufficient bandwidth is not available, the QoS values may be reduced. In one extreme, the minimum data

rate, mean data rate, and delay bound may be all set to 0, indicating that no QoS is to be provided to this TS.

If the result code is REJECTED_WITH_SUGGESTED_BSS_TRANSITION, the BSSTransitionCandidateList conveys the set of suggested candidate BSSs for the transition. The non-AP STA shall transition to one of the candidate APs in the list as defined in 11.20.6, as if it received a BSS transition management request message. Once the BSS transition is complete, the STA can retry the TS setup process, as defined in 11.4.4.

11.4.4 TS setup

Figure 11-8 shows the sequence of messages occurring at a TS setup. This message sequence in this figure

and in the subsequent figures does not show the acknowledgment.

Figure 11-8—TS setup (omitted)

The non-AP STA SME decides that a TS needs to be created. How it does this, and how it selects the TSPEC

parameters, is beyond the scope of this standard. The SME generates an MLME-ADDTS.request primitive

containing a TSPEC. A TSPEC may also be generated autonomously by the MAC without any initiation by

the SME. However, if a TSPEC is generated subsequently by the SME, the TSPEC containing the same

TSID generated autonomously by the MAC shall be overridden. If one or more TSPECs are initiated by the

SME, the autonomous TSPEC shall be terminated.

The non-AP STA MAC transmits the TSPEC in an ADDTS Request frame to the HC and starts a response

timer called ADDTS timer of duration dot11ADDTSResponseTimout.

The HC MAC receives this management frame and generates an MLME-ADDTS.indication primitive to its

SME containing the TSPEC.

The SME in the HC decides whether to admit the TSPEC as specified, refuse the TSPEC, not admit but

suggest an alternative TSPEC, or not admit but suggest transition to other BSSs if non-AP STA indicates that it supports BSS Transition capability in the Extended Capability information element. How the SME decides on the candidate BSS is beyond the scope of this standard. The SME then generates an MLME-ADDTS.response primitive containing the TSPEC and a ResultCode value. The contents of the TSPEC and Status fields contain values specified in 10.3.24.4.2.

The HC MAC transmits an ADDTS Response frame containing this TSPEC and status. The encoding of the

ResultCode values to Status Code field values is defined in Table 11-2.

Table 11-2—Encoding of ResultCode to Status Code field value

| ResultCode | Status code |
| --- | --- |
| REQUESTED_TCLAS_NOT_SUPPORTED | <ANA> |
| TCLAS_RESOURCES_EXHAUSTED | <ANA+1> |
| REJECTED_WITH_SUGGESTED_BSS_TRANSITION | <ANA+2> |

The non-AP STA MAC receives this management frame and cancels its ADDTS timer. It generates an

MLME-ADDTS.confirm primitive to its SME containing the TSPEC and status.

The non-AP STA SME decides whether the response meets its needs. How it does this is beyond the scope

of this standard. If the result code is SUCCESS, the TS enters into an active state. Otherwise, the non-AP STA can try to transition to other BSSs or repeat the whole process using the same TSID and direction and a modified TSPEC until the non-AP STA SME decides that the granted TXOP is adequate or inadequate and cannot be improved. If the non-AP STA is recommended to transition to other BSSs, it shall transition according to the process defined in 11.20.6, as if it received an unsolicited BSS Transition Management Request frame with Abridge bit set as 0 and Preferred Candidate List Included bit set as 1. Once the transition is completed, the non-AP STA can proceed with a TS setup process with the new HC.

A HC can update the suggested BSS Transition Candidate List Entries by sending an unsolicted BSS Transition Management Request frame immediately after the ADDTS Response frame with the status code set as  REJECTED_WITH_SUGGESTED_BSS_TRANSITION.  Upon receiving the BSS Transition Management Request frame, the non-AP STA shall disregard the previous BSS Transiton Candidate List Entries in the MLME-ADDTS.response received from the same HC.

The parameters that are set for a TS can be renegotiated in a similar manner, when such a request is

generated by the SME through ADDTS.request primitive. When a request for the modification of the TS

parameters is accepted by the HC, it shall reset both the suspension interval and the inactivity interval

timers.

If the AP/HC grants a TXOP for an ADDTS Request frame with the Ack Policy subfield set to Block Ack

and the Direction field set to either downlink or bidirectional, then it shall initiate a Block Ack negotiation

by sending an ADDBA Request frame to the non-AP STA that originated the ADDTS Request frame. If a

non-AP STA is granted a TXOP for an ADDTS Request frame with the Ack Policy subfield set to Block

Ack and the Direction field set to other than downlink, then it shall initiate a Block Ack negotiation by

sending an ADDBA Request frame to the recipient of the TS.

References:

IEEE Std. 802.11-2007

IEEE 802.11v D3.02, September 2008

11-08-1294-00-000v-Enhancing-BSS-Transition-Management.ppt, this is the corresponding .ppt file