IEEE P802.11
Wireless LANs

| Enhancing BSS Transition Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Guyves Achtari | Nortel Networks | 3500 Carling Avenue, Ottawa | +1 613-763-2762 | gachtari@nortel.com |

Abstract

The current 802.11v D4.01 standard draft fails to regulate all technical aspects required by an effective network-initiated BSS transition process. Amendment proposals (on IEEE802.11v) are made to enable a streamlined network-initiated BSS transition indication process when the current AP can not accept more resource requests, while neighboring APs have abundant radio resource left unused. The resulting network-initiated load shifting indication process is more effective and robust than a transition solely initiated by the STA.

7.3.1.9 Status Code field

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| <ANA> | Requested TCLAS processing is not supported by the AP |
| <ANA> | The AP has insufficient TCLAS processing resources to satisfy the request |
| <ANA> | The TS has not been created because the request cannot be honored; however, the HC suggests the STA to transition to other BSSs to setup the TS. |
| (<ANA>+1) 31— 65535 | Reserved |

10.3.24.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDTS.confirm(

ResultCode,

DialogToken,

TSDelay,

TSPEC,

Schedule,

TCLAS,

TCLASProcessing,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETE RS, REJECTED_WITH_SU GGESTED_CHANGES, REJECTED_FOR_DEL AY_PERIOD, TIMEOUT, TRANSMISSION_FAILURE, REJECTED_WITH_SUGGESTED _BSS_TRANSITION REQUESTED_TCLAS_ NOT_SUPPORTED, TCLAS_RESOURCES_ EXHAUSTED | Indicates the results of the corresponding MLME-ADDTS.request primitive. |
| DialogToken | Integer | As defined in the corresponding MLMEADDTS.request primitive | Specifies a number unique to the QoS Action primitives and frames used in adding (or modifying) the TS. |
| TSDelay | Integer | ≥0 | When the result code is REJECTED_FOR_DELAY_ PERIOD, provides the amount of time a STA should wait before attempting another ADDTS request frame. |
| TSPEC | As defined in frame format | As defined in frame format | Specifies the TS information, traffic characteristics, and QoS requirements of the TS. |
| Schedule | As defined in frame format | As defined in frame format | Specifies the schedule information, service start time, SI, and the specification interval. |
| TCLAS (0 or more) | As defined in frame format | As defined in frame format | Specifies the rules and parameters by which an MSDU may be classified to the specified TS. |
| TCLASProcessing | As defined in frame format | As defined in frame format | Specifies how the TCLAS elements are to be processed when there are multiple TCLAS elements. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

For the ResultCode value of SUCCESS, the TSPEC and the optional TCLAS parameters describe the characteristics of the TS that has been created (or modified); and the specified (nonzero) parameters [with the exception of Service Start Time, Medium Time, and any possibly unspecified minimum set of parameters (see 9.9.3.2) in the TSPEC in ADDTS Request frame] exactly match those of the matching MLME-ADDTS.request primitive.

For other values of ResultCode, no new TS has been created. In the case of REJECTED_WITH_SUGGESTED_CHANGES, the TSPEC represents an alternative proposal by the HC. A TS is not created with this definition. If the suggested changes are acceptable to the non-AP STA, it is the responsibility of the non-AP STA to set up the TS with the suggested changes. If this is the result of a modification of an existing TS, the status of that TS remains unchanged.

In the case of REJECTED_WITH_SUGGESTED_BSS_TRANSITION,  non-AP STA should retry TS setup process with the newly associated AP once the transition is done.

10.3.24.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDTS.response (

ResultCode,

DialogToken,

Non-APSTAAddress,

TSDelay,

TSPEC,

Schedule,

TCLAS,

TCLASProcessing,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | ResultCode Enumeration SUCCESS, INVALID_PARAMETERS, REJECTED_WITH_SUGGESTED_ CHANGES, REJECTED_FOR_DELAY_PERIOD, TIMEOUT, TRANSMISSION_FAILURE, REQUESTED_TCLAS_NOT_SUPPORTED, TCLAS_RESOURCES_EXHAUSTED, REJECTED_WITH_SUGGESTED _BSS_TRANSITION | Indicates the results of the corresponding MLME-ADDTS.indication primitive. |
| DialogToken | Integer | As defined in the corresponding MLMEADDTS.indication | DialogToken of the matching MLME-ADDTS.indication primitive. |
| Non- APSTAAddress | MACAddress |  | Contains the non-AP STA address of the matching MLME-ADDTS.indication primitive. |
| TSDelay | Integer | ≥0 | When the result code is REJECTED_FOR_DELAY_ PERIOD, provides the amount of time a STA should wait before attempting another ADDTS request. |
| TSPEC | As defined in frame format | As defined in frame format | Specifies the QoS parameters of the TS. |
| Schedule | As defined in frame format | As defined in frame format | Specifies the schedule information, service start time, SI, and the specification interval. |
| TCLAS (0 or more) | As defined in frame format | As defined in frame format | Specifies the rules and parameters by which an MSDU may be classified to the specified TS. |
| TCLASProcessing | As defined in frame format | As defined in frame format | Specifies how the TCLAS elements are to be processed when there are multiple TCLAS elements. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

The DialogToken and non-APSTAAddress parameters contain the values from the matching MLMEADDTS.indication primitive.

If the result code is SUCCESS, the TSPEC and (optional) TCLAS parameters contain the values from the matching MLME-ADDTS-indication.

If the result code is REJECTED_WITH_SUGGESTED_CHANGES, the TSPEC and TCLAS parameters represent alternative proposed TS. The TS, however, is not created. The TSID and direction values within the TSPEC are as in the matching MLME-ADDTS.indication primitive. The difference may lie in the QoS

(e.g., minimum data rate, mean data rate, and delay bound) values, as a result of admission control performed at the SME of the HC on the TS requested to be added (or modified) by the non-AP STA. If sufficient bandwidth is not available, the QoS values may be reduced. In one extreme, the minimum data rate, mean data rate, and delay bound may be all set to 0, indicating that no QoS is to be provided to this TS.

If the result code is REJECTED_WITH_SUGGESTED_BSS_TRANSITION, the non-AP STA should initiate a transition query as defined in 11.20.6. Once the transition is completed, the STA should retry TS setup process, as defined in 11.4.4.

11.4.4 TS setup

Figure 11-8 shows the sequence of messages occurring at a TS setup. This message sequence in this figure and in the subsequent figures does not show the acknowledgment.

Figure 11-8—TS setup (omitted)

The non-AP STA SME decides that a TS needs to be created. How it does this, and how it selects the TSPEC parameters, is beyond the scope of this standard. The SME generates an MLME-ADDTS.request primitive containing a TSPEC. A TSPEC may also be generated autonomously by the MAC without any initiation by the SME. However, if a TSPEC is generated subsequently by the SME, the TSPEC containing the same

TSID generated autonomously by the MAC shall be overridden. If one or more TSPECs are initiated by the

SME, the autonomous TSPEC shall be terminated.

The non-AP STA MAC transmits the TSPEC in an ADDTS Request frame to the HC and starts a response timer called ADDTS timer of duration dot11ADDTSResponseTimout.

The HC MAC receives this management frame and generates an MLME-ADDTS.indication primitive to its SME containing the TSPEC.

The SME in the HC decides whether to admit the TSPEC as specified, refuse the TSPEC, not admit but suggest an alternative TSPEC, or not admit but suggest transition to other BSSs if non-AP STA indicates that it supports such capability in the Extended Capability information element. The SME then generates an MLME-ADDTS.response primitive containing the TSPEC and a ResultCode value. The contents of the TSPEC and Status fields contain values specified in 10.3.24.4.2.

The HC MAC transmits an ADDTS Response frame containing this TSPEC and status. The encoding of the ResultCode values to Status Code field values is defined in Table 11-2.

Table 11-2— Encoding of ResultCode to Status Code field value

| ResultCode | Status code |
| --- | --- |
| REQUESTED_TCLAS_NOT_SUPPORTED | <ANA> |
| TCLAS_RESOURCES_EXHAUSTED | <ANA+1> |
| REJECTED_WITH_SUGGESTED_BSS_TRANSITION | <ANA+2> |

The non-AP STA MAC receives this management frame and cancels its ADDTS timer. It generates an MLME-ADDTS.confirm primitive to its SME containing the TSPEC and status.

The non-AP STA SME decides whether the response meets its needs. How it does this is beyond the scope

of this standard. If the result code is SUCCESS, the TS enters into an active state. If the result code is REJECTED_WITH_SUGGESTED_BSS_TRANSITION, the non-AP STA may try to transition to other BSS. Otherwise the whole process using the same TSID and direction and a modified TSPEC until the non-AP STA SME decides that the granted TXOP is adequate or inadequate and cannot be improved. In case that the non-AP STA is recommended to transition to other BSSs, it should do so according to the process defined in 11.20.6, Once the transition is completed, it should proceed with a TS setup process with the new HC.

The parameters that are set for a TS can be renegotiated in a similar manner, when such a request is generated by the SME through ADDTS.request primitive. When a request for the modification of the TS parameters is accepted by the HC, it shall reset both the suspension interval and the inactivity interval timers.

If the AP/HC grants a TXOP for an ADDTS Request frame with the Ack Policy subfield set to Block Ack and the Direction field set to either downlink or bidirectional, then it shall initiate a Block Ack negotiation by sending an ADDBA Request frame to the non-AP STA that originated the ADDTS Request frame. If a non-AP STA is granted a TXOP for an ADDTS Request frame with the Ack Policy subfield set to Block

Ack and the Direction field set to other than downlink, then it shall initiate a Block Ack negotiation by sending an ADDBA Request frame to the recipient of the TS.