IEEE P802.11
Wireless LANs

| LB-137 Vendor Specific List Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This proposal addresses a number of comments related to the vendor specific list which resulted from LB137.  This proposal addresses the following CIDs: 4025, 4026, 4156, 4231, 4358, 4400, 4401, 4409, 4410, 4411, 4495, 4496, 4497, 4498, 4562, 4563, 4669, 4757, 4758, 4759, 4760 and 4761.

This document is based on 802.11u-d4.01.

7	Frame formats

7.3	Management frame body components

Change the following text as shown:

7.3.3	Native query protocol elements

Native Query Protocol elements are defined to have a common format consisting of a two-octet Info ID field, a two-octet length field encoded following the conventions given in 7.1.1., and a variable-length element-specific information field. Each element is assigned a unique Info ID as defined in this standard. See Figure 7-95ay.

Insert the following text as shown; this will result in clause renumbering which affects the right-most column in the table:

Table 7-43w—Native Query Protocol info ID definitions

| Info name | Info ID | Native info element (clause) |
| --- | --- | --- |
| Reserved | 0-255 | n/a |
| Capability List Information | 256 | 7.3.3.1 |
| Venue Name Information | 257 | 7.3.3.2 |
| Emergency Call Number Information | 258 | 7.3.3.3 |
| Emergency Public Network Access Information | 259 | 7.3.3.4 |
| Network Authentication Type Information | 260 | 7.3.3.5 |
| Roaming Consortium List | 261 | 7.3.3.6 |
| AP Date, Time and Time Zone information | 262 | 7.3.3.9 |
| IP Address Type Availability | 263 | 7.3.3.10 |
| NAI Realm | 264 | 7.3.3.11 |
| Reserved | 262-56796 | n/a |
| Native Query Protocol vendor specific query | 56797 | 7.3.3.7 |
| Native Query Protocol vendor-specific response | 56798 | 7.3.3.8 |
| Reserved | 56798-65535 | n/a |

Change the following text as shown:

7.3.3.7	Native Query Protocol vendor-specific query

The Native Query Protocol vendor-specific query is used to query for information not defined in this standard within a single defined format, so that reserved Info IDs are not usurped for nonstandard purposes and interoperability is more easily achieved in the presence of nonstandard information. The element is in the format shown in Figure 7-95bk

Change the following figure as shown:

|  | Info ID | Length |  | OUI | Vendor-specific content |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 |  | 3 | n |

Figure 7-95bk—Native Query Protocol vendor-specific query

Insert the following text as shown:

The Info ID field is equal to the value in Table 7-43w corresponding to the Native Query Protocol vendor specific query.

The value of the Length field is 3 plus the number of octets in the Vendor-specific content field.

The value of the OUI field is a public OUI assigned by the IEEE that is 3 octets in length.

The Vendor-specific content field is content that has been defined by the entity defined by the OUI field.  The maximum number of octets in the Vendor-specific content fields is equal to the maximum MMPDU size minus 7.

Insert the following sub-clause, renumbering as necessary:

7.3.3.8	Native Query Protocol vendor-specific response

The Native Query Protocol vendor-specific response is used to carry information not defined in this standard within a single defined format, so that reserved Info IDs are not usurped for nonstandard purposes and so that interoperability is more easily achieved in the presence of nonstandard information. The element is in the format shown in Figure 7-95bk1

|  | Info ID | Length | Status Code | OUI | Vendor-specific content |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | 3 | n |

Figure 7-95bk1—Native Query Protocol vendor-specific response

The Info ID field is equal to the value in Table 7-43w corresponding to the Native Query Protocol vendor specific response.

The value of the Length field is 5 plus the number of octets in the Vendor-specific content field.

The status code is a 2-octect field drawn from Table 7-23.

The value of the OUI field is a public OUI assigned by the IEEE that is 3 octets in length.

The Vendor-specific content field is content that has been defined by the entity defined by the OUI field.  The maximum number of octets in the Vendor-specific content fields is equal to the maximum MMPDU size minus 9.