IEEE P802.11
Wireless LANs

| LB137: QoSMap Primitives Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This proposal addresses a number of comments related to the QosMap primitives which resulted from LB137.  This proposal addresses the following CIDs: 4257, 4383, 4678 and 4783.

In addition, the proposal updates the MLME-ASSOCIATE.response/confirm and MLME-REASSOCIATE.response/confirm primitives with the QoSMap element.  This should have been changed in the submissions which generated 802.11u-d4.0.

This document is based on 802.11u-d4.01.

10.	Layer management

10.3	MLME SAP interface

10.3.6.2 MLME-ASSOCIATE.confirm

10.3.6.2.2 Semantics of the service primitive

Insert text into  the primitive parameter list as shown:

MLME-ASSOCIATE.confirm 	(
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,
Content of FT Authentication Information Elements, 
SupportedRegulatoryClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
QoS Map Set
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.6.4 MLME-ASSOCIATE.response

10.3.6.4.2 Semantics of the service primitive

Insert text into  the primitive parameter list as shown:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
Content of FT Authentication Information Elements
SupportedRegulatoryClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
QoS Map Set
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.7.2 MLME-REASSOCIATE.confirm

10.3.7.2.2 Semantics of the service primitive

Insert text into  the primitive parameter list as shown:

MLME-REASSOCIATE.confirm	(
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,
Content of FT Authentication Information Elements, 
SupportedRegulatoryClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
QoS Map Set
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.7.4 MLME-REASSOCIATE.response

10.3.7.4.2 Semantics of the service primitive

Insert text into  the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
Content of FT Authentication Information Elements
SupportedRegulatoryClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
QoS Map Set
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

Change the text as shown below:

10.3.40	QoS Map Set element management

The QoS Map Set element is provided to non-AP STAs in (Re)-association response frames. However, if the SME of an AP detects a change of the QoS Map information while one or more non-AP STAs are associated to the BSS, then the AP’s SME may transmit an un-solicited QoS Map Set element to associated STAs. The AP’s SME invokes the MLME-QoSMap.request primitive to transmit individually-addressed frames containing a QoS Map Set element to associated STAs.  When a non-AP STA receives such unsolicited QoS Map information, its MLME generates a MLME-QoSMap.indication to the STA’s SME.  In turn, the SME should take appropriate action, e.g., initiate an ADDTS or DELTS if admission control changes are necessary.

10.3.40.2	MLME-QoSMap.indication

10.3.40.2.1	Function

This primitive reports the QoS Mapping information sent from the AP to the non-AP STA. It is valid at a non-AP STA.

10.3.40.2.2	Semantics of the service primitive

The primitive parameters are as follows:

MLME-QoSMap.indication	(

DialogToken
QoS Map Set
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 | The dialog token to identify the QoS Map Set message transaction. |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.40.2.3	When generated

The primitive is generated when the non-AP STA receives a QoS Map Set element in an un-solicited QoS Map Configure action frame from the AP.

The SME of the non-AP STA should use the information to decide proper actions. For example, an ADDTS or DELTS procedure should be activated if the QoS Map information indicates a change in the admission control.

10.3.40.2.4	Effect of receipt

The non-AP STA operates according to the procedures defined in Error: Reference source not found.

Renumber this clause as shown and resequence (i.e., move) the text appropriately:

10.3.40.1	MLME-QoSMap.request

10.3.40.1.1	Function

This primitive is used by an AP to transmit an un-solicited QoS Map Set to a specified non-AP STA MAC entity. The specified non-AP STA MAC address is an individual MAC address.

10.3.40.1.2	Semantics of the service primitive

The primitive parameters are as follows:

MLME-QoSMap.request 	(

Non-APSTAAddress,
DialogToken,
QoS Map Set
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which query message is received. |
| Dialog Token | Integer | 0 | The dialog token to identify the QoS Map Set message transaction. |
| QoS Map Set | As defined in frame format | As defined in 7.3.2.64 | Specifies the QoS Map Set the non-AP STA should use. |

10.3.40.1.3	When generated

This primitive is generated by the MLME at the AP as a result of any change in the AP’s QoS Map configuration.

10.3.40.1.4	Effect of receipt

The primitive causes the MAC entity at the AP to send a QoS Map Configure frame in a QoS Map Configure action frame to the non-AP STA.