IEEE P802.11
Wireless LANs

| Proposed resolutions to LB133 CID-1239 and CID-1257 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Qi Wang | Broadcom Corporation | 150 Mathilda Place Sunnyvale, CA 94086, USA | +1-408-922-9798 | qi.wang@broadcom.com |
| Dorothy Stanley | Aruba Networks | 1332 Crossman Ave Sunnyvale, CA 94089, USA | +1-630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document proposes comment resolutions to LB133 CID-1239 and CID-1257. The proposed resolutions intend to clarify the status of mandatory versus optional requirement for each feature defined in the 802.11v amendment.

This document proposes text modification to 11v draft standard 3.02.  The proposed changes are indicated below:

- Insert the following paragraph:

5.2.11.19 STA Statistics (Triggered)

The STA Statistic (Triggered) report enables a STA statistics report when the statistics of interst reach a predefined threshold.

11.1.2.3a Multiple BSSIDs procedures

The implementation of BSSID is optional for a WNM STA.  A STA that implements Multiple BSSIDs  has the MIB attribute dot11MgmtOptionMultiBSSIDImplemented set to true. When dot11MgmtOptionMultiBSSIDImplemented is true, dot11WirelessManagementImplemented shall be true.

A STA that has a value of true for the MIB attribute dot11MgmtOptionMultiBSSIDEnabled is defined as a STA that supports Multiple BSSIDs. A STA for which the MIB attribute dot11MgmtOptionMultiBSSIDEnabled is set to true shall set the Multiple BSSID field of the Extended Capabilities element to 1.

- Note to the editor: please use the term “Multiple BSSIDs” consistently throughout the 11v spec.

11.2.1.4a FMS power management

The implementation of FMS is optional for a WNM STA. A STA that implements Flexible Multicast Service  has the MIB attribute dot11MgmtOptionFMSImplemented set to true. When dot11MgmtOptionFMSImplemented is true, dot11WirelessManagementImplemented shall be true.  When dot11MgmtOptionFMSEnabled is set to true, the STA supports FMS and it is possible to create different delivery intervals for different group addressed streams.

11.2.1.13TIM broadcast

The implementation of TIM Broadcast is optional for a WNM STA. A STA that implements TIM Broadcast has the MIB attribute dot11MgmtOptionTIMBroadcastImplemented set to true. When dot11MgmtOptionTIMBroadcastImplemented is true, dot11WirelessManagementImplemented shall be true.

A STA that has a value of true for the MIB attribute dot11MgmtOptionTIMBroadcastEnabled is defined as a STA that supports TIM Broadcast. A STA for which the MIB attribute dot11MgmtOptionTIMBroadcastEnabled is set to true shall set the TIM Broadcast field of the Extended Capabilities element to 1.

- 11.2.2.1 WNM-Sleep mode capability

The implementation of WNM-Sleep mode is optional for a WNM STA.  A STA that implements WNM-Sleep mode  has the MIB attribute dot11MgmtOptionWNMSleepModeImplemented set to true. When dot11MgmtOptionWNMSleepModeImplemented is true, dot11WirelessManagementImplemented shall be true. A STA where the MIB attribute dot11MgmtOptionWNMSleepModeEnabled is set to true is defined as a STA that supports WNM-Sleep mode. A STA supporting WNM-Sleep mode shall set the WNM-Sleep Mode field of the Extended Capabilities element to 1. When dot11MgmtOptionWNMSleepModeEnabled is true, dot11MgmtOptionTFSEnabled shall be true.

11.10.7 Triggered autonomous reporting

Triggered autonomous reporting is defined only for the Transmit Stream/Category Measurement measurement type – see 11.10.8.8. When dot11MgmtOptionDiagnosticsEnabled is set to true, triggered autonomous reporting is used for Multicast Diagnostics (11.20.2). When dot11MgmtOptionTriggerSTAStatisticsEnabled is set to true, triggered autonomous reporting is used for STA Statistics Reports (11.10.8.5).

11.10.8.5 STA statistics report

- Insert the following at the end of 11.10.8.5:

A STA may request that a STA Statistics report be sent when statistics of interest reach a threshold as defined in the Measurement Request element of the STA Statistics Request frame (see 7.3.2.21.8). This is termed a triggered STA Statistics measurement.  The implementation of is Triggered STA Statistics Reporting is optional for a WNM STA. A STA that implements Triggered STA Statistics Reporting has the MIB attribute dot11MgmtOptionTriggerSTAStatisticsImplemented set to true. When dot11MgmtOptionTriggerSTAStatisticsImplemented is true, dot11WirelessManagementImplemented shall be true.

A triggered STA Statistic measurement shall be requested by setting the Enable and Report bits to 1 within a Measurement Request element containing the STA Statistics Measurement Type. The Measurement Request field shall contain a STA Statistics Request with the trigger conditions specified in the Triggered Reporting subelement, as defined in 7.3.2.21.8. One or more trigger conditions may be set with specified thresholds. See 11.10.7. To prevent generation of too many triggered reports, the value of the Trigger Timeout field shall be set to a value greater or equal to the value of dot11MinTriggerTimeout. If the value of the Trigger Timeout field is less than the value of dot11MinTriggerTimout, the STA shall reject the measurement request and respond with a report where the Measurement Report Mode field is set to "Incapable".

11.20.1 Wireless network management dependencies

When dot11WirelessManagementImplemented is set to true, the STA is a WNM STA and dot11ExtendedChannelSwitchEnabled and dot11RadioMeasurementEnabled shall be true.

When dot11WirelessManagementImplemented is true, and one or more of bits <ANA-first> to <ANA-last> in the Extended Capabilities element are set to 1, the Extended Capabilities element shall be included in Beacon frames, Association Request and Response frames, Reassociation Request and Response frames, and Probe Request and Response frames. When dot11WirelessManagementImplemented is true, for each bit <ANA-first> to <ANA-last> in the received Extended Capabilities element that is set to 0, a STA shall not request the corresponding feature from the sending STA. A STA receiving a request for a Wireless Network Management feature from another STA shall reject the request if the receiving STA has not advertised support for the corresponding Wireless Network Management feature.

- 11.20.4.1 Location track configuration procedures

11.20.4.2 Location track notification procedures

The implementation of Location Track Notification is optional for a WNM STA.  A STA that implements Location Track Notification has the MIB attribute dot11MgmtOptionLocationTrackNotificationImplemented set to true. When dot11MgmtOptionLocationTrackNotificationImplemented is true, dot11WirelessManagementImplemented shall be true. A STA with a value of true for the MIB attribute dot11MgmtOptionLocationTrackNotificationEnabled is defined as a STA that supports Location Track Notification.  When Location Track Notification is supported, a STA configured by another STA as described in the previous subclause shall transmit Location Track Notification frames as shown in the informative diagram in Figure v114. The implementation of Motion Detection or the Time of Departure reporting is optional for a WNM STA.  A STA that implements Motion Detection has the MIB attribute dot11MgmtOptionMotionDectectionImplemented set to true. When dot11MgmtOptionMotionDetectionImplemented is true, dot11WirelessManagementImplemented shall be true. A STA with a value of true for the MIB attribute dot11MgmtOptionMotionDetectionEnabled is defined as a STA that supports Motion Detection. A STA that implements Time of Departure has the MIB attribute dot11MgmtOptionMotionTODImplemented set to true. When dot11MgmtOptionTODImplemented is true, dot11WirelessManagementImplemented shall be true. A STA with a value of true for the MIB attribute dot11MgmtOptionTODEnabled is defined as a STA that supports Time of Departure for location tracking.

11.20.5 Timing Measurement procedure

The implementation of Timing Measurement is optional for a WNM STA. A STA that implements Timing Measurement has the MIB attribute dot11MgmtOptionTimingMsmtImplemented set to true. When dot11MgmtOptionTimingMsmtImplemented is true, dot11WirelessManagementImplemented shall be true. If dot11MgmtOptionTimingMsmtEnabled is set to true, the Timing Measurement field in the Extended Capabilities element shall be set to 1 and the STA supports the timing measurement procedure. A STA provides timing measurement values to a peer STA using the Timing Measurement public action frame. A receiving STA uses the timing values in the Timing Measurement public action frame to:

a)	estimate the location of the STA transmitting the action frame and

b)	estimate the offset of the local clock with respect to the clock of the STA transmitting the action frame.

- 11.20.8 Co-located interference reporting

Co-located interference might cause degradation of 802.11 STA performance either periodically or continuously. The co-located interference reporting allows a requesting STA to receive information concerning the co-located interference being experienced by another STA on the operating channel. Such interference may be due to an interaction between radios where a reporting STA is co-located with another radio device. Co-located interference information can be used by the requesting STA to manage communication to the reporting STA such that the effect of the interference is limited.

The implementation of Colocated interference reporting is optional for a WNM STA. A STA that implements Co-located Interference reporting has the MIB attribute dot11MgmtOptionCOLOINTImplemented set to true. When dot11MgmtOptionCOLOINTImplemented is true, dot11WirelessManagementImplemented shall be true. A STA that has a value of true for the MIB attribute dot11MgmtOptionCOLOINTReportingEnabled is defined as a STA that supports co-located interference reporting. A STA for which the MIB attribute dot11MgmtOptionCLIReportingEnabled is set to true shall set the Co-located Interference Reporting field of the Extended Capabilities element to 1.

- 11.20.9 QoS Traffic Capability procedure

The implementation of QoS traffic capability is optional for a WNM STA. A STA that implements QoS traffic capability has the MIB attribute dot11MgmtOptionTrafficGenerationImplemented set to true. When dot11MgmtOptionTrafficGenerationImplemented is true, dot11WirelessManagementImplemented shall be true. If the MIB attribute dot11MgmtOptionTrafficGenerationEnabled is set to true, a non-AP QoS STA supports the QoS Traffic Capability and shall include the QoS Traffic Capability element in an Association Request frame or in a Reassociation Request frame when it is sending such a frame to associate or reassociate with an AP. If there is any change in QoS Traffic Capability Flags while associated with an AP, the non-AP STA shall send a QoS Traffic Capability Update frame (see 7.4.11.23) including the updated QoS Traffic Capability Flags to the AP.

11.20.10 AC station count

The implementation of AC station count is optional for a WNM STA,  A STA that implements AC station count has the MIB attribute dot11MgmtOptionStationCountImplemented set to true. When dot11MgmtOptionStationCountImplemented is true, dot11WirelessManagementImplemented shall be true.When dot11MgmtOptionACStationCountEnabled is set to true, the STA shall set the AC Station Count field to 1 in the Extended Capabilities element to indicate that the STA supports the AC Station Count capability. When dot11MgmtOptionACStationCountEnabled is set to false, the STA shall set the AC Station Count field in the Extended Capabilities element to 0 to indicate that the STA does not support this capability.

If the MIB attribute dot11MgmtOptionACStationCountImplemented is set to true, a QoS AP shall be able to construct the QoS Traffic Capability Bitmask and AC STA Count list as specified in 7.3.2.73. The AP shall construct the STA Count List value based on the UP-to-AC mappings as defined in Table 9-1, the QoS Traffic Capability Bitmask/Flags of the non-AP STAs that are currently associated with it, and additional information. If the MIB attribute dot11MgmtOptionACStationCountEnabled is set to true, a QoS AP shall include the QoS Traffic Capability element in a Probe Response frame and in a Beacon frame.

If the MIB attribute dot11MgmtOptionACStationCountEnabled is set to true, a non-AP QoS STA may use the STA Count field values as one of the factors when determining association, reassociation, and the BSS transition. If the MIB attribute dot11MgmtOptionACStationCountEnabled is set to false, a non-AP QoS STA shall not use the STA Count field values as one of the factors when determining association, reassociation, and the BSS transition.

- 11.20.11 TFS procedures

- 11.20.11.1 TFS capability

The implementation of TFS is optional for a WNM STA. A STA that implements TFS has the MIB attribute dot11MgmtOptionTFSImplemented set to true. When dot11MgmtOptionTFSImplemented is true, dot11WirelessManagementImplemented shall be true. A STA that has a value of true for the MIB attribute dot11MgmtOptionTFSEnabled is defined as a STA that supports TFS. A STA for which the MIB attribute dot11MgmtOptionTFSEnabled is set to true shall set the TFS field of the Extended Capabilities element to 1.

A STA with a value of true for the MIB attribute dot11MgmtTFSEnabled may send a TFS Request, TFS Response or TFS Notify frame to a STA within the same infrastructure BSS whose last received Extended Capabilities element contained a value of 1 for the TFS bit in the Capabilities field. The Traffic Filtering service is not supported in an IBSS.

- 11.20.13 Proxy ARP service

The implementation of Proxy ARP Service is optional for a WNM STA,  A STA that implements the Proxy ARP Service has the MIB attribute dot11MgmtOptionProxyARPImplemented set to true. When dot11MgmtOptionProxyARPImplemented is true, dot11WirelessManagementImplemented shall be true. When dot11MgmtOptionProxyARPEnabled is set to true, the Proxy ARP Service bit in the Extended Capabilities field is set to 1 to indicate that the AP supports the Proxy ARP Service. When dot11MgmtOptionProxyARPEnabled is set to false, the Proxy ARP Service bit is set to 0 to indicate that the AP does not support the Proxy ARP Service.

When the AP sets the Proxy ARP field to 1 in the Extended Capabilities element, the AP shall maintain a Hardware Address to Internet Address mapping for each associated station, and shall update the mapping when the Internet Address of the associated station changes. When the IPv4 address being resolved in the ARP request packet is used by a non-AP STA currently associated to the BSS, the Proxy ARP service shall respond to the request on behalf of the non-AP STA [RFC 925], unless the packet updates the Hardware Address to Internet Address mapping. The Proxy ARP service also shall respond to Internet Control Message Protocol version 6 (ICMPv6) Neighbor Discovery packets on behalf of the non-AP STA, to support IPv6 services [RFC 2461], unless the packet updates the Hardware Address to Internet Address mapping.

- 11.20.14 Channel usage procedures

Channel Usage information is a set of channels provided by an AP to non-AP STAs for operation of a non-infrastructure network. The Channel Usage information provided by the AP to the non-AP STA is to advise the STA how to co-exist with the infrastructure network.

The implementation of Channel Usage is optional for a WNM STA. A STA that implements Channel Usage has the MIB attribute dot11MgmtOptionChannelUsageImplemented set to true. When dot11MgmtOptionChannelUsageImplemented is true, dot11WirelessManagementImplemented shall be true.A STA that has a value of true for the MIB attribute dot11MgmtOptionChannelUsagEnabled is defined as a STA that supports Channel Usage. A STA for which the MIB attribute dot11MgmtOptionChannelUsageEnabled is true shall set the Channel Usage field of the Extended Capabilities element to 1.

- A.4 PICS proforma–IEEE Std. 802.11, 2007 Edition

- A.4.3 IUT configuration

- Insert this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFv | Is Wireless Network Management supported? |  | (CF8 & CF11 & CF13 & CF15 & DSE5 & DSE6 & DSE7 & DSE8 & DSE9 ):O | Yes, No, N/A |

- Insert this new clause after A.4.20:

- EDITORIAL NOTE—last clause in 802.11k-D.70 is A.4.17.

- A.4.21 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM1 | Extended Capabilities element | 7.3.2.27 | CFv:M | Yes, No, N/A |
| WNM2 | STA Statistics (Triggered) and Multicast Diagnostics | 11,10.7 | CFv:M | Yes, No, N/A |
| WNM2.1 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.2 | Triggered STA Statistics | 7.3.2.21.8, 7.4.6.1 7.3.2.22.8, 7.4.6.2 11.10.8.5 | CFv:O | Yes, No, N/A |
| WNM2.3 | Multicast Diagnostics | 7.3.2.21.12, 7.4.6.1, 7.4.6.2 7.3.2.22.11, 11.10.16 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request frame | 7.3.2.62, 7.4.11.2 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report frame | 7.3.2.63, 7.4.11.3 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.20.3 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request frame | 7.3.2.64, 7.4.11.4 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report frame | 7.3.2.65, 7.4.11.5 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4.3 | Configuration Profile Diagnostic Type | 7.3.2.65.3, 11.20.3.2 | CFv:M | Yes, No, N/A |
| WNM4.4 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.65.2, 11.20.3.3 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.64.2, 7.3.2.65.4, 11.20.3.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.64.3, 7.3.2.65.4, 11.20.3.5 | (CFv AND PC34):M | Yes, No, N/A |
| WNM5 | Location | 11.20.4, 7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Civic Request/Report | 11.10.8.9 | CFv:M | Yes, No, N/A |
| WNM5.2 | Location Identifier Request/Report | 11.10.8.10 | CFv:M | Yes, No, N/A |
| WNM5.3 WNM5.3.1 WNM5.3.2 | Location Track Notification Time Of Departure Notifications Motion Detection Notifications | 11.20.4, 7.4.7.10 11.20.4, 7.3.2.66 11.20.4, 7.3.2.66 | CFv:O CFv:O CFv:O | Yes, No, N/A |
| WNM5.4 | Location Configuration Request frame | 7.4.11.7, 7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM5.4.1 | Normal Indication | 7.4.11.7, 7.3.2.66 | CFv:O | Yes, No, N/A |
| WNM5.4.2 | Motion Indication | 7.4.11.7, 7.3.2.66 | CFv:O | Yes, No, N/A |
| WNM5.5 | Location Configuration Response frame | 7.4.11.8, 7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM6 WNM6.1 WNM6.2 | Multiple BSSID Support Multiple BSSID element Multiple BSSID-index element | 11.1.2.3a, 11.1.3, 11,10.11 7.3.2.46 7.3.2.69 | CFv:O WNM6: M WNM6: M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.20.6 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report Element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query frame | 7.4.11.8 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request frame | 7.4.11.9 | (CFv AND CF2):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response frame | 7.4.11.10 | (CFv AND CF2):M | Yes, No, N/A |
| *WNM8 | FMS | 9.2.7, 11.20.7 | CFv:O | Yes, No, N/A |
| WNM8.1 | FMS Request frame | 7.4.11.11 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FMS Response frame | 7.4.11.12 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.20.13 | CFv:O | Yes, No, N/A |
| WNM10 WNM10.11.1 WNM10.11.2 | Co-located Interference Reporting Co-located Interference Request frame Co-located Interference Response frame | 7.4.11.13, 7.4.11.14, 11.20.9 7.4.11.13 7.4.11.14 | CFv:O WNM10: M WNM10: M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| WNM11 WNM11.1 | BSS Max Idle Period BSS Max Idle Period element | 7.3.2.74, 11.20.12 7.3.2.74 | CFv:M WNM: M | Yes, No, N/A Yes, No, N/A |
| *WNM12 | TFS | 11.20.11 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.75, 7.4.11.15 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.76, 7.4.11.16 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.11.17 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | WNM-Sleep Mode | 11.2.2 | CFv:O | Yes, No, N/A |
| WNM13.1 | WNM-Sleep Mode Request frame | 7.3.2.77, 7.4.11.18 | WNM13: M | Yes, No, N/A |
| WNM13.2 | WNM-Sleep Mode Response frame | 7.3.2.77, 7.4.11.19 | WNM13: M | Yes, No, N/A |
| WNM14 WNM14.1 WNM14.2 WNM14.3 | TIM broadcast TIM Broadcast Request frame TIM Broadcast Response frame TIM Broadcast frame | 11.2.1.13 7.3.2.78, 7.4.11.20 7.3.2.79, 7.4.11.21, 7.4.11.22, | CFv:O WNM14: M WNM14: M WNM14: M | Yes, No. N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| WNM15 WNM15.1 WNM15.2 | QoS Traffic Capability QoS Traffic capability element QoS Traffic capability update frame | 11.20.9 7.3.2.73 7.4.11.23 | (CFv AND CF2):O WNM15: M WNM15: M | Yes, No, N/A Yes, No, N/A Yes, No. N/A |
| WNM16 | AC Station Count | 11.20.10 | (CFv AND CF2):O | Yes, No, N/A |
| WNM17 | Timing Measurement | 11.20.5 | CFv:O | Yes, No, N/A |
| WNM18 WNM18.1 WNM18.2 | Channel Usage Channle Usage request frame Channel Usage response frame | 11.20.14; 7.3.2.81, 7.4.7.12 7.3.2.81, 7.4.7.13 | CFv:O WNM18: M WNM18: M | Yes, No, N/A Yes, No. N/A Yes. No. N/A |

- Annex D

(normative)

- ASN.1 encoding of the MAC and PHY MIB

- Insert the following element at the end of the list of child objects for dot11smt object identifier:

--***********************************************************

--*Wireless Network Management

--************************************************************

-- dot11WirelessMgmtOptionsTable	::= { dot11smt 17}

- Insert the following text at the end of the Station management MIB:

-- **********************************************************************

-- * dot11WirelessMgmtOptions TABLE

-- **********************************************************************

dot11WirelessMgmtOptionsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WirelessMgmtmOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“Wireless Management attributes. In tabular form to

allow for multiple instances on an agent. This table

only applies to the interface if dot11WirelessManagementImplemented

is set to TRUE in the dot11StationConfigTable. Otherwise

this table should be ignored.”

::= { dot11smt 17 }

dot11WirelessMgmtOptionsEntry OBJECT-TYPE

SYNTAX Dot11WirelessMgmtOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WirelessMgmtOptionsTable. For all Wireless Management features, an “enabled” MIB variable is used to enable or disable the corresponding feature.  An “Implemented” MIB variable is used for an optional feature to indicate whether the feature is implemented. A mandatory feature does not have a corresponding “Implemented” MIB variable. It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one. As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11WirelessMgmtOptionsTable 1 }

- Editor: Make the following text  changes to Annex D  in 802.11v Draft:

- Dot11WirelessMgmtOptionsEntry ::=

- SEQUENCE {

dot11MgmtOptionLocationEnabled				TruthValue,

dot11MgmtOptionFMSImplemented			 TruthValue,

dot11MgmtOptionFMSEnabled			TruthValue,

dot11MgmtOptionEventsEnabled			TruthValue,

dot11MgmtOptionDiagnosticsEnabled				TruthValue,

dot11MgmtOptionMultiBSSIDImplemented				TruthValue,

dot11MgmtOptionMultiBSSIDEnabled				TruthValue,

dot11MgmtOptionTFSImplemented 				TruthValue,

dot11MgmtOptionTFSEnabled			TruthValue,

dot11MgmtOptionWNMSleepModeImplemented				TruthValue, 																			 		dot11MgmtOptionWNMSleepModeEnabled				TruthValue,

dot11MgmtOptionTIMBroadcastImplemented				TruthValue,

dot11MgmtOptionTIMBroadcastEnabled				TruthValue,

dot11MgmtOptionProxyARPEnabled				TruthValue,
		dot11MgmtOptionProxyARPImplemented 				Truthvalue

dot11MgmtOptionBSSTransitionEnabled				TruthValue,

dot11MgmtOptionTrafficGenerationImplemented				TruthValue,

dot11MgmtOptionTrafficGenerationEnabled				TruthValue,

dot11MgmtOptionACStationCountImplemented				TruthValue,

dot11MgmtOptionACStationCountEnabled				TruthValue,

dot11MgmtOptionMulticastDiagnosticsEnabled				TruthValue,
		dot11MgmtOptionCOLOINTReportingEnabled				TruthValue,

dot11MgmtOptionCOLOINTReportingImplemented 				TruthValue,

dot11MgmtOptionLocationTrackNotificationImplemented		TruthValue, 
		dot11MgmtOptionLocationTrackNotificationEnabled			TruthValue,
		dot11MgmtOptionCOLOINTReportingImplemented 				TruthValue,

dot11MgmtOptionMotionDetectionImplemented				TruthValue, 
		dot11MgmtOptionMotionDetectionEnabled				TruthValue,
		dot11MgmtOptionTODImplemented 				TruthValue,

dot11MgmtOptionTODEnabled			 TruthValue,

dot11MgmtOptionTimingMsmtImplemented 				TruthValue,

dot11MgmtOptionTimingMsmtEnabled				TruthValue
		dot11MgmtOptionChannelUsageImplemented				TruthValue,
		dot11MgmtOptionChannelUsageEnabled				TruthValue}

dot11MgmtOptionLocationTrackNotificationImplemented OBJECT-TYPE

- SYNTAX TruthValue

- MAX-ACCESS read-only

- STATUS current

- DESCRIPTION

- "This attribute, when TRUE, indicates that the station

implementation is capable of supporting Location Track Notification when the dot11WirelessManagementImplemented is TRUE."

- DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry x }

dot11MgmtOptionLocationTrackNotificationEnabled OBJECT-TYPE

- SYNTAX TruthValue

- MAX-ACCESS read-write

- STATUS current

- DESCRIPTION

- "This attribute, when TRUE, indicates that the

capability of the station to provide Location Track Notification is

enabled. The capability is disabled, otherwise"

- DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry y }