IEEE P802.11
Wireless LANs

| Proposal for Native-GAS Query for IP Address Type availability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

This proposal is making the necessary corrections to the IP Address Availability Information, to address the comments submitted during LB#137

This proposal is based on draft TGu version D4.01.

- Modify the text in Clause 7.3.3.9 as follows:

7.3.3.9 IP Address Type Availability Information

The IP Address Type Availability information provides non-AP STA with the information about the availability of IP address version and type which could be allocated to the non-AP STA after successful association. This information may be returned in response to a Native GAS Query Request. The format of the IP Address Type Availability information is shown in Figure 7-95bm.

| - IP Address Type Availability information |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | B0 – B1 | B2 – B7 |  |  |
|  | Info ID | Length | Status Code | IPv6 Address | IPv4 Address |  |  |
| Bits: |  |  |  | 2 | 6 |  |  |
| Octets: | 2 | 2 | 2 |  |  |  |  |

Figure 7-95bm

The Info ID field is equal to the value in Table 7-43x corresponding to the IP Address Type Availability information.

The Length is a 2 octets field. The value of the Length field is 3 octets.

The Status Code is a 2 octets field whose value is drawn from Table 7-23.

The IPv6 address field format is shown in Table 7-43aa.

Table 7-43aa - IPv6 address field values

| - IPv6 address values |  |
| --- | --- |
| Address Value | Meaning |
| 0 | Address type not available |
| 1 | Address Type available |
| 2 | Availability of the address type not known |
| 3 | Reserved |

The IPv4 address field format is shown in Table 7-43ab

Table 7-43ab—IPv4 address field values

.

| - IPv4 address values |  |
| --- | --- |
| Address Value | Meaning |
| 0 | Address type not available |
| 1 | Public IPv4 address available |
| 2 | Port-restricted IPv4 address available |
| 3 | Single NATed private IPv4 address available |
| 4 | Double NATed private IPv4 address available |
| 5 | Port-restricted IPv4 address and Single NATed IPv4 address available |
| 6 | Port-restricted IPv4 address and double NATed IPv4 address available |
| 7 | Availability of the address type is not known |
| 8-63 | Reserved |