IEEE P802.11
Wireless LANs

| Proposal for Native-GAS Query for AP’s Date, Time and Time Zone information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

This proposal is making the necessary corrections to the AP Date, Time and Time Zone Information to address the comments submitted during LB#137.

Remove the following reference from Clause 2:

2. Normative references

Insert the following new references into 2 maintaining the ordering:

IANA, EAP Method Type Numbers, http://www.iana.org/assignments/eap-numbers.

IANA, PPP Protocol Numbers, http://www.iana.org/assignments/ppp-numbers.

IEEE P802.21/D14.00, Draft IEEE Standard for Local and Metropolitan Area Networks: Media Independent Handover Services, September 2008.

IEEE 1003.1-2004, Standard for information technology - portable operating system interface (POSIX)

IETF RFC 3629, UTF-8, a transformation format of ISO 10646, F. Yergeau, November 2003.

IETF RFC 3986, Uniform Resource Identifier (URI): Generic Syntax, January 2005.

IETF RFC 4282, The Network Access Identifier, December 2005

IETF RFC 5222, LoST: A Location-to-Service Translation Protocol, August 2008

ISO 3166-1, http://www.iso.org/iso/iso_catalogue/catalogue_tc/catalogue_detail.htm?csnumber=39719. ISO 8601:2004, Data elements and interchange formats - Information interchange, Representation of dates and times

OASIS, Emergency Management Technical Committee, “Common Alerting Protocol Version 1.1” http://xml.coverpages.org/CAPv11-12649.pdf, April 2005

OASIS, Emergency Management Technical Committee, “Emergency Data Exchange Language (EDXL) Distribution Element, v. 1.0”. OASIS Standard EDXL-DE v1.0, http://docs.oasis-open.org/emergency/edxl-de/v1.0, May-2006.

- Apply the following changes to Clause 7.3.3.8:

- 7.3.3.8 AP Date, Time and Time Zone Information

The AP Date, Time and Time Zone information provides the correct date and time zone at the AP's location and the best known time of the AP at the time of the query. This information may be returned in response to a Native GAS Query Request. The format of the AP Date, Time and Time Zone information is provided in Figure 7-95bj.

| - AP Date, Time and Time Zone information format |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Info ID | Length | Status Code | Date | Time | Time Zone |
| Octets: | 2 | 2 | 2 | 4 | variable | variable |

The Info ID field is equal to the value in Table 7-43x corresponding to the AP Date, Time and Time Zone Information.

The Length is a 2 octets field. The value of the length field is variable, the minimum value of the length field is 14 octets.

The Status Code is a 2 octets field whose value is drawn from Table 7-23.

The format of the Date is as described in Figure 7-95bk, and represents the correct date at the APs location.

Replace Figure 7-95bk in Clause 7.3.3.8 with the following figure:

|  | Day of Month | Month | Year |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure 7-95bk—AP Date, Time and Time Zone information format

Add the following text after  Figure 7-95bk in Clause 7.3.3.8:

The Day of Month field has a value of 1 to 31.

The Month field has a value of 1 to 12.

The Year field has a value of 0 to 65534.

Modify the text in Clause 7.3.3.8 as follows:

Example: 19841113 meaning November 13, 1984 is encoded as 13111984.

The format of the Time field is as described in Figure 7-95bl and represents the time known by the AP at the time of the query.

Replace Figure 7-95bl in Clause 7.3.3.8 with the following figure:

|  | Hours | Minutes | Seconds |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |

Figure 7-95bl—Time Field format

Add the following text after  Figure 7-95bl in Clause 7.3.3.8:

The value of the Hours field is 1 to 12.

The value of the Minutes field is 0 to 59.

The value of the Seconds field is 0 to 59.

Modify the text in Clause 7.3.3.8 as follows:

Example: 114537 meaning 11h45min37.

The format of the Time Zone is as specified in section 8.3 of IEEE Std 1003.1-2004:

stdoffset[dst[offset][,start[/time],end[/time]]]

The length of the field is no less than 4 octets and no more than TZNAME_MAX, as defined in IEEE Std 1003.1-2004. The Time Zone field represents the correct time zone at the APs location. The encoding of the field is in ASCII character as shown in the following Example-1.

Example-1: EST5

Example-2: EST5EDT4,M3.2.0/02:00,M11.1.0/02:00

In the Example-2 above, the string is interpreted as a time zone that is normally five hours behind UTC, and four hours behind UTC during DST, which runs from the second Sunday in March at 02:00 local time through the first Sunday in November at 02:00 local time. Normally the time zone is abbreviated "EST" but during DST it is abbreviated "EDT".