IEEE P802.11
Wireless LANs

| LB-137 Venue Group & Venue Type Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |

Abstract

This proposal addresses a number of comments related to the Venue Group and Venue Type, which resulted from LB137.  This proposal addresses the following CIDs: 4086 4396, 4615, 4076, 4455 and 4397.  This proposal allows making the Venue Group and Venue Type field in Interworking elements to be optional. It also introduces new Venue Group and Venue Type entries.

This document is based on 802.11u-d4.01 and 11-08/1343r1

### 7.3.2.59 Interworking information element

The Interworking information element contains information about the interworking service capabilities of a STA as shown in Figure 7-95ao.

|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN | Emergency Service Accessible | Venue Group (optional) | HESSID (optional) |
| Octets: | 1 | 1 | 1 |  |  |  |  | 0 or 2 | 0 or 6 |

Figure 7-95ao-Interworking element format

The value of the length field is 1 if no optional field is present, 3 if only Venue info field is present, 7 if only HESSID field is present or 9 if all optional fields are present.

The Network Types are shown in 7-43r. It is set by AP to advertise its Network Type to non-AP STAs. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See U.4 for informative text on usage of fields contained within the Interworking element.

Table 7-43r Network Type codes

| Network Type Codes | Meaning | Description |
| --- | --- | --- |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. These networks may or may not employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts are available. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http:/https redirect or DNS redirection). |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. |
| 4 | Peer to Peer Network | A small network of personal devices such as camera and computer, where one device assumes the role of AP. |
| 5 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |

Bit 4 is the Internet field. The AP sets  this field to 1 if the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet. If the  Network Type is set to 2, 3, or 14, then this field is set to 1 by the AP when the network provides connectivity to the Internet, and it is set to 0 when the network provides unspecified connections to external networks.. The non-AP STA may also set this bit during the active scan to specify its desire to connect to a network which provides Internet connection.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 to indicate that the network requires a further step for access; otherwise it is set to 0. For more information, refer to Network Authentication Type Information in 7.3.35.

Bit 6 is the Emergency Alert System Notification (EASN) capability field.

It is set to 1 by an AP to indicate that a higher layer EAS message is available at the AP for download."  Change 4th sentence to "The EASN capability field set to 0 indicates that no higher

It is set to 1 by an AP to indicate that a higher layer EAS message is available at the AP for download. It is a reserved field for the non-AP STA. The EASN capability field set to 0 indicates that no higher layer EAS message is available at the AP for download. When the field is set to 1 by an AP, a non-AP STA requests further information from the higher layers to receive the full EAS information.

Bit 7 is the Emergency Network field. When the AP sets it to 0, this field indicates that no emergency services are directly configured to be reachable through the SSID. When set to 1, this field indicates that higher layer emergency services are reachable through the SSID, in addition to any other services offered. Emergency services may be supported either by an ESO (emergency services only) network or a network that supports emergency services accessible with public credentials. An AP advertises this capability if at least one of the two methods is supported.

For a network to be identified as an Emergency Services (ES) network, location capability is enabled on an AP if the AP is located in a regulatory domain that requires location capabilities. In addition, this field is only advertised if all of the following are true (see U.4.2 for further information):

- If an ESO network, the SSID is configured for open authentication and used exclusively for providing emergency services

- If an Emergency Services network with public credentials, the SSID is configured for RSN authentication

- dot11QosOptionImplemented is true

- dot11EBREnabled is true

- Network supports end-to-end QoS from non-AP STA to all APs in ESS to PSAP. It is outside the scope of the Interworking Service how this information is known to the AP

The Venue info is a 2 octet field. It contains Venue Group and Venue Type duple. The format of Venue info is shown in Figure 7-43r1.

|  | Venue Group | Venue Type |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure 7-43r1 Venue info format

The Venue Group and Venue Type fields are both one octet values selected from 7-43s and 7-43t respectively. The entries in 7-43s and 7-43t are drawn from the International Building Code's Use and Occupancy Classifications [B33].

Table 7-43s-Venue Group codes and descriptions

| Venue Group Code | Venue Group Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Assembly |
| 2 | Business |
| 3 | Educational |
| 4 | Factory and Industrial |
| 5 | Institutional |
| 6 | Mercantile |
| 7 | Residential |
| 8 | Storage |
| 9 | Utility and Miscellaneous |
| 10 | Vehicular |
| 11 | Outdoor |
| 12 | Personal Network |
| 13 – 254 | Reserved |
| 255 | Wildcard |

7-43t-Venue Type assignments

| Venue Group | Venue Type Code | Venue Description |
| --- | --- | --- |
| 0 | 0 | Unspecified |
| 0 | 1 – 255 | Reserved |
| 1 | 0 | Unspecified Assembly |
| 1 | 1 | Arena |
| 1 | 2 | Stadium |
| 1 | 3 | Passenger Terminal (e.g., airport, bus, ferry, train station) |
| 1 | 4 | Amphitheater |
| 1 | 5 | Amusement Park |
| 1 | 6 | Place of Worship |
| 1 | 7 | Convention Center |
| 1 | 8 | Library |
| 1 | 9 | Museum |
| 1 | 10 | Restaurant |
| 1 | 11 | Theater |
| 1 | 12 | Bar |
| 1 | 13 | Coffee Shop |
| 1 | 14 | Zoo or Aquarium |
| 1 | 15 | Emergency Coordination Center |
| 1 | 16 – 255 | Reserved |
| 2 | 0 | Unspecified Business |
| 2 | 1 | Doctor or Dentist office |
| 2 | 2 | Bank |
| 2 | 3 | Fire Station |
| 2 | 4 | Police Station |
| 2 | 6 | Post Office |
| 2 | 7 | Professional Office |
| 2 | 8 | Research and Development Facility |
| 2 | 9 | Attorney Office |
| 2 | 10 – 255 | Reserved |
| 3 | 0 | Unspecified Educational |
| 3 | 1 | School, Primary |
| 3 | 2 | School, Secondary |
| 3 | 3 | University or College |
| 3 | 4-255 | Reserved |
| 4 | 0 | Unspecified Factory and Industrial |
| 4 | 1 | Factory |
| 4 | 2 – 255 | Reserved |
| 5 | 0 | Unspecified Institutional |
| 5 | 1 | Hospital |
| 5 | 2 | Long-Term Care Facility (e.g., Nursing home, Hospice, etc.) |
| 5 | 3 | Alcohol and Drug Re-habilitation Center |
| 5 | 4 | Group Home |
| 5 | 5 | Prison or Jail |
| 5 | 6 - 255 | Reserved |
| 6 | 0 | Unspecified Mercantile |
| 6 | 1 | Retail Store |
| 6 | 2 | Grocery Market |
| 6 | 3 | Automotive Service Station |
| 6 | 4 | Shopping Mall |
| 6 | 5 - 255 | Reserved |
| 7 | 0 | Unspecified Residential |
| 7 | 1 | Hotel or Motel |
| 7 | 2 | Dormitory |
| 7 | 3 | Boarding House |
| 7 | 4 - 255 | Reserved |
| 8 | 0 - 255 | Reserved |
| 9 | 0 - 255 | Reserved |
| 10 | 0 | Unspecified Vehicular |
| 10 | 1 | Automobile or Truck |
| 10 | 2 | Airplane |
| 10 | 3 | Bus |
| 10 | 4 | Ferry |
| 10 | 5 | Ship or Boat |
| 10 | 6 | Train |
| 10 | 7 | Motor Bike |
| 10 | 7 - 255 | Reserved |
| 11 | 0 | Unspecified Outdoor |
| 11 | 1 | Muni-mesh Network |
| 11 | 2 | City Park |
| 11 | 3 - 255 | Reserved |
| 12 | 0 | Reserved |

The HESSID field specifies the value of HESSID, see 11.18.1. A non-AP STA uses this field to indicate the desired HESSID in an active scan.

- 10.3.2.1 MLME-SCAN.request

- Semantics of the service primitive

- Change the primitive parameter list in as shown:

MLME-SCAN.request(

BSSType,
BSSID,
SSID,
ScanType,
ProbeDelay,
ChannelList,
MinChannelTime,
MaxChannelTime,

NetworkType,
HESSID,

VendorSpecificInfo
)

- Insert the following rows before the VendorSpecificInfo row of the untitled table defining the primitive parameters as follows:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Network Type | As defined in Table 7-43r | 0 to 15 | Specifies a desired specific Network Type or the wildcard Network Type. |
| HESSID | MAC Address | Any valid individual MAC address or the broadcast MAC address | Specifies the desired specific HESSID network identifier or the wildcard network identifier. |
| Network Type | As defined in Table 7-43r | 0 to 15 | Specifies a desired specific Network Type or the wildcard Network Type. |

- 10.3.10.1 MLME-START.request

- 10.3.10.1.2 Semantics of the service primitive

- Change the primitive parameter list as shown:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay.

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

,

InterworkingInfo,

AdvertismentProtocolInfo,

VendorSpecificInfo

)

- Insert the following rows before the VendorSpecificInfo row of the untitled table defining the primitive parameters the table as follows:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingInfo | As defined in 7-3.2.59 | As defined in Interworking element in 7.3.2.59 | Specifies a Interworking element values |
| AdvertisementProtocolInfo | Integer or Sequence of Integers | As defined in Advertisement Protocol element in 7-43-u. | Identifies zero or more advertisement protocols and advertisement control to be used in the BSSs. |