IEEE P802.11
Wireless LANs

| LB-137 Interworking element comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |

Abstract

This proposal addresses a number of comments related to the Interworking element, which resulted from LB137.  This proposal addresses the following CIDs: 4005, 4074, 4075, 4078, 4180, 4213, 4215, 4395, 4448, 4449, 4616, 4638, 4732, 4733, 4734 and 4842.

This document is based on 802.11u-d4.01.

### 7.3.2.59 Interworking information element

The Interworking information element contains information about the interworking service capabilities of a STA as shown in Figure 7-95ao.

|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN | Emergency Service Accessible | Venue Group | Venue Type | HESSID (optional) |
| Octets: | 1 | 1 | 1 |  |  |  |  | 1 | 1 | 0 or 6 |

Figure 7-95ao-Interworking element format

The value of the length field is 3 if no HESSID field is present or 9 if an HESSID field is present.

The Network Types are shown in 7-43r. It is set by AP to advertise its Network Type to non-AP STAs. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See U.1 for informative text on usage of fields contained within the Interworking element.

Table 7-43r Network Type codes

| Network Type Codes | Meaning | Description |
| --- | --- | --- |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. These networks may or may not employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts are available. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http:/https redirect or DNS redirection). |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. |
| 4 | Peer to Peer Network | A small network of personal devices such as camera and computer, where one device assumes the role of AP. |
| 5 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |

Bit 4 is the Internet field. The AP sets  this field to 1 if the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 by the AP to indicate that the network requires a further step for access; otherwise it is set to 0. For more information, refer to  Network Authentication Type Information in 7.3.35.

Bit 6 is the Emergency Alert System Notification (EASN) field. It is set to 1 by an AP  to indicate that a higher layer EASN message is available at the AP for download. It is a reserved field for the non-AP STA. The EASN capability field set to 0 indicates that no higher layer EASN message is available at the AP for download. When the field is set to 1 by an AP, a non-AP STA requests further information from the higher layers to receive the full EASN message.

Bit 7 is the Emergency Service Accessible field. When the AP sets it to 0, this field indicates that no emergency services is directly configured to be reachable through the SSID. When set to 1, this field indicates that higher layer emergency services are reachable through the SSID, in addition to any other services offered. Emergency services may be supported either by an ESO (emergency services only) network or a network that supports emergency services accessible with public credentials. An AP advertises this capability if at least one of the two methods is supported.

For a network to be identified as an Emergency Services (ES) network, location capability is enabled on an AP if the AP is located in a regulatory domain that requires location capabilities. In addition, this field is only advertised if all of the following are true (see U.4.2 for further information):

- If an ESO network, the SSID is configured for open authentication and used exclusively for providing emergency services

- If an Emergency Services network with public credentials, the SSID is configured for RSN authentication

- dot11QosOptionImplemented is true

- dot11EBREnabled is true

- Network supports end-to-end QoS from non-AP STA to all APs in ESS to PSAP. It is outside the scope of the Interworking Service how this information is known to the AP

The Venue Group and Venue Type fields are both one octet values selected from 7-43s and 7-43t respectively. The entries in 7-43s and 7-43t are drawn from the International Building Code's Use and Occupancy Classifications [B33]. An AP sets the default value of Venue Group to Residential and the default value of Venue Type to unspecified. A non-AP STA sets the default value of Venue Group to unspecified and the default value of Venue Type to unspecified. A non-AP STA sets the default value of Venue Group to unspecified and the default value of Venue Type to

Table 7-43s-Venue Group codes and descriptions

| Venue Group Code | Venue Group Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Assembly |
| 2 | Business |
| 3 | Educational |
| 4 | Factory and Industrial |
| 5 | Institutional |
| 6 | Mercantile |
| 7 | Residential |
| 8 | Storage |
| 9 | Utility and Miscellaneous |
| 10 | Vehicular |
| 11 | Outdoor |
| 12 - 255 | Reserved |

7-43t- Venue Type assignments

| Venue Group | Venue Type Code | Venue Description |
| --- | --- | --- |
| 0 | 0 | Unspecified |
| 0 | 1 - 255 | Reserved |
| 1 | 0 | Unspecified Assembly |
| 1 | 1 | Arena |
| 1 | 2 | Stadium |
| 1 | 3 | Passenger Terminal (e.g., airport, bus, ferry, train station) |
| 1 | 4 | Amphitheater |
| 1 | 5 | Amusement Park |
| 1 | 6 | Place of Worship |
| 1 | 7 | Convention Center |
| 1 | 8 | Library |
| 1 | 9 | Museum |
| 1 | 10 | Restaurant |
| 1 | 11 | Theater |
| 1 | 12 | Bar |
| 1 | 13 | Coffee Shop |
| 1 | 14 | Zoo or Aquarium |
| 1 | 15 | Emergency Coordination Center |
| 1 | 16 - 255 | Reserved |
| 2 | 0 | Unspecified Business |
| 2 | 1 | Doctor or Dentist office |
| 2 | 2 | Bank |
| 2 | 3 | Fire Station |
| 2 | 4 | Police Station |
| 2 | 6 | Post Office |
| 2 | 7 | Professional Office |
| 2 | 8 | Research and Development Facility |
| 2 | 9 | Attorney Office |
| 2 | 10 - 255 | Reserved |
| 3 | 0 | Unspecified Educational |
| 3 | 1 | School, Primary |
| 3 | 2 | School, Secondary |
| 3 | 3 | University or College |
| 3 | 4-255 | Reserved |
| 4 | 0 | Unspecified Factory and Industrial |
| 4 | 1 | Factory |
| 4 | 2 - 255 | Reserved |
| 5 | 0 | Unspecified Institutional |
| 5 | 1 | Hospital |
| 5 | 2 | Long-Term Care Facility (e.g., Nursing home, Hospice, etc.) |
| 5 | 3 | Alcohol and Drug Re-habilitation Center |
| 5 | 4 | Group Home |
| 5 | 5 | Prison or Jail |
| 5 | 6 - 255 | Reserved |
| 6 | 0 | Unspecified Mercantile |
| 6 | 1 | Retail Store |
| 6 | 2 | Grocery Market |
| 6 | 3 | Automotive Service Station |
| 6 | 4 | Shopping Mall |
| 6 | 5 - 255 | Reserved |
| 7 | 0 | Unspecified Residential |
| 7 | 1 | Hotel or Motel |
| 7 | 2 | Dormitory |
| 7 | 3 | Boarding House |
| 7 | 4 - 255 | Reserved |
| 8 | 0 - 255 | Reserved |
| 9 | 0 - 255 | Reserved |
| 10 | 0 | Unspecified Vehicular |
| 10 | 1 | Automobile or Truck |
| 10 | 2 | Airplane |
| 10 | 3 | Bus |
| 10 | 4 | Ferry |
| 10 | 5 | Ship or Boat |
| 10 | 6 | Train |
| 10 | 7 - 255 | Reserved |
| 11 | 0 | Unspecified Outdoor |
| 11 | 1 | Muni-mesh Network |
| 11 | 2 | City Park |
| 11 | 3 - 255 | Reserved |

The HESSID field specifies the value of HESSID, see 11.18.1. A non-AP STA uses this field to indicate the desired HESSID in an active scan.