IEEE P802.11
Wireless LANs

| Resolving some SAE comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-1111 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Netrworks | 1322 Crossman ave, Sunnyvale, CA | +1 408 227 4500 | dharkins@arubanetworks.com |

Abstract

This document addresses CIDs 143, 458, 861, 865, 1891. It also cleans up another artefact that was not correctly added to the draft after the adoption of 11-08/0799r2.

It is proposed that the comments are resolved as follows:

- 143 – counter, SAE is place in the SME and MLME primitives are added for communication toand from the SME.

- 458 – accept

- 861 – counter, the pwd-seed is used to deterministically choose which solution of the quadratic equation to use.

- 865 – counter, both results are positive, but the LSB of y is compared with the LSB of pwe-seed to determine which result to use.

- 1137—counter, normative text has been added to the clause

- 1891—accept

Instruct editor to make the following changes described in bold and underlined text. Existing text is modified by indicating added text with an underline and deleted text with a strikethrough. The underline must not carry over into the draft unless otherwise specified.

Add the following two sections:

10.3.53 MLME-SAESend.request

10.3.53.1 Function

This primitive requests that the MAC entity send an SAE frame to a specified peer MAC entity..

10.3.53.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-SAESend.request(

peerMAC

Content of SAE frame

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC | MAC Address | Valid individual MAC address | Specifies the address of the peer MAC entity with which to perform SAE authentication and key establishment. |
| Content of SAE frame | Sequence of octets | As defined in 7.4.12.1, 7.4.12.2 or 7.4.12.3 | The contents of the SAE commit message or SAE confirm message. |

10.3.53.3 When generated

This primitive is generated by the SME to request that an SAE frame be sent to the specified peer.

10.3.53.4 Effect of receipt

On receipt of this primitive, the MLME constructs an 802.11 authentication frame containing the specified SAE message. The frame is scheduled for transmission.

10.3.54 MLME-SAE.confirm

10.3.54.1 Function

This primitive reports the results of a request to send an SAE 802.11 authentication frame.

10.3.54.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-SAE.request(

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, or, UNSPECIFIED_FAILURE | Reports the outcome of the request to send an SAE 802.11 authentication frame. |

10.3.54.3 When generated

This primitive is generated by the MLME as a result of an MLME-SAESend.request primitive.

10.3.54.4 Effect of receipt

The SME is notified of the results of the SAE send request.

10.3.55 MLME-SAERecv.indication

10.3.55.1 Function

This primitive indicates to the SME that the MLME has received an SAE 802.11 authentication frame from a peer MAC entity.

10.3.55.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-SAERecv.indication(

PeerMAC

Content of Peer Link Management Frame

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC | MAC Address | Valid individual MAC address | Specifies the address of the peer MAC entity to be deauthenticated and with which all SAE state is to be destroyed |
| Content of SAE 802.11 authentication frame | Sequence of octets | As defined in 7.4.12.1, 7.4.12.2, or 7.4.12.3 | The contents of the commit or confirm message |

10.3.55.3 When generated

This primitive is generated by the MLME as a result of the receipt of an SAE 802.11 authentication frame.

10.3.55.4 Effect of receipt

The SME is notified of the reception of an SAE 802.11 authentication frame and is provided the contents therein.

Modify the following section:

11B.2.3.1 Elliptic Curve Finite Cyclic Groups

Elliptic curves for use in SAE are based on finite fields over a prime number, p, that is comprised of the set of integers {0, 1, 2, …, p-1 }. Each such integer in the set is represented by a binary string that is the equal to the bitlength of p, prepending the integer with 0 bits, if necessary, until the required length is achieved. Points on the curve are represented by an x-coordinate and a y-coordinate. The curve is represented by an equation, y2 = x3 + ax + b, for some fixed value of a and b, a prime, p, and a co-factor h. Each elliptic curve has a special point called the “point-at-infinity”Elements in these groups are represented by (x,y) on an elliptic curve. The convention used herein represents a point with an upper-cast name and a scalar value with a lower-case name.

Modify the following section:

11B.2.3.1.1 Generation of the Password Element

The Password Element of an elliptic curve is called PWE and is generated in a random hunt-and-peck fashion. A counter is used with the password to generate a seed value. This counter, represented as a single octet, is initially set to one (1). Then the password seed is stretched using the KDF function from section 8.8.3 to the length of the prime number from the group definition with the string “SAE Hunting and Pecking.” The resulting password value is then reduced modulo the prime. The reduced password value is then used as the x-coordinate of a curve and the y-coordinate is determined by solving for y using the equation of the curve. There are two possible solutions for y, one positive and one negative.  and the equation for the curve is checked to see if a solution for y exists. If no solution exists, the counter is incremented, a new password-seed is derived and the hunting-and-pecking continues. If a solution exists, there will be two possible values for y. The password seed is used to determine which one to use. If the LSB of the password seed is odd (i.e. if the low-order bit is set )  equal to the LSB of y returned as the solution to the quadratic equation then the point candidate PWE is (x, -y)   (x, y) otherwise if the password seed is even the point candidate PWE is (x, y) (x, p-y). The point is checked to see whether it lies on the curve. If it does not The candidate PWE is then multiplied by the co-factor of the curve to produce a test point. If the test point is “the point-at-infinity” the counter is incremented , a new password seed is derived and the hunting-and-pecking process continues. If it does lie on the curve not equal the “point-at-infinity” this the candidate PWE point becomes shall become the PWE. (Note: the test point—the co-factor of the curve multiplied by the candidate PWE—does not become the PWE).

Algorithmically this process can be described as follows:

found = 0

counter = 1

z = len(prime)

do {

pwd-seed = H(PSK || counter)

pwd-value = KDF-z(pwd-seed, “SAE Hunting and Pecking”) modulo prime

x = pwd-value modulus p

y = solve_equation(curve, x)

if (is_odd(pwd-seed))

PWE = (x,-y)

else

PWE = (x,y)

Fi

if (on_curve(PWE))

found=1

fi

if an equation exists for y from x and the equation for the curve: y2 = x3 + ax + b

then

if LSB of pwe-seed equals LSB of y

then

PWE = (x,y)

else

PWE = (x, p-y)

Fi

T = h * PWE

if T does not equal the “point-at-infnity”

then

found = 1

fi

fi

counter = counter + 1

} while (found=0)

Modify the following chapters:

11B.2.3.1.2 Construction of a commit

Upon discover of a peer a secret element is shall be derived from PWE based on the identities of the two MPs.

11B.2.3.1.3 Processing of a Peer’s commit

Upon receipt of a peer’s commit message a shared secret value, k, is shall be derived using the scalar and element (peer-commit-scalar and peer-commit-element, respectively) from the peer’s commit message and the MP’s secret value.

11B.2.3.1.5 Processing of a Peer’s confirm

Upon receipt of a peer’s confirm message a verifier is cmputed which is the expected value of the peer’s confirmation, peer-confirm. If the verifier equals peer-confirm the MP shall accepts the peer’s authentication.

If the verifier differs from the peer-confirm the MP shall rejects the peer’s authentication.

11B.2.3.1.6 Generation of the PMK

If the MP accepts the peer’s authentication a shared secret, ss PMK, for use with the MSA 4-way Abbriviated Hhandshake is shall be derived using the random function H and the order of the group, r:

11B.2.3.2.2 Construction of a commit

Upon discovery of a peer a secret element is shall be derived from pwe based on the MAC  addresses identities of the two MPs.

11B.2.3.2.3 Processing of a Peer’s commit

Upon receipt of a peer’s commit message a shared secret value, k, is shall be derived using the scalar and element (peer-commit-scalar and peer-commit-element, respectively) from the peer’s commit message and the MP’s secret value.

11B.2.3.2.6 Generation of the PMK

If the MP accepts the peer’s authentication a shared secret,  PMK, for use with the MSA 4-way Abbriviated Hhandshake is shall be derived using the random function H and the order of the group, r:

11B.2.4 Anti-Clogging Tokens

When that counter hits or exceeds dot11MeshSAEThresh the MP shall responds to each commit message with a rejection that includes an anti-clogging token statelessly bound to the sender of the commit message.

11B.2.5 SAE Finite State Machine

The model in which SAE is defined consists of a parent processed, managed by the SME, which receives messages, and dispatches them to the appropriate protocol instance, also managed by the SME.

11B.2.5.2.1 Parent Process Events and Outputs

- Initiate—an MLME-SAE.Init.request Initiate event is used to instantiate a protocol instance to begin SAE with a designated peer.

- Kill—an MLME-SAE.Kill.request event is used to remove a protocol instance with a designated peer.

Modify the Dot11OperationsEntryTable in Annex D

dot11QAPMissingAckRetryLimit		INTEGER,

dot11EDCAAveragingPeriod			INTEGER , }

dot11SAERetransPeriod			INTEGER,

dot11SAEThresh				INTEGER }

Add the following to the end of the dot11OperationsEntry Table in Annex D

dot11SAERetransPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DEFVAL { 40 }

DESCRIPTION

“This object specifies the initial retry timeout, in millisecond units,

used by the SAE authentication and key establishment protocol.”

::= {dot11OperationEntry 21 }

dot11SAEThresh OBJEC-TYPE

SYNTAX INTEGER

MAC-ACCESS read-only

STATUS current

DEFVAL { 5 }

DESCRIPTION

“This object specifies the maximum number of SAE protocol instances allowed

to simultaneously be in either Commit or Confirmed state.”

::= { dot11OperationEntry 22 }

Delete the following from the dot11MeshPointConfig Table in Annex D

dot11SAEThresh OBJEC-TYPE

SYNTAX INTEGER

MAC-ACCESS read-only

STATUS current

DEFAULT { 0 }

DESCRIPTION

“This object specifies the maximum number of SAE protocol instances allowed

to simultaneously be in either Commit or Confirmed state.”

::= { dot11OperationEntry 21 }