IEEE P802.11
Wireless LANs

| Channel Switch across the regulatory class comment resolution text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |

Abstract

This document provides the resolution to the CID995 relating to the channel switch acrossing the regulatory classes.

## Summary of the intention of this document

- Provide a guideline how the mesh STA should switch channel when it intends to change the channel to a channel in the different regulatory class.

## Suggested update to the draft spec

- Use the Supported Regulatory Class information element (newly defined) to announce the supported regularotry class information.

- This information is present in probe response frames, which is transmitted upon the reception of the probe request frame. Beacon frame may not include this information to mitigate the beacon bloat.

- Unlike the Country IE contained in AP beacon frames the Supported Regulatory Class IE is a purely informative and does not require neighoring mesh STA to comply with the country information indicated.

- When a mesh STA detects the need to switch to a different frequency channel it should select a frequency channel that can meet the requirements that are indicated by the Supported Regulatory Class IE from its neighbors.

- Channel swichting remains a best effort concept. There is no guarantee that the MBSS can be re-build on the new frequency channel as it was established before.

Apply the following changes.
Corresponding changes to D2.02 or the later version is indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only.

- Beacon frame format

Change the contents of the of Error: Reference source not found-Beacon frame body as follows:

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 4 | Service Set Identifier (SSID) | When dot11MeshEnabled is true and the interface on which the beacon is being sent is not configured as an Access Point, the SSID information element is set to the wildcard value as described in Error: Reference source not found. |

Insert the following additional rows (preserving their order) in Error: Reference source not found-Beacon frame body just before the Vendor Specific information element

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 25 | Mesh ID | The Mesh ID information element may be present within Beacon frames when dot11MeshEnabled is true. |
| 26 | Mesh Configuration | The Mesh Configuration information element may be present within Beacon frames when dot11MeshEnabled is true. |
| 27 | Mesh TIM | The Mesh TIM element may be present in Beacon frames generated by the MP when dot11MeshEnabled is true. |
| 28 | Awake Window | The Mesh Awake Window information element may be present within Beacon frames when dot11MeshEnabled is true. |
| 29 | Synchronization Protocol | The Synchronization Protocol information element may be present within Beacon frames when dot11MeshEnabled is true. |
| 30 | Supported Regulatory Class | The Supported Regulatory Class information element may be present within Beacon frames when dot11MeshEnabled is true and dot11RegulatoryClassesImplemented is true. |
| 31 | Beacon Timing | The Beacon Timing information element may be present within Beacon frames when dot11MeshEnabled is true. |
| 32 | MDAOP Advertisements | The MDAOP Advertisements information element may be present within Beacon frames when dot11MeshEnabled is true and the MP supports MDA. |
| 33 | MSCIE | The MSCIE element may be present when dot11MeshEnabled is true. |

- Probe Response frame format

Insert the following additional rows (preserving their order) in before the last row of Probe Response frame body-Probe Response frame body just before the Vendor Specific information element:

| - Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 23 | Mesh ID | The Mesh ID information element may be present within Probe Response frames when dot11MeshEnabled is true. |
| 24 | Mesh Configuration | The Mesh Configuration information element may be present within Probe Response frames when dot11MeshEnabled is true. |
| 25 | Awake Window | The Mesh Awake window element may be present within Probe Response frames when dot11MeshEnabled is true. |
| 26 | Synchronization Protocol | The Synchronization Protocol information element may be present within Beacon frames when dot11MeshEnabled is true. |
| 27 | Supported Regulatory Class | The Supported Regulatory Class information element is present within Probe Response frames when dot11MeshEnabled is true and dot11RegulatoryClassesImplemented is true. |
| 28 | Beacon Timing | The Beacon Timing information element may be present within Probe Response frames when dot11MeshEnabled is true. |
| 29 | MDAOP Advertisements | The MDAOP Advertisements information element may be present within Beacon frames when dot11MeshEnabled is true and the MP supports MDA. |
| 30 | MSCIE | The MSCIE element is present when dot11MeshEnabled and dot11RSNAEnabled are set to true. |

- Information elements

Insert the following rows (ignoring the header row and footer note) in Element IDsinput.doc—Element IDs in the correct position to preserve ordering by the “Element ID” column and update the “Reserved” range of codes appropriately.

| - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| Information element | Element ID | Length (in octets) | Extensible |
| Mesh Configuration Error: Reference source not found |  | 21 |  |
| Mesh ID Error: Reference source not found |  | 2 to 34 |  |
| Link Metric Report Error: Reference source not found |  | 3 to 257 |  |
| Congestion Notification Error: Reference source not found |  | 10 |  |
| Peer Link Management Error: Reference source not found |  | 5 to 9 |  |
| Supported Regulatory Class (see 7.3.2.xx) |  | 5 to 257 |  |
| Mesh Channel Switch Announcement Error: Reference source not found |  | 17 |  |
| Mesh TIM Error: Reference source not found |  | 6 to 256 |  |
| Awake Window Error: Reference source not found |  | 4 |  |
| Beacon Timing Error: Reference source not found |  | 7 to 257 |  |
| Synchronization Protocol Supported Regulatry Classs element |  | 6 |  |
| MDAOP Setup Request Error: Reference source not found |  | 7 |  |
| MDAOP Setup Reply Error: Reference source not found |  | 4 or 6 |  |
| MDAOP Advertisements Error: Reference source not found |  | 3 to 257 |  |
| MDAOP Set Teardown Error: Reference source not found |  | 3 or 9 |  |
| Portal Announcement (PANN) Error: Reference source not found |  | 15 |  |
| Root Announcement (RANN) Error: Reference source not found |  | 23 |  |
| Path Request (PREQ) Error: Reference source not found |  | 39 to 257 |  |
| Path Reply (PREP) Error: Reference source not found |  | 34 to 257 |  |
| Path Error (PERR) Error: Reference source not found |  | 14 |  |
| Proxy Update (PU) Error: Reference source not found |  | 12 to 252 |  |
| Proxy Update Confirmation (PUC) Error: Reference source not found |  | 10 |  |
| MSCIE Error: Reference source not found |  | 9 |  |
| MSAIE Error: Reference source not found |  | 17 to 257 |  |
| Anti-Clogging Token Error: Reference source not found |  | 10-255 |  |
| NOTE-The length of an element marked “See NOTE” is specified in this Table, however additional fields may be added in future revisions, with new fields appearing following the existing fields. |  |  |  |

Note to Editor:

Insert the following subclause before the subclaue describing Mesh Channel Switch element

- Supported Regulatry Classs element

The Supported Regulatory Class element contains a list of Regulatory classes and the channel subbands in which a mesh STA is capable of operating. The format of the Supported Regulatory Class element is shown in Figure sXX.

| Element ID | Length | Regulatory Class #1 | First Channel Number #1 | Number of Channels #1 | … | Regulatory Class #N | First Channel Number #N | Number of Channels #N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | … | 1 | 1 | 1 |

- Figure sXX – Supported Regulatory Class element

The Element ID is set to the value given in Element IDsinput.doc for this information element. The Length field for this information element indicates the number of octets in the information field (fields following the Element ID and Length fields).

One triplet (Regulatory Class, FirstChannel Number, Number of Channels) is used to specify each Regulatory class.

The New Regulatory Class field is set to the number of the regulatory class as defined in Annex J.

The First Channel Number field is set to the first channel (as defined in 17.3.8.3.3) in a subband of supported channels.

The Number of Channels field is set to the number of channels in a subband of supported channels.

The Supported Regulatory Class element is included in Probe Response frames, and may be included in Beacon frames. The use of the Supported Regulatory Class element is described in 11B.4 (Mesh network channel switching).

- Mesh network channel switching

Note to Editor:

Add the following subclause “11B.4.2 Mesh network channel switching” to the end of the clause 11B.4.

11B.4.2 Channel Switch across the Regulatory class

When dot11RegulatoryClassesImplemented is true and the mesh STAs is capable of operating in the multiple regulatory classes, a mesh STA may include the Supported Regulatory Class element within its Beacon frame and shall include the Supported Regulatory Class element within its Probe Response frame, in order to announce its supported regulartry classes. Parsing this information element, each mesh STA can obtain the regulatory classses supported by the neighboring mesh STAs.

When dot11RegulatoryClassesImplemented is true, mesh STAs may switch the operating channel to a channel in the different Regulatory class. However, the mesh STA that initiate the channel switch attempt across the regulatory class shall verify that all of its neighbor mesh STAs indicate that the New Regularotry Class is supported by these mesh STAs. If the neighbor mesh STA does not include Supported Regulatory Class element in the Beacon frame, it may issue Probe Request frame to receive the Supported Regulatory Class through Probe Response frame.

Note: It is possible that a channel switch is not successful in moving all neighbor peer MPs to the new channel, since the mesh STA that initiates the channel switch attempt does not know if the 2-hop neighbors support the new Regulatory class. The channel switch across the Regulatory class should be utilized as the last choice in the selection of the new channel.

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D2.02, Sept 2008.

[2] “Letter Ballot 126 Comment Resolutions”,  11-08/0493r24, November 2008.