IEEE P802.11
Wireless LANs

| SSPN update for Broadcast and multicast MPDU transfer procedure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |

Abstract

This proposal addresses the following CIDs: 4126, 4128, 4129, 4130, 4158, 4402, 4517, 4518, 4519, 4520, 4567, 4670, 4671, 4672, 4673, 4674, 4777, 4122, 4123, 4124, 4125, 4176, 4243, 4515, 4871

This document is based on 802.11u-d4.01.

9.2.7 Broadcast and multicast MPDU transfer procedure

Change the first paragraph of 9.2.7 as follows:

In the absence of a PCF, when broadcast or group addressed MPDUs are transferred from a STA with the To DS field clear, only the basic access procedure shall be used. Regardless of the length of the frame, no RTS/ CTS exchange shall be used. In addition, no ACK shall be transmitted by any of the recipients of the frame. Any broadcast or group addressed MPDUs transferred from a STA with a To DS field set shall, in addition to conforming to the basic access procedure of CSMA/CA, obey the rules for RTS/CTS exchange and the ACK procedure because the MPDU is directed to the AP. When dot11SSPNInterfaceEnabled is true, an AP shall distribute the broadcast/multicast message into the BSS only if dot11NonAPStationAuthSourceMulticast in the dot11InterworkingEntry identified by the source MAC address of the frame is set to TRUE. When dot11SSPNInterfaceEnabled is false, tThe broadcast/multicast message shall be distributed into the BSS. The STA originating the message shall receive the message as a broadcast/multicast message. Therefore, all STAs shall filter out broadcast/multicast messages that contain their address as the source address. When dot11SSPNInterfaceEnabled is false, bBroadcast and multicast MSDUs shall be propagated throughout the ESS. When dot11SSPNInterfaceEnabled is set to TRUE, broadcast and multicast MSDUs shall be propagated throughout the ESS only if dot11NonAPStationAuthSourceMulticast in the dot11InterworkingEntry identified by the source MAC address of the frame is set to TRUE.