IEEE P802.11
Wireless LANs

| Proposed Changes to IEEE 802.11u Draft |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Osama Aboul-Magd | Nortel Networks | 3500 Carling Avenue Ottawa, ONT, K2H-8E9 Canada | 613-763-5827 | osama@nortel.com |

Abstract

This submission proposes changes to the current revision of the IEEE 802.11u draft related to QoS mapping.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7.3.2.63 QoS Map Set element

The QoS Map Set is transmitted from an AP to a non-AP STA and provides the mapping from the DSCP field contained in the packet’s IP header to the user priority that is used for transmission over the air. The QoS Map Set element is shown in Figure 7-95at.

The Length field is set to 16+2n, where n is the number of Exception elements in the QoS Map set.DSCP exception elements are optionally included in the QoS Map Set. If included, the QoS Map Set has a maximum of 21 exception elements. The format of the exception element is shown in Figure 7-95au.

T.3.2 Example of QoS Mapping from different networks

IEEE 802.1p map to EDCA ACs, as described in Table 9-1. The use of DSCP sets differs from network to network. Table T-1 shows examples of DCSP mappings.

NOTE—The mapping of the DSCP to 3GPP Traffic Class is available in Annex A of 3GPP TR23.836 V1.0.0 (similar to that of GSMA IREG34). The Table T-1 is extended to cover the EDCA ACs mapping. This mapping may also apply to other networks that adopt the 3GPP QoS definitions, e.g., 3GPP2.

Table T-2—Example Enterprise DSCP to UP/AC mapping

| Application Class (RFC 4594) | PHB | 802.1D User Priority | Access Category |
| --- | --- | --- | --- |
| Network Control | CS6 | 7 | AC_VO |
| Telephony | EF | 6 | AC_VO |
| RT Interactive | CS4 | 6 | AC_VO |
| Multimedia Conf | AF4x | 5 | AC_VI |
| Signaling | CS5 | 5 | AC_VI |
| Broadcast Video | CS3 | 4 | AC_VI |
| Multimedia Stream | AF3x | 4 | AC_VI |
| Low Latency Data | AF2x | 3 | AC_BE |
| High Throughput Data | AF1x | 2 | AC_BE |
| OAM | CS2 | 2 | AC_BE |
| Standard | DF | 1 | AC_BE |
| Low Priority/Background | CS1 | 0 | AC_BK |

Table T-2 shows an example mapping based on application classes defined in RFC 4594. Mapping between DSCP and UP can be done using exceptional elements or by range. The use of exceptional elements will map a DSCP to a UP according to Table T-2. Mapping by range will require the setting of DSCP ranges as shown in Table T-x.

Table-x: UP to DSCP Rnage Mapping Example

|  | DSCP Low | DSCP High |
| --- | --- | --- |
| UP 0 Range | 0 | 0 |
| UP 1 Range | 1 | 9 |
| UP 2 Range | 10 | 16 |
| UP 3 Range | 17 | 23 |
| UP 4 Range | 24 | 31 |
| UP 5 Range | 32 | 40 |
| UP 6 Range | 41 | 47 |
| UP 7 Range | 48 | 63 |

Furthermore mapping by range will require an additional exceptional element to map DSCP 32 to UP 6.