IEEE P802.11
Wireless LANs

| Dropped Frames Count |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

This proposal addresses comments from CIDs #4390 and #4391 from LB#137.

This proposal is based on draft D4.01

Insert the following text to Clause 6.2.1:

6.2.1 MAC Data Services

6.2.1.1 MA-UNITDATA.request

6.2.1.1.4 Effect of receipt

Insert the following text after the first paragraph of 6.2.1.14.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data and subtypes 0 to 3, inclusive, then the AP’s MAC sublayer 0 to 3, inclusive, the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBestEffortRate in the dot11InterworkingEntry identified by the source MAC address of the received frame. The specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, the following operations shall be performed by the AP’s MAC sublayer:

—dot11NonAPStationBestEffortMSDUCount shall be incremented by 1.

—dot11NonAPStationBestEffortOctetCount shall be incremented by the number of octets in the MSDU.

If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedBestEffortMSDUCount shall be incremented by 1.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data and subtypes 8 to 11, inclusive (QoS data subtypes), and the priority is an integer in the range of 0 to 7, inclusive, then the AP’s MAC sublayer shall derive the access category from the priority using the mapping in Table 9-1. The AP’s MAC sublayer shall retrieve the MIB variables listed below from the dot11InterworkingEntry identified by the source MAC address of the received frame and perform the following operations:

—If the access category is AC_VO, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthVoiceRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate limiting mechanism does not discard the frame, then dot11NonAPStationVoiceMSDUCount shall be incremented by 1 and dot11NonAPStationVoiceOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedVoiceMSDUCount shall be incremented by 1.

—If the access category is AC_VI, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthVideoRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationVideoMSDUCount shall be incremented by 1 and dot11NonAPStationVideoOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedVideoMSDUCount shall be incremented by 1.

—If the access category is AC_BE, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBestEffortRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationBestEffortMSDUCount shall be incremented by 1 and dot11NonAPStationBestEffortOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedBestEffortMSDUCount shall be incremented by 1.

—If the access category is AC_BK, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBackgroundRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationBackgroundMSDUCount shall be incremented by 1 and dot11NonAPStationBackgroundOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedBackgroundMSDUCount shall be incremented by 1.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data whose priority is an integer in the range of 8 to 15, inclusive, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthHccaRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationHCCAMSDUCount shall be incremented by 1, and dot11NonAPStationHCCAOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedHCCAMSDUCount shall be incremented by 1.

6.2.1.2 MA-UNITDATA.indication

6.2.1.2.4 Effect of receipt

insert the following text after the first paragraph of 6.2.1.2.4.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of a group addressed frame of type data, then the AP’s MAC sublayer shall discard the frame if dot11NonAPStationAuthSourceMulticast is false in the dot11InterworkingEntry identified by the source MAC address of the received frame. If dot11NonAPStationAuthSourceMulticast is true, the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthSourceMulticastRate in the dot11InterworkingEntry identified by the source MAC address of the received frame. The specific mechanism to perform rate limiting is outside the scope of this specification.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data and subtypes 0 to 3, inclusive, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBestEffortRate in the dot11InterworkingEntry identified by the source MAC address of the received frame. The specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, the following operations shall be performed by the AP’s MAC sublayer:

—dot11NonAPStationBestEffortMSDUCount shall be incremented by 1

—dot11NonAPStationBestEffortOctetCount shall be incremented by the number of octets in the MSDU

If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedBestEffortMSDUCount shall be incremented by 1.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data and subtypes 8 to 11, inclusive (QoS data subtypes), and where the priority is an integer in the range of 0 to 7, inclusive, then the AP’s MAC sublayer shall derive the access category from the priority using the mapping in Table 9-1. The AP’s MAC sublayer shall retrieve the MIB variables listed below from the dot11InterworkingEntry identified by the source MAC address of the received frame and perform the following operations:

—If the access category is AC_VO, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthVoiceRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationVoiceMSDUCount shall be incremented by 1 and dot11NonAPStationVoiceOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedVoiceMSDUCount shall be incremented by 1.

—If the access category is AC_VI, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthVideoRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationVideoMSDUCount shall be incremented by 1 and dot11NonAPStationVideoOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedVideoMSDUCount shall be incremented by 1.

—If the access category is AC_BE, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBestEffortRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationBestEffortMSDUCount shall be incremented by 1 and dot11NonAPStationBestEffortOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedBestEffortMSDUCount shall be incremented by 1.

—If the access category is AC_BK, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBackgroundRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationBackgroundMSDUCount shall be incremented by 1 and dot11NonAPStationBackgroundOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedBackgroundMSDUCount shall be incremented by 1.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data whose priority is an integer in the range of 8 to 15, inclusive, then, the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthHccaRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationHCCAMSDUCount shall be incremented by 1, and dot11NonAPStationHCCAOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism discards the frame, then dot11NonAPStationDroppedHCCAMSDUCount shall be incremented by 1.

Insert the following text to Annex D:

Dot11InterworkingEntry ::=

SEQUENCE {

dot11NonAPStationMACAddress 			MACAddress,

dot11NonAPStationUserIdentity 			SnmpAdminString,

dot11NonAPStationInterworkingCapability 		BITS,

dot11NonAPStationAssociatedSSID 		OCTET STRING,

dot11NonAPStationUnicastCipherSuite 		OCTET STRING,

dot11NonAPStationBroadcastCipherSuite 		OCTET STRING,

dot11NonAPStationAuthAccessCategories 		BITS,

dot11NonAPStationAuthMaxVoiceRate 		Unsigned32,

dot11NonAPStationAuthMaxVideoRate 		Unsigned32,

dot11NonAPStationAuthMaxBestEffortRate 	Unsigned32,

dot11NonAPStationAuthMaxBackgroundRate 	Unsigned32,

dot11NonAPStationAuthMaxVoiceOctets 		Unsigned32,

dot11NonAPStationAuthMaxVideoOctets 		Unsigned32,

dot11NonAPStationAuthMaxBestEffortOctets 	Unsigned32,

dot11NonAPStationAuthMaxBackgroundOctets 	Unsigned32,

dot11NonAPStationAuthMaxHCCAOctets 		Unsigned32,

dot11NonAPStationAuthMaxTotalOctets		Unsigned32,

dot11NonAPStationAuthHCCA 			TruthValue,

dot11NonAPStationAuthMaxHCCARate 		Unsigned32,

dot11NonAPStationAuthHCCADelay 		Unsigned32,

dot11NonAPStationMaxAuthSourceMulticastRate 	Unsigned32,

dot11NonAPStationVoiceMSDUCount 		Counter32,

dot11NonAPStationVideoMSDUCount 		Counter32,

dot11NonAPStationBestEffortMSDUCount 			Counter32,

dot11NonAPStationBackgroundMSDUCount 		Counter32,

dot11NonAPStationHCCAMSDUCount 			Counter32,

dot11NonAPStationVoiceOctetCount 			Counter32,

dot11NonAPStationDroppedVoiceMSDUCount 		Counter32,

dot11NonAPStationDroppedVideoMSDUCount 	Counter32,

dot11NonAPStationDroppedBestEffortMSDUCount 	Counter32,

dot11NonAPStationDroppedBackgroundMSDUCount 	Counter32,

dot11NonAPStationDroppedHCCAMSDUCount 	Counter32,

Insert the following text to Annex D:

dot11NonAPStationDroppedVoiceMSDUCount Counter32

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" For EDCA operation, this counter shall be incremented for each MSDU dropped by the AP on the voice access category."

::= { dot11InterworkingEntry 39 }

dot11NonAPStationDroppedVideoMSDUCount Counter32

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“For EDCA operation, this counter shall be incremented for each MSDU dropped by the AP on the video access category."

::= { dot11InterworkingEntry 40 }

dot11NonAPStationDroppedBestEffortMSDUCount Counter32

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“For EDCA operation, this counter shall be incremented for each MSDU dropped by the AP on the best effort access category and for each MSDU successfully received on either user priority 0 or 3. For DCF or PCF operation, this counter shall be incremented for each MSDU dropped by the AP."

::= { dot11InterworkingEntry 41 }

dot11NonAPStationDroppedBackgroundMSDUCount Counter32

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For EDCA operation, this counter shall be incremented for each MSDU dropped by the AP on the background access category"

::= { dot11InterworkingEntry 42}

dot11NonAPStationDroppedHCCAMSDUCount Counter32

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“For HCCA operation, this counter shall be incremented for each MSDU dropped by the AP.”

::= { dot11InterworkingEntry 43 }