IEEE P802.11
Wireless LANs

| 802.11p Dallas 2008 PICs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Randy Roebuck | Sirit Inc / OmniAir | 1321 Valwood Parkway, Suite 620, Carrollton, TX | 972-243-6244 x2123 | RDRoebuck@sirit.com |
| Wayne Fisher | ARINC, Inc. | 2551 Riva Road, Annapolis, MD | 410-266-4958 | wfisher@arinc.com |
| Alastair Malarky | Mark IV | 6030 Ambler Drive, Mississaugua, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |

Abstract

Adjust PICs to reflect the 802.11p draft 4.02 + approved submissions (1148, 1276, 1277, 1278, 1279, 1280 1281 & 1376).  It addresses LB125 comments # 410 (Roebuck), 411 (Marshall), 414 (Stephens) & 416 (Ecclesine).  Also, It corrects 1279r2 language for PICs.

Motion # 1

- Background and Rationale:

In review of 1279r2 language and completing PIC section, it was identified as no rescription exists on scanning, authentication and association.  It is not believed on this change was intended.

- Proposed Change:

Editorial please change the wording on 11.a on the following sentence from:

11.a STAs communicating outside the context of a BSS

A STA may communicate outside of the context of a BSS. A STA communicating outside the context of a BSS may transmit and receive frames without scanning or MAC sublayer authentication or association procedures. When dot11OCBEnabled is FALSE a STA shall not communicate outside the context of a BSS.

Please change the wording to:

11.a STAs communicating outside the context of a BSS

A STA may transmit and receive frames outside of the context of a BSS.  When dot11OCBEnabled is TRUE, a STA shall not perform scanning or MAC sublayer authentication or association. When dot11OCBEnabled is FALSE a STA shall not communicate outside the context of a BSS.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the recommendations of this submission, and instruct the editor to incorporate the proposed revisions provided into the amendment.

- Motion by: _Roebuck___________________Date: __11/13/2008_______________

- Second:  _Malarky_____________________

| Approve: 13 | Disapprove: 1 | Abstain: 2 |
| --- | --- | --- |

Motion # 2

- Background and Rationale:

In review existing PICs in IEEE 802.11p draft 4.02 and Dallas 2008 motions, this submission proposes changes to rectify this section:

Editor, please replace the existing PICs tables with:

- Proposed Modifications to P802.11p PICs as following:

A.4.3 IUT Configuration

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF2.1 | Independent station operating outside context of BSS | -- | CF<n> | Yes[] No[] |
| *CF<n+1> | 5.850-5.925 GHz band in United States | -- | CF6&CF8&CF10&CF11:O | Yes[] No[] |
| *CF<n> | Communications outside context of BSS supported (dot11OCBEnabled is true) | 11.a, Annex J | CF<n+1>:M | Yes[] No[] |

A.4.4.1 MAC protocol capabilities

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| PC1.1.a | No authentication state | 5.7.6,5.7.7,8.1 | CF2.1:M | Yes[]No[] N/A[] |
| PC11.7.1 | No passive scanning | 11.a | CF2.1:M | Yes[]No[] N/A[] |
| PC11.8.1 | No active scanning | 11.a | CF2.1:M | Yes[]No[] N/A[] |
| PC11.11 | TSF timer in a STA that has the MIB attribute dot11OCREnabled set to true | 11.a.2 | CF2.1:M | Yes[]No[] N/A[] |
| PC11.12 | Incrementing & maintaining of TSF timer | 10.3.c.1.1 | CF<n>:M | Yes[]No[] N/A[] |
| PC<n.1> | No STA communication outside BSS if dot11OCBEnabled is false | 11.a | CF<n>:M | Yes[]No[] N/A[] |
| PC<n.2> | No STA communication within BSS if dot11OCBEnabled is true | 11.a | CF<n>:M | Yes[]No[] N/A[] |
| PC<n.3> | Only one EDCA parameter set used on current channel outside of BSS if dot11OCBEnabled is true | 11.a | CF<n>:M | Yes[]No[] N/A[] |

A.4.4.2 MAC frames

| Item | MAC frame | References | Status | Support |
| --- | --- | --- | --- | --- |
| FT7.1 | Timing and information frame | Clause 7 | CF<n>:M | Yes[]No[] N/A[] |
| FR7.1 | Timing and information frame | Clause 7 | CF<n>:M | Yes[]No[] N/A[] |

A.4.4.4 MAC addressing function

| Item | MAC addressing function | References | Status | Support |
| --- | --- | --- | --- | --- |
| AD<n> | MAC addressing when dot11OCBEnabled is true. | 7.1.3.3.3 | CF<n>:M | Yes[]No[] N/A[] |
| AD<n+1> | MAC & PHY operation resumes with appropriate MIB attributes less than 2 TU | 11.a | CF<n>:M | Yes[]No[] N/A[] |

A.4.8 OFDM PHY function

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| *OF1.7 | 10 MHz Channel spacing | 17.2.2, 17.2.3, 17.2.3.3 | CF11:O CF<n+1>:M | Yes[]No[] N/A[] |

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| OF3: PMD Operating Specification General |  |  |  |  |
| *OF3.2.7 | 5.85-5.925 GHz band | Annex J | CF<n>:M | Yes[]No[] N/A[] |
| OF3.3.n | 5.9 GHz band (10 MHz channel spacing) | Annex J | CF<n+1>:M | Yes[]No[] N/A[] |
| OF3.3.n+1 | 5.9 GHz band (20 MHz channel spacing) | Annex J | CF<n+1>:O | Yes[]No[] N/A[] |
| OF3.3.n+2 | 5.9 GHz band (5 MHz channel spacing) | Annex J | CF<n+1>:O | Yes[]No[] N/A[] |
| OF3.10.4 | Type 4 (-40 oC to 85 oC) | 17.3.8.8 | O | Yes[]No[] |
| OF4: PMD Transmit Specification |  |  |  |  |
| OF4.1.4 | Power Level (5.850-5.925 GHz) | I.2.3 | CF<n+1>:M | Yes[]No[] N/A[] |
| OF4.15a | Spectrum mask, Class A (10 MHz channel spacing) | I.2.3 | CF<n+1>:M | Yes[]No[] N/A[] |
| OF4.15b | Spectrum mask, Class B (10 MHz channel spacing) | I.2.3 | CF<n+1>:O | Yes[]No[] N/A[] |
| OF4.15c | Spectrum mask, Class C (10 MHz channel spacing) | I.2.3 | CF<n+1>:O | Yes[]No[] N/A[] |
| OF4.15d | Spectrum mask, Class D (10 MHz channel spacing) | I.2.3 | CF<n+1>:O | Yes[]No[] N/A[] |
| OF5.2.1 | Optional adjacent channel rejection | 17.3.10.2 | O | Yes[]No[] N/A[] |
| OF5.3.1 | Optional nonadjacent channel rejection | 17.3.10.3 | O | Yes[]No[] N/A[] |

A.4.15 QoS enhanced distributed channel access (EDCA)

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QD<n> | Default EDCA parameters for communications outside context of BBS | 7.3.2.29 | CF<n>:M | Yes[]No[] N/A[] |

- Motion # 2 (if technical and/or significant):

(And instructions to the editor.)

Move to accept the recommendations of this submission, and instruct the editor to incorporate the proposed revisions provided into the amendment.

- Motion by: __Randy Roebuck_  Date: _11/13/2008_

- Second:  __Francois Simon_

| Approve: 16 | Disapprove: 0 | Abstain: 0 |
| --- | --- | --- |

- References:

The following documents have been considered in the assessment herein:

- IEEE Std 802.11-2007

IEEE P802.11p-D4.02

IEEE 802-11k-2008

IEEE 802-11r-2008

IEEE 802.11y-2008

Approved submissions (1148, 1276, 1277, 1278, 1279, 1280 1281 & 1376) from IEEE P802.11p D4.02