IEEE P802.11
Wireless LANs

| Proposed Resolutions for Comment Group #7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-10-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jesse Walker | Intel Corporation | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 712 1849 | jesse.walker@intel.com |

Abstract

This document contains proposed resolutions to the following comments from SB on D6.0

- Comment Group #7 (CID 109, 153, 155, 156, 157)

All changes apply to TGw draft D6.0, unless specified otherwise.

# Comment Group #7

Proposed Resolution to CID 109 as “Accept”, “See CID 157”

Proposed Resolution to CID 153 as “Accept”, “See CID 157”

Proposed Resolution to CID 155 as “Counter”, “See CID 157”

Proposed Resolution to CID 156 as “Accept”, “See CID 157”

Proposed Resolution to CID 157 as “Accept”, “Resolutions as in doc 11-08/1409

Proposed Resolution to CID 223 as “Counter”, Updated from “See CID 270…” to “See CID 157…”

Proposed Resolution to CID 268 as “Counter”, “See CID 157”

Proposed Resolution to CID 270 as “Counter”, “See CID 157”

In Section 3 add

xxx. Robust Action frame: an Action frame which is not a Public Action frame.

In Section 5.4.3.8, replace “Action” with “Robust Action”

p. 24, line 1, replace “Robust Action frame” with Robust Management frame”

p. 31, lines 1-2: change the text as follows: shall discard unprotected unicast Robust Action frames received from any STA that advertised MFPC=1, and it shall discard received unprotected unicast Disassociation and Deauthentication frames from a STA that advertised MFPC=1 after the PTK and IGTK have been installed. The receiver shall process unprotected unicast Disassociation and Deauthentication frames before the PTK and IGTK are installed.

p. 31, lines 8-9: change the text as follows: protected unicast Robust Action frames to and from any STA, shall not transmit unprotected unicast Robust Action frames to any STA, and shall discard unprotected unicast Robust Action frames received from any STA, and it shall discard received unprotected unicast Disassociation and Deauthentication frames from a STA after the PTK and IGTK have been installed. The receiver shall process unprotected unicast Disassociation and Deauthentication frames before the PTK and IGTK are installed.

P 31, line 33, replace “management Action frames” with “Robust Management frames”

Add the following as the last paragraph of 8.4.11 (page 31):

Note that Robust Management frame protection cannot be applied until the PTK and IGTK has been established with the STA. Hence, a STA shall not transmit Robust Action frames until it has installed the PTK for the peer STA, or, in the case of broadcast/multicast, has installed the IGTK, and shall also discard any Robust Action frames received before the PTK and IGTK are installed.

Replace the content of 8.4.12 (page 33) as follows:

A STA with dot11RSNAProtectedManagementFramesEnabled set to TRUE shall negotiate Robust Management frame protection with a STA that advertised MFPC=1.

Replace the contents of 8.7.2.1a with the following:

if ((dot11RSNAEnabled = TRUE) and (frame is a Robust Management frame)) then

if ((dot11RSNAProtectedManagementFramesEnabled = FALSE) then

Transmit the MMPDU without protection

else // dot11RSNAProtectedManagementFramesEnabled = TRUE

if (dot11RSNAUnprotectedManagementFramesAllowed = TRUE) then

if (MMPDU has an individual RA) then

if (peer STA advertised MFPC = 1) then

if (Pairwise key exists for the MMPDU's RA) then

// Note that it is assumed that no entry in the key

// mapping table will be of an unsupported cipher.

Transmit the MMPDU, to be protected after fragmentation

// see 8.7.2.2a

else if (Robust Action frame) then

// pairwise key was not found

Discard the MMPDU and generate an MLME.confirm primitive to notify the SME that the MMPDU was not delivered

else // Disassociation or Deauthentication

Transmit the MMPDU without protection

endif

else // (peer STA didn’t advertised MFPC = 1)

Transmit the MMPDU without protection

endif

else // MMPDU has a multicast/broadcast RA

if (IGTK exists) then

// if we find a suitable IGTK

Transmit the MMPDU with protection // See 8.7.2.2a

else

Discard the MMPDU and generate an MLME.confirm primitive to notify the SME that the MMPDU was undeliverable

endif

endif

else // dot11RSNAUnprotectedManagementFramesAllowed = FALSE

if (MMPDU has an individual RA) then

if (peer STA advertised MFPC = 1) then

if (Pairwise key exists for the MMPDU's RA) then

// Note that it is assumed that no entry in the key

// mapping table will be of an unsupported cipher.

Transmit the MMPDU, to be protected after fragmentation

// see 8.7.2.2a

else if (Robust Action frame) then

// pairwise key was not found

Discard the MMPDU and generate an MLME.confirm primitive to notify the SME that the MMPDU was not delivered

else // FrameControlSubType is Disassociation or Deauthentication

Transmit the MMPDU without protection

endif

else // peer STA didn’t advertise MFPC = 1

Discard the MMPDU and generate an MLME.confirm primitive to notify the SME that the MMPDU was not delivered

endif

else // MMPDU has a multicast/broadcast RA

if (IGTK exists) then

// if we find a suitable IGTK

Transmit the MMPDU with protection // See 8.7.2.2a

else

Discard the MMPDU and generate an MLME.confirm primitive to notify the SME that the MMPDU was undeliverable

endif

endif

endif

endif

else // (dot11RSNAEnabled = FALSE) or (not a Robust Management Frame)

Use 8.7.2.1 to transmit the frame

endif

Replace the contents of 8.7.2.3a with the following:

if ((dot11RSNAEnabled = TRUE) and (frame is a Robust Management frame)) then

if ((dot11RSNAProtectedManagementFramesEnabled = FALSE) then

if (Protected Frame subfield of the Frame Control field is set to 1) then

Discard the frame

else

Receive the MMPDU

endif

else // dot11RSNAProtectedManagementFramesEnabled = TRUE

if (dot11RSNAUnprotectedManagementFramesAllowed = TRUE) then

if (STA with frame TA advertised MFPC = 0) then

if (Protected Frame subfield of the Frame Control field is set to 1) then

Discard the frame

else

Receive the MMPDU

endif

else // STA with frame TA advertised MFPC = 1

if (MMPDU has an individual RA) then

if (Pairwise key does not exist) then

if (frame is a Disassociation or Deauthenticate) then

Make the MPDU available for futher processing

else // frame is not a Disassociation or Deauthenticate

Discard the frame

endif

else if (security association has an AES-CCM key) then

if (Protected Frame subfield of the Frame Control field is set to 0) then

//unprotected frame

Discard the frame

else // frame is encrypted

if (PN is not sequential) then

Discard the MPDU as a replay

Increment dot11RSNAStatsCCMPReplays

else

Decrypt frame using AES-CCM key

if (the integrity check fails) then

Discard the frame

Increment dot11RSNAStatsCCMPDecryptErrors

else

Make the MPDU available for further processing

endif

endif

endif

else // key for some other cipher – for future expansion

endif

else // MMPDU has a multicast/broadcast RA

if (IGTK does not exist) then

Discard the frame

else // IGTK exists

if (MMIE is not present) then

Discard the frame

else // MMIE is present

if (AES-1280CMAC IGTK) then

if (PN is not valid) then

Discard the frame as a replay

Increment dot11RSNAStatsCMACReplay

else if (security association has an AES-128-CMAC IGTK) then

Discard the frame

Increment dot11RSNAStatsCMACICVError

else

Make frame available for further processing

endif

else // some other kind of key – for the future

endif

endif

endif

endif

endif

else // dot11RSNAUnprotectedManagementFramesAllowed = FALSE

if (MMPDU has an individual RA) then

if (peer STA advertised MFPC = 1) then

if (Pairwise key exists for the MMPDU's RA) then

if (security association has an AES-CCM key) then

if (Protected Frame subfield of the Frame Control field is set to 0) then

//unprotected frame

Discard the frame

else // frame is encrypted

if (PN is not sequential) then

Discard the MPDU as a replay

Increment dot11RSNAStatsCCMPReplays

else

Decrypt frame using AES-CCM key

if (the integrity check fails) then

Discard the frame

Increment dot11RSNAStatsCCMPDecryptErrors

else

Make the MPDU available for further processing

endif

endif

endif

else // key for some other cipher – for future expansion

endif

else if (Robust Action frame) then

Discard the frame

else // FrameControlSubType is Disassociation or Deauthentication

Make frame available for processing

endif

else // peer STA didn’t advertise MFPC = 1

Discard the frame

endif

else // MMPDU has a multicast/broadcast RA

if (IGTK exists) then

if (MMIE is not present) then

Discard the frame

else // MMIE is present

if (AES-1280CMAC IGTK) then

if (PN is not valid) then

Discard the frame as a replay

Increment dot11RSNAStatsCMACReplay

else if (security association has an AES-128-CMAC IGTK) then

Discard the frame

Increment dot11RSNAStatsCMACICVError

else

Make frame available for further processing

endif

else // some other kind of key – for the future

endif

endif

else // IGTK does not exist

Discard the frame

endif

endif

endif

endif

else // (dot11RSNAEnabled = FALSE) or (not a Robust Management Frame)

Use 8.7.2.3 to receive the frame

endif

p 49, line 36, replace “Ping request action management frame” with “SAQuery Request frame”.

p. 50, lines 25 and 60, replace “Ping Response action management frame” with “SAQuery Response frame”.

p. 52, line 4, replace “Ping Response action management frame” with “SAQuery Response frame”.