IEEE P802.11
Wireless LANs

| Figures for Flags fields in HWMP and Proxy IEs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Michael Bahr | Siemens AG | Otto-Hahn-Ring 6, 80200 München, Germany | +49-89-636-49926 | bahr at siemens dot com |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd – Lake Mary, FL 32771 -- USA | +14075624050 | guenael.strutt at motorola dot com |
| Javier Cardona | cozybit Inc. | 165 Jessie St., San Francisco, CA 94105 | +1-415-974-6770 | javier at cozybit dot com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave Sunnyvale, CA 94085 | 408-331-6835 | audeh at tropos dot com |

Abstract

This submission provides the requested graphical structure for Flags fields of HWMP information elements PREQ, PREP, and RANN and Proxy Update information elements PU and PUC in response to comments from letter ballot LB 126.

# Base version of text

The following changes are based on IEEE 802.11s draft standard D2.02.

# Resolved CIDs

The following 8 CIDs will be resolved with this submission:

1388, 1548, 595, 1389, 1549, 1856, 1390, 1552.

The proposed changes to 1655 are also reflected in the new text in clause 7.3.2.96 (PREQ).

# Instruction to editor:

In clause 7.3.2.95 RANN information element, replace paragraph:

The Flags field is set as follows. Bit 0: Portal Role (0 = non-portal, 1 = portal). Bit 1 – 7: Reserved

with:

The format of the Flags field is shown in Figure sb1.

| B0 | B1 B7 |
| --- | --- |
| Portal Role | Reserved |
| Bits: 1 | 7 |

Figure sb1— RANN Flags field format

The Flags field is set as follows.

Bit 0: Portal Role (0 = non-portal, 1 = portal). Bit 1 – 7: Reserved.

Bit 1-7: Reserved.

The above change addresses CIDs 1388, 1548.

# Instruction to editor:

In clause 7.3.2.96 PREQ information element, replace paragraph:

The Flags field is set as follows. Bit 0: Portal Role (0 = non-portal, 1 = portal), Bit 1: (0 = group addressed, 1 = individually addressed) (see 11B.9.3), Bit 2: Proactive PREP (0 = off, 1 = on), Bit 3 – 5: Reserved, Bit 6: Address Extension (AE) (1= (destination count ==1 && proxied device address present), 0 = otherwise), Bit 7: Reserved.

with:

The format of the Flags field is shown in Figure sb2.

| B0 | B1 | B2 | B3 B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- |
| Portal Role | Addressing Mode | Proactive PREP | Reserved | AE | Reserved |
| Bits: 1 | 1 | 1 | 3 | 1 | 1 |

Figure sb2— PREQ Flags field format

The Flags field is set as follows.

Bit 0: Portal Role (0 = non-portal, 1 = portal).

Bit 1: Addressing Mode (0 = group addressed, 1 = individually addressed) (see 11B.9.3).

Bit 2: Proactive PREP (0 = off, 1 = on).

Bit 3-5: Reserved.

Bit 6: Address Extension (AE) (1= (destination count ==1 && proxied device address present), 0 = otherwise).

Bit 7: Reserved.

The above change addresses CIDs 595, 1389, 1549, 1856. They also anticipate the acception of the resolution to CID 1655 and follow up the idea of this resolution in this paragraph.

# Instruction to editor:

In clause 7.3.2.97 PREP element, replace paragraph:

The Flags field is set as follows. Bit 0-5: Reserved. Bit 6: Address Extension (AE) (1=proxied address present, 0=otherwise). Bit 7: Reserved.

with:

The format of the Flags field is shown in Figure sb3.

| B0 B5 | B6 | B7 |
| --- | --- | --- |
| Reserved | AE | Reserved |
| Bits: 6 | 1 | 1 |

Figure sb3— PREP Flags field format

The Flags field is set as follows.

Bit 0-5: Reserved.

Bit 6: Address Extension (AE) (1=proxied address present, 0=otherwise).

Bit 7: Reserved.

The above change addresses CIDs 1390, 1552.

# Instruction to editor:

In clause 7.3.2.99 Proxy Update (PU) information element, replace paragraph:

The Flags field is set as follows. Bit 0: (0 = add proxy information, 1 = delete proxy information). Bit 1 – 7: Reserved

with:

The format of the Flags field is shown in Figure sb4.

| B0 | B1 B7 |
| --- | --- |
| Add / Delete | Reserved |
| Bits: 1 | 7 |

Figure sb4— PU Flags field format

The Flags field is set as follows.

Bit 0: 0 = add proxy information, 1 = delete proxy information.

Bit 1 – 7: Reserved

The above change addresses no specific CIDs but implements the idea of the resolved comments in this section.

# Instruction to editor:

In clause 7.3.2.100 Proxy Update Confirmation (PUC) information element, replace paragraph:

The Flags field is set as follows. Bit 0 – 7: Reserved

with:

The Flags field is reserved for future use.

The above change addresses no specific CIDs but implements the idea of the resolved comments in this section.