IEEE P802.11
Wireless LANs

| Text Changes for QoS STA function applied to Mesh STA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youko Omori | NEC Corporation | 1753, Shimonumabe Nakahara-ku, Kawasaki, Kanagawa 211-8666, Japan | +81-44-396-2663 | omori@az.jp.nec.com |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |
| Dee Denteneer | Philips | Philips Research, HTC 27 (WL 1.132), 5656 AE Eindhoven, The Netherlands | +31-402-746-937 | dee.denteneer@philips.com |
| Guido R. Hiertz | Philips | ComNets, Department of Communication Networks, Faculty 6, RWTH Aachen University, Kopernikusstr. 5, 52074 Aachen, Germany | +49-241-802-5829 | hiertz@ieee.org |

Abstract

This document provides resolution to the CIDs relating to QoS STA.

The proposed text resolves CID 362, 1187, 1486, 1495, 1496, and 1186.

5.2.11.1 Introduction to mesh

Modify the text in the clause as shown below:

An example Mesh is illustrated in Figure s2. Mesh Stations mesh STAare QoS STAs that support mesh services, i.e. they participate in interoperable formation and operation of the mesh network. The QoS functionality of a mesh STA is limited as follows. Mesh STAs support non-AP STA operation under the HCF using TXOPs gained through the EDCA mechanism that defined in 9.9.1. Since a Mesh BSS (MBSS) has no HC either of HCCA, polled TXOP operation, admission control or TSPEC setup are not applicable for mesh STAs. Mesh STAs may utilize No Ack, No explicit acknowledgment and Block Ack operation. Amesh STA may be collocated with one or more other entities (e.g., AP, portal, etc.). The implementation of collocated entities is beyond the scope of this standard. The configuration of a mesh STA that is collocated with an Access Point is referred to as a Mesh Access Point (MAP). Such a configuration allows a single entity to logically provide both mesh functionalities and AP functionalities simultaneously. STAs associate with APs to gain access to the network. Only mesh STAs participate in mesh functionalities such as path selection and forwarding. Mesh STAs  interface the network to other IEEE 802 LAN segments. Figure s2 illustrates this.

7.1.3.4.1 Sequence Number field

Modify the text in the clause as shown below:

QoS STAs associated in a QoS BSS and mesh STAs associated in an MBSS maintain one modulo-4096 counter, per TID, per unique receiver (specified by the Address 1 field of the MAC header). Sequence numbers for QoS data frames are assigned using the counter identified by the TID subfield of the QoS Control field of the frame, and that counter is incremented by 1 for each MSDU belonging to that TID. Sequence numbers for management frames, QoS data frames with a broadcast/multicast address in the Address 1 field, and all non-QoS data frames sent by QoS STAs or mesh STAs are assigned using an additional single modulo-4096 counter, starting at 0 and incrementing by 1 for each MSDU or MMPDU. Sequence numbers for QoS (+)Null frames may be set to any value.

7.1.3.5 QoS Control field

Modify the text in the clause as shown below:

The QoS Control field is a 16-bit field that identifies the TC or TS to which the frame belongs and various other QoS-related information about the frame that varies by frame type and subtype. The QoS Control field is present in all data frames in which the QoS subfield of the Subtype field is set to 1 (see 7.1.3.1.2). Each QoS Control field comprises five subfields, as defined for the particular sender (HC, or non-AP STA or mesh STA) and frame type and subtype. The usage of these subfields and the various possible layouts of the QoS Control field are described 7.1.3.5.1 through 7.1.3.5.7 and illustrated in Table 7-4.

Table 7-4—QoS Control field

| Applicable frame (sub) types | Bits 0–3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8–15 |
| --- | --- | --- | --- | --- | --- |
| QoS (+)CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP Limit |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS data frames sent by non-AP STAs | TID | 0 | Ack Policy | Reserved | TXOP Duration Requested |
|  | TID | 1 | Ack Policy | Reserved | Queue Size |
| Mesh STAs | TID | EOSP | Ack Policy | Reserved | Reserved |

7.1.3.5.2 EOSP (end of service period) subfield

Insert the following sentence at the end of the text in 7.1.3.5.2:

The usage of EOSP subfield used by a Mesh STA is described in Clause 11B.12.10.

9.1.3.1 HCF contention-based channel access (EDCA)

Modify the text in the clause as shown below:

For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function

(EDCAF), contends for TXOPs using a set of EDCA parameters from the EDCA Parameter Set element except when no EDCA Parameter Set element is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA. In case no EDCA Parameter Set element is received from the AP of the BSS with which the STA is associated or when the STA is a mesh STA it utilizes the default values for the parameters, where

Modify the text in the clause as shown below:

b) The minimum specified idle duration time is not the constant value (DIFS) as defined for DCF, but is a distinct value (contained in the MIB attribute table dot11QAPEDCATableAIFSN for an AP and in the MIB table dot11EDCATableAIFSN for a non-AP STA; see 9.9.1) assigned either by a management entity or by an AP. For a mesh STA the minimum specified idle duration time is contained in the MIB table dot11EDCATableAIFSN assigned by a management entity.

Modify the text in the clause as shown below:

The management frames shall be sent using the access category AC_VO without being restricted by admission control procedures. A QoS STA or mesh STA shall also send management frames using the access category AC_VO before associating with any BSS, even if there is no QoS facility available in that BSS. BlockAckReq and BlockAck control frames shall be sent using the same QoS parameters as the corresponding QoS data frames. PS-Poll control frames shall be sent using the access category AC_BE to reduce the likelihood of collision following a Beacon frame. For the purpose of determining the proper AC for an RTS frame, the RTS frame shall inherit the UP of the data or management frame(s) that are included in the frame exchange sequence where the RTS is the first frame.

9.9.1.2 EDCA TXOPs

Modify the text in the clause as shown below:

It should be noted, that when transmitting multiple frames in a TXOP using acknowledgment mechanisms

other than Normal Ack, a protective mechanism should be used (such as RTS/CTS or the protection

mechanism described in 9.13). A QoS AP may send broadcast/multicast frames without using any protection

mechanism. In a QoS IBSS and MBSS, broadcast/multicast frames shall be sent one at a time, and backoff shall be

performed after the transmission of each of the broadcast/multicast frames.

9.9.1.5 EDCA backoff procedure

Modify the text in the clause as shown below:

- If the QSRC[AC] or the QLRC[AC] for the QoS STA or mesh STA has reached dot11ShortRetryLimit or dot11LongRetryLimit respectively, CW[AC] shall be reset to CWmin[AC].

9.9.1.6 Retransmit procedures

Modify the text in the clause as shown below:

QoS STAs and mesh STAs shall maintain a short retry counter and a long retry counter for each MSDU or MMPDU that belongs to a TC requiring acknowledgment. The initial value for the short and long retry counters shall be zero. QoS STAs and mesh STAs also maintain a short retry counter and a long retry counter for each AC. They are defined as QSRC[AC] and QLRC[AC], respectively, and each is initialized to a value of zero.

After transmitting a frame that requires acknowledgment, the STA shall perform the acknowledgment procedure, as defined in 9.2.8. The short retry count for an MSDU or MMPDU and the QSRC[AC] shall be incremented every time transmission of a MAC frame of length less than or equal to dot11RTSThreshold fails for that MSDU or MMPDU. This short retry count and the QoS STA QSRC[AC]  and mesh STAs  OSRC[AC] shall be reset when a MAC frame of length less than or equal to dot11RTSThreshold succeeds for that MSDU or MMPDU. The long retry count for an MSDU or MMPDU and the QLRC[AC] shall be incremented every time transmission of a MAC frame of length greater than dot11RTSThreshold fails for that MSDU or MMPDU. This long retry count and the QLRC[AC] shall be reset when a MAC frame of length greater than dot11RTSThreshold succeeds for that MSDU or MMPDU. All retransmission attempts for an MPDU that has failed the acknowledgment procedure one or more times shall be made with the Retry field set to 1 in the data or management frame.

Retries for failed transmission attempts shall continue until the short retry count for the MSDU or MMPDU is equal to dot11ShortRetryLimit or until the long retry count for the MSDU or MMPDU is equal to dot11LongRetryLimit. When either of these limits is reached, retry attempts shall cease, and the MSDU or MMPDU shall be discarded.

For internal collisions occurring with the EDCA access method, the appropriate retry counters (short retry counter for MSDU or MMPDU and QSRC[AC] or long retry counter for MSDU or MMPDU and QLRC[AC]) are incremented. For transmissions that use Block Ack, the rules in 9.10.3 also apply. STAs shall retry failed transmissions until the transmission is successful or until the relevant retry limit is reached.

With the exception of a frame belonging to a TID for which Block Ack is set up, a QoS STA or a mesh STA shall not initiate the transmission of any management or data frame to a specific RA while the transmission of another management or data frame with the same RA and having been assigned its sequence number from the same sequence counter has not yet completed to the point of success, retry fail, or other MAC discard (e.g., lifetime expiry).

QoS STAs and mesh STAs shall maintain a transmit MSDU timer for each MSDU passed to the MAC. The MIB attribute dot11EDCATableMSDULifetime specifies the maximum amount of time allowed to transmit an MSDU for a given AC. The transmit MSDU timer shall be started when the MSDU is passed to the MAC. If the value of this timer exceeds the appropriate entry in dot11EDCATableMSDULifetime, then the MSDU, or any remaining, undelivered fragments of that MSDU, shall be discarded by the source STA without any further attempt to complete delivery of that MSDU.

- PICS proforma - IEEE Std 802.11, 2006 Edition

- MAC protocol

- MAC protocol capabilities

Insert the following to end of table in A.4.4.1:

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *PC36 | Wireless LAN Mesh | Error: Reference source not found | O | Yes No  |
| PC36.1 | Mesh key holder security association | Error: Reference source not found | PC34&PC36:M | Yes No  |
| PC36.2 | Mesh key transport protocol | Error: Reference source not found | PC34&PC36:O | Yes No N/A  |
| PC36.3 | EAP Encapsulation Mechanism | Error: Reference source not found | PC34&PC36:O | Yes No N/A  |
| PC36.4 | Mesh Service Supported | Error: Reference source not found | CF12:M | Yes No  |

A.4.14 QoS base functionality

Modify thetable in the clause as shown below:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| QB1 | QoS frame format | 7.2.1.1–7.2.1.3, 7.2.2, 7.2.3.1, 7.2.3.4–7.2.3.7, 7.2.3.9, 7.2.3.12 | (CF12 or MP1):M | Yes No N/A  |
| QB2 | Per traffic identifier (TID) duplicate detection | 7.1.3.4, 7.1.3.5, 9.2.9 | (CF12 or MP1):M | Yes No N/A  |
| QB3 | Decode of no-acknowledgment policy in QoS data frames | 7.1.3.5.3, 9.9.1.4, 9.9.1.5, 9.9.3.1, 9.9.3.2 | (CF12 or MP1):M | Yes No N/A  |
| QB4 | Block Acknowledgments (Block Acks) | 7.2.1.7, 7.2.1.8, 7.4.4, 9.10, 11.5 | (CF12 or MP1):O | Yes No N/A  |
| QB5 | Automatic power-save delivery (APSD) | 7.4.2, 11.2.1 | CF12:O | Yes No N/A  |
| QB6 | Direct-link setup (DLS) | 7.3.2.20, 7.4.3, 10.3.12, 11.7 | (CF1 AND CF12):M (CF2 AND CF12):O | Yes No N/A  |

A.4.15 QoS enhanced distributed channel access (EDCA)

Modify thetable in the clause as shown below:

| Item | Protocol Capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| QD1 | Support for four transmit queues with a separate channel access entity associated with each | 9.1.3.1, 9.9.1.1 | (CF12 or MP1):M | Yes No N/A  |
| QD2 | Per-channel access function differentiated channel access | 9.9.1.2, 9.9.1.3, 9.9.1.5 | (CF12 or MP1):M | Yes No N/A  |
| QD3 | Multiple frame transmission support | 9.9.1.4 | (CF12 or MP1):O | Yes No N/A  |
| QD4 | Maintenance of within-queue ordering, exhaustive retransmission when sending non-QoS data frames | 9.9.1.6 | (CF12 or MP1):M | Yes No N/A  |
| QD5 | Interpretation of admission control mandatory (ACM) bit in EDCA Parameter Set element | 7.3.2.14, 9.9.3.1 | (CF2 & CF12):M | Yes No N/A  |
| QD6 | Contention-based admission control | 9.9.3.1, 7.3.2.15, 7.3.2.16, 7.4.2.1– 7.4.2.3, 11.4 | (CF1 & CF12):O (CF2 & CF12):O | Yes No N/A  |
| QD7 | Power management | 11.2 | CF12:O | Yes No N/A  |

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D2.02, Sept 2008.

[2] “Letter Ballot 126 Comment Resolutions”,  11-08/0493r25, November 2008.

[3] IEEE P802.11-2007, June 2007