IEEE P802.11
Wireless LANs

| Proposal to resolve comments related to clause 5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-2-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| John Kenney | Toyota InfoTechnology Center, USA |  | 574-272-1403 | johnkenney@alumni.nd.edu |

Abstract

Proposes resolutions to CIDs: 6-9, 11, 13, 15, 17-19, 21-22 of LB141.  Note that the file name of this submission refers to D5.0, but in this revision the changes are shown relative to D5.01.

|  | LB141 Comment Resolution |
| --- | --- |

- Comments addressed in this submission:  [From Spreadsheet]

This submission proposes resolutions to a set of comments from LB141 that were assigned to John Kenney.  The resolution includes proposed text changes to draft 802.11p, clauses 5.2.2a, 5.3.1, and 11.a.  Section 2 below shows the D5.01 text of these clauses.  Section 3 shows the proposed revisions. Two variations on the proposed revision for 11.a are shown, one that only clarifies the term “communication outside the context of a BSS” and one that additionally re-organizes the paragraph for clarity.  Section 4 shows a proposed revision to Clause 5.2.6, related to CID 6

Note that CID 13 is shown as accepted in the comment resolution master spreadsheet, and is not explicitly addressed in this submission.   In addition, CIDs 10, 12, and 20 are addressed in submission 09-0020, and so are not addressed in this submission.

| ID | Commenter | Clause/ Pg/ Ln/ Type | Comment | Suggested Remedy | Resolution | Comment Resolution | Resolution Document |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Roy, Richard | 5/2/29 E | In support of the several comments in the previous LB 125 that stated clause 5 should contain more "justification' for the additional functionality in the p amendment, 1375r1 proposes just such changes. | As given in 11-08-1375-01-000p-clause 5 changes.doc | Counter | After reviewing 11-08-1375r2, some of the suggestions are accepted, some are accepted in principle, and others are declined. I recommend we accept the addition to 5.2.6 on QoS. We should discuss the recommended change to 5.1.1.3 (handling mobile STAs). I do not recommend that we change 5.1.1.5, 5.2, or 5.4.3.5. My recommendations regarding the comments on 5.2.2a are incorporated in my suggested revision in Section 3. |  |
| 7 | Stephens, Adrian | General/2/30 T | Explain to me (and in clause 5) why the TSF time is relevant for OCB communication. The point is that it is used to manage beaconing and power-saving. Seeing as neither are used in OCB. And seeing that TSF is a property of a BSS (which doesn't exist in OCB), I don't understand what it is being used for. The notes (e.g. 15.17) kind of hint at a higher layer global timing sync using TSF, it would be nice to state this more clearly. | Put explanation in clause 5 or remove all TSF-related stuff from this amendment. | Counter | Accept in principle. Actual text is TBD. Note this comment applies to the document generally, not to clause 5. On the 1/8/09 teleconference the group agreed informally that further explanation of the role of the TSF timer for communication OCB is warranted in Clause 11.a.2, but not in Clause 5. We need text. Following the informal consensus on the 2/19/09 teleconference to remove the SET and INC TSF primitives, perhaps we can do with less additional explanation to satisfy this comment. But we still have the GET primitive, so some explanation is needed. |  |
| 8 | Emeott, Stephen | 5.2.2.a/ 2/ 37 TR | The concept of a station communicating outside the the context of a BSS has enough similarities to the IBSS mode (e.g. stations communicate directly, do not associate with one another, security is optional, beacons sent to advertise presences) that it should be built upon IBSS mode and called IBSS on-demand or something similar. There is no compelling reason to define a special mode given all the similarities to IBSS. Moreover, building the amendment upon the IBSS mode allows the IBSS beacon and/or probe responses to be reused and eliminates the need to consume a new managment subframe type (see 7.1.3.1.2). | Make the suggested change | Decline | While the commenter has identified some similarities with IBSS mode, the task group is motivated by use cases that have important differences with IBSS communication. For example, in one important use case a STA broadcasts data with no consideration of whether other STAs are in its vicinity, let alone any prior setup with potential recipients of its data. There is no IBSS-like organization between one STA and another. In another important use case a fixed STA offers a service to potential client STAs (probably mobile) within its range. Again, there is no IBSS-like organization between the server and client at the MAC sublayer. Note also that the amendment no longer uses the beacon frame. See also CID 34 regarding the management subframe type. |  |
| 9 | Roy, Richard | 5.2.2.a/ 2/ 37 TR | The rewrite of this subclause contains several confusing and misleading statements (see 1375r1). | As given in 11-08-1375-01-000p-clause 5 changes.doc | Counter | After reviewing 11-08-1375r2, some of the suggestions are accepted, some are accepted in principle, and others are declined. My specific recommendations regarding the comments on 5.2.2a are incorporated in my suggested revision in Section 3. |  |
| 11 | Chu, Liwen | 5.2.2.a/ 2/ 40 TR | The statement of "A STA will communicate outside the context of a BSS only if dot11OCBEnable is set to true. The data frames can be sent to either unicast or groupcast destination MAC addresses" is not true. Public action frames are also used for the communication of outside the context of BSS. Also Communication outside the context of a BSS does not just include data frames. Management frames such as "Timing and information" frame, ADDBA etc. and control frames can also be transmitted. | Change the sentences to "If a STA sets dot11OCBEnabled to true, it will communicate outside the context of a BSS. Public action frames can also be used for outside the context of BSS communication. When dot11OCBEnabled is set to true, public action frames are not used for outside the context of BSS communication." Removing the sentence "A STA will communicate outside the context of a BSS only if dot11OCBEnable is set to true" from the draft. | Counter | Accept in principle. See revised 5.2.2a text. The phrase “outside the context of a BSS” is now explicitly limited to data frames. The text now explicitly states that the state of dot11OCBEnbabled does not affect the validity of any control or management frame, with the exceptions for scanning, authentication, and association noted in Clause 11.a. |  |
| 13 | Emeott, Stephen | 5.2.2.a/ 2/ 43 ER | Sentence should read "This type of communication, which is only possible …" | Make the suggested change | Accepted | Accepted as per Suggested Remedy | 09-0004r0 |
| 15 | Noens, Richard | 5/ 3/ 4 TR | The sentence "Rather than scanning to find other STAs with which to communicate outside the context of a BSS, a STA with dot11OCBEnabled set to true will initially transmit and receive on a channel known a priori, either through regulatory designation or some other out of band communications" does not make sense. | Fix this sentence to convey the intended meaning. | Accept | Removed reference to scanning and communication within a BSS. See revised 5.2.2a text. |  |
| 17 | Stephenson, Dave | 5.2.2a/ 3/ 5 TR | It would be helpful to have an explanatory comment stating how the SME will determine changes in the operating channel. E.g., via out-of-band communication or some OTA frame exchange. | Per comment. | Accept | See revised 5.2.2a text. |  |
| 18 | Stephens, Adrian | 5.2.2.a/ 3/ 6 T | "A STA's SME will determine PHY layer parameters (e.g. data rate), as well as any changes in operating channel." This is not so. The data rate is a parameter of the TXVECTOR. Unless there is an MLME interface that provides control of this to the SME (there isn't), only the MLME can do this. | Replace SME with MLME, or add a suitable interface to the MLME to allow it to be controlled. | Counter | Accept in principle. Deleted specific mention of data rate. See revised 5.2.2a text. |  |
| 19 | Chu, Liwen | 5.2.2.a/3/ 10 TR | "The BSSID of a frame sent outside the context of a BSS will either be the wildcard BSSID or a non-wildcard BSSID determined by a higher layer or the SME (see Clause 7.1.3.3.3)." Public action frames will not use wildcard BSSID and non-BSSID is decided by MLME. So the sentence should be changed to "The BSSID of a frame sent by a STA with dot11OCBEnabled set to true will either be the wildcard BSSID or a non-wildcard BSSID determined by a higher layer or the SME (see Clause 7.1.3.3.3)." | As proposed. | Counter | Accept in principle. The new language refers to dot11OCBEnabled rather than to “frames sent outside the context of a BSS”. |  |
| 21 | Malarky, Alastair | 5.3.1/ 3/ 21 T | The intent of the indicated change was to show that items a, b & c are not applicable when operating outside the context of a BSS. However the indicated change prevents them being used in an IBSS. This conflicts with 5.8.3 in the standard. | Change added text to "(BSS and IBSS operation only)" | Counter | See revised 5.3.1 text. |  |
| 22 | Stephens, Adrian | 5.3.1/ 3/ 21 T | The term "BSS operation" is not defined. | Define the term or remove the additions. | Accept | See revised 5.3.1 text. |  |

- Background:  Existing D5.01 text for clauses 5.2.2a, 5.3.1, and 11.a

Insert the following new subclause (5.2.2.a) after the last subclause in 5.2.2 inserting the appropriate

subclause numbers:

5.2.2.a STA communication outside the context of a BSS

In addition to defining STA communication within a BSS, this standard also allows communication between 802.11 STAs outside the context of a BSS. A STA will communicate outside the context of a BSS only if dot11OCBEnabled is set to true. Communication outside the context of a BSS involves the exchange of data frames between STAs that are not members of a BSS. The data frames can be sent to either unicast or groupcast destination MAC addresses. This type of communication, which is possible when IEEE 802.11 STAs are able to communicate directly, allows immediate communication, avoiding the latency associated with establishing a BSS. The transmitting and receiving STAs do not join a BSS or utilize the 802.11 authentication or association services. This capability is particularly well-suited for use in rapidly varying communication environments such as those involving mobile STAs where the interval over which the communication exchanges take place may be of very short-duration (e.g. measured in milliseconds). Since 802.11 MAC sublayer authentication services are not used when exchanging frames outside the context of a BSS, any required authentication services would be provided by the station management entity (SME) or by applications outside of the MAC sublayer. Communication between STAs outside the context of a BSS might take place in a frequency band that is dedicated for its use, and such bands might require licensing depending on the regulatory domain. STAs that do not have the MIB variable dot11OCBEnabled defined operate as if dot11OCBEnabled is set to false.

Rather than scanning to find other STAs with which to communicate outside the context of a BSS, a STA with dot11OCBEnabled set to true will initially transmit and receive on a channel known in advance, either through regulatory designation or some other out of band communication. A STA's SME will determine PHY layer parameters (e.g. data rate), as well as any changes in operating channel. The Timing Advertisement frame (see clause 7.2.3.a) provides one means for STAs to exchange management information (e.g. supported rates and QoS parameters) prior to communicating outside the context of a BSS. The BSSID of a frame sent outside the context of a BSS will either be the wildcard BSSID or a nonwildcard BSSID determined by a higher layer or the SME (see Clause 7.1.3.3.3). A STA with dot11OCBEnabled set to true might be connected to a network, but the specification of that network is outside the scope of this standard.

5.3.1 SS

Change the lettered items (a) - (c) of Clause 5.3.1 as follows:

a) Authentication (BSS operation only)

b) Deauthentication (BSS operation only)

c) Data confidentiality (BSS operation only)

11.a STAs communicating outside the context of a BSS

A STA may communicate outside the context of a BSS. When dot11OCBEnabled is true, a STA shall not perform scanning or MAC sublayer authentication or association. When dot11OCBEnabled is false a STA shall not communicate outside the context of a BSS. When a STA is a member of a BSS or is the point coordinator within a BSS, dot11OCBEnabled shall be set to false. When dot11OCBEnabled is true a STA shall not communicate within a BSS. A STA that communicates outside the context of a BSS shall use only one EDCA parameter set for accessing the medium on the current channel. The EDCA parameter set used outside the context of a BSS is either the default EDCA parameter set specified in table 7-37a or it is set by the SME in dot11EDCATable. If a STA does not have dot11OCBEnabled defined, the attribute is viewed as set to false.

The SME may change the parameters of the MAC and PHY sublayers as well as change the MAC address using the MLME-RESET.request primitive and setting MIB attributes. After the MAC sublayer is reset by the SME or MIB attributes are changed, MAC and PHY sublayer operation shall resume with the appropriate

MIB attributes in less than 2 TU.

- Proposed new text for clauses 5.2.2a, 5.3.1, and 11.a

5.2.a STA transmission of data frames outside the context of a BSS

In addition to defining procedures for IEEE 802.11 STA communication within a BSS, this standard also allows a STA that is not a member of a BSS to transmit data frames.  Such data frames are defined as being transmitted outside the context of a BSS.  A STA will transmit a data frame outside the context of a BSS only if dot11OCBEnabled is set to true.

When dot11OCBEnabled is true a data frame can be sent to either a unicast or a groupcast destination MAC address. This type of communication is only possible between STAs that are able to communicate directly over the wireless medium. It allows immediate communication, avoiding the latency associated with establishing a BSS. When dot11OCBEnabled is true a STA is not a member of a BSS and it does not utilize the 802.11 authentication or association services. This capability is particularly well-suited for use in rapidly varying communication environments such as those involving mobile STAs where the interval over which the communication exchanges take place may be of very short-duration (e.g. measured in milliseconds). Since 802.11 MAC sublayer authentication services are not used when dot11OCBEnabled is true, any required authentication services would be provided by the station management entity (SME) or by applications outside of the MAC sublayer. STAs that do not have the MIB variable dot11OCBEnabled defined operate as if dot11OCBEnabled is set to false.

Communication of data frames when dot11OCBEnabled is true might take place in a frequency band that is dedicated for its use, and such bands might require licensing depending on the regulatory domain.  A STA with dot11OCBEnabled set to true will initially transmit and receive on a channel known in advance, either through regulatory designation or some other out-of-band communication. A STA's SME will determine PHY layer parameters, as well as any changes in operating channel, e.g. using out-of-band communication or over-the-air frame exchange. The Timing Advertisement frame (see clause 7.2.3.a) provides one means for STAs to exchange management information (e.g. supported rates and QoS parameters) prior to communicating data frames outside the context of a BSS. The BSSID field of a data frame sent by a STA with dot11OCBEnabled set to true will be the wildcard BSSID value. A STA with dot11OCBEnabled set to true might be connected to a network, but the specification of that network is outside the scope of this standard.

5.3.1 SS

Change the lettered items (a) - (c) of Clause 5.3.1 as follows:

a) Authentication (this service is not active when dot11OCBEnabled is true)

b) Deauthentication (this service is not active when dot11OCBEnabled is true)

c) Data confidentiality (this service is not active when dot11OCBEnabled is true)

First proposed modification to 11.a (only clarifies “communication outside the context of a BSS”):

11.a STAs communicating outside the context of a BSS

A STA may transmit data frames outside the context of a BSS. When dot11OCBEnabled is true, a STA shall not

perform scanning or MAC sublayer authentication or association. When dot11OCBEnabled is false a STA

shall not transmit data frames outside the context of a BSS. When a STA is a member of a BSS or is the point coordinator

within a BSS, dot11OCBEnabled shall be set to false. When dot11OCBEnabled is true a STA shall

not communicate within a BSS. A STA that communicates outside the context of a BSS shall use only one

EDCA parameter set for accessing the medium on the current channel. The EDCA parameter set used outside

the context of a BSS is either the default EDCA parameter set specified in table 7-37a or it is set by the

SME in dot11EDCATable. If a STA does not have dot11OCBEnabled defined, the attribute is viewed as set to false.

The SME may change the parameters of the MAC and PHY sublayers as well as change the MAC address using the MLME-RESET.request primitive and setting MIB attributes. After the MAC sublayer is reset by the SME or MIB attributes are changed, MAC and PHY sublayer operation shall resume with the appropriate

MIB attributes in less than 2 TU.

Second proposed modification to 11.a (also re-structures first paragraph for clarity):

11.a STAs communicating outside the context of a BSS

When dot11OCBEnabled is true:

- a STA may transmit data frames outside the context of a BSS

- a STA shall not perform scanning or MAC sublayer authentication or association

- a STA shall not communicate within a BSS

- a STA shall use only one EDCA parameter set for accessing the medium on the current channel (either the default EDCA parameter set specified in Table 7-37a of this standard or the EDCA parameter set specified in dot11EDCATable of the STA’s MIB)

When dot11OCBEnabled is false a STA shall not transmit data frames outside the context of a BSS. When a STA is a member of a BSS or is the point coordinator within a BSS, dot11OCBEnabled shall be set to false.

If a STA does not have dot11OCBEnabled defined, the attribute is viewed as set to false.

The SME may change the parameters of the MAC and PHY sublayers as well as change the MAC address using the MLME-RESET.request primitive and setting MIB attributes. After the MAC sublayer is reset by the SME or MIB attributes are changed, MAC and PHY sublayer operation shall resume with the appropriate

MIB attributes in less than 2 TU.

- Proposed change to Clause 5.2.6 (as suggested in CID 6 via 11-08-1375-002)

Change the second paragraph in 5.2.6 to read as follows (insert the yellow highlighted text):

5.2.6 QoS BSS: The QoS network

The enhancements that distinguish QoS STAs from non-QoS STAs and QoS APs from non-QoS APs are collectively termed the QoS facility. The quantity of certain, QoS-specific, mechanisms may vary among QoS implementations, as well as between QoS STAs and QoS APs, over ranges specified in subsequent

clauses. All service primitives, frame formats, coordination function and frame exchange rules, and management interface functions except for the Block Acknowledgment (Block Ack) function, direct-link setup (DLS), and automatic power-save delivery (APSD) are part of the core QoS facilities. A QoS STA or QoS AP must implement those core QoS facilities necessary for its QoS functions to interoperate with other STAs in the BSS or to interoperate with other STAs

when exchanging QoS data frames outside the context of a BSS. Functions such as the Block Ack, DLS, and APSD are separate from the core QoS facilities; and the presence of these functions is indicated by STAs separately from the core QoS facilities. A comprehensive statement on mandatory and optional functionalities is available in Annex A.

- Motion

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D5.01 noted in sections 3 and 4 above and instruct the editor to make these changes to the latest draft of P802.11p.  With regard to Clause 11.a, adopt the “Second proposed changes”.

Motion by:                			Date:

Second:  ____		____

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

The state of dot11OCBEnabled does not affect the validity of management or control frame transmissions, except with regard to scanning, authentication, and association as noted in Clause 11.a