IEEE P802.11
Wireless LANs

| OUI Extension |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| R. Roy | Affiliation: Connexis |  | 650-861-3351 | dickroy@alum.mit.edu |

Abstract

This submission describes an extension to 802.11 to provide support for 36-bit unique organization identifiers currently being issued by the IEEE RA, including the 36-bit unique identifier assigned to IEEE 1609, an assignment of particular relevance to TGp (cf. TGp's PAR).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

To extend 802.11 to provide support for 36-bit unique organization identifiers currently being issued by the IEEE RA, including the 36-bit unique identifier assigned to IEEE 1609, an assignment of particular relevance to TGp (cf. TGp's PAR), the following changes are proposed:

7.3.2.26 Vendor Specific information element

The Vendor Specific information element is used to carry information not defined in this standard within a

single defined format, so that reserved information element IDs are not usurped for nonstandard purposes

and so that interoperability is more easily achieved in the presence of nonstandard information. The

information element is in the format shown in Figure 7-75 and requires that the first 3 or 5 octets of the

information field contain the unique organization identifier  OUI of the entity that has defined the content of the particular Vendor Specific information element. The length of the information field (n) is  3 length(OUI) <= n <=255. The OUI field shall be a contain a unique organization identifier assigned by the IEEE-RA. It The IEEE-RA currently assigns both 24-bit and 36-bit unique organization identifiers.  (A 24-bit unique organization identifier assigned by the IEEE is sometimes referred to as an OUI-24 and in many instances simply as an OUI.  A 36-bit unique organization identifier is sometimes referred to as individual address block identifier (IAB) and in some instances as an OUI-36.) The OUI field is 3 octets in length if the unique organization identifier is 24-bits in length, and 5 octets in length if the unique organization identifier is 36-bits in length.  The IEEE RA assigns 36-bit organization identifiers that have their first three bytes set to the value 0x0050C2.  The last 12 bits of these 36-bit identifiers are unique to the organization that is responsible for defining meaning of the contents of the last 4 bits of the 40-bit (5 octet) OUI field. For example, for the 36-bit unique organization identifier 0x0050C24A4, the OUI field would contain 0x0050C24A4y where the nibble y is specified by the organization whose unique organization identifier is 0x0050C24A4. The length of the vendor-specific content is n–length(OUI) 3 octets.

| Element ID | Length | OUI | Vendor-specific content |
| --- | --- | --- | --- |

Octets:            1                               1                                 3 or 5                                   n-length(OUI) 3

Figure 7-75—Vendor Specific information element format

Multiple Vendor Specific information elements may appear in a single frame. Each Vendor Specific

information element can have a different OUI value. The number of Vendor Specific information elements

that may appear in a frame is limited only by the maximum frame size.

And change clause 7.4.5 to read as follows:

7.4.5 Vendor-specific action details

The Vendor Specific Action frame is defined for vendor-specific signaling. The format of the Vendor

Specific Action frame is shown in Figure 7-101. An OUI, in the octet field immediately after the Category

field, differentiates the vendors.

| Category | OUI | Vendor Specific Content |
| --- | --- | --- |

Octets:                       1                    3 or 5                      Variable

Figure 7-101—Vendor Specific Action frame format

The Category field is set to the value indicating the vendor-specific category, as specified in Table 7-24.

The OUI field is a public OUI assigned by the IEEE. It is 3 octets in length. It contains the OUI of the entity that has defined the content of the particular vendor-specific action.

The OUI field contains a unique organization identifier assigned by the IEEE-RA. The IEEE-RA currently assigns both 24-bit and 36-bit unique organization identifiers.  (A 24-bit unique organization identifier assigned by the IEEE is sometimes referred to as an OUI-24 and in many instances simply as an OUI.  A 36-bit unique organization identifier is sometimes referred to as individual address block identifier (IAB) and in some instances as an OUI-36.) The OUI field is 3 octets in length if the unique organization identifier is 24-bits in length, and 5 octets in length if the unique organization identifier is 36-bits in length.  The IEEE RA assigns 36-bit organization identifiers that have their first three bytes set to the value 0x0050C2.  The last 12 bits of these 36-bit identifiers are unique to the organization that is responsible for defining meaning of the contents of the last 4 bits of the 40-bit (5 octet) OUI field. For example, for the 36-bit unique organization identifier 0x0050C24A4, the OUI field would contain 0x0050C24A4y where the nibble y is specified by the organization whose unique organization identifier is 0x0050C24A4.

The Vendor Specific Content contains vendor-specific field(s). The length of the Vendor Specific Content in a Vendor Specific Action frame is limited by the maximum allowed MMPDU size.

As background material, from the IEEE RA website:

The EUI64.html tutorial states:

"The IEEE administers the assignment of _/OUI-24 or OUI-36 /_ /company_id / values."

From oui.txt which can be downloaded from the website:

00-50-C2   (hex)		IEEE REGISTRATION AUTHORITY

0050C2     (base 16)		IEEE REGISTRATION AUTHORITY

445 HOES LANE

PISCATAWAY NJ 08854

UNITED STATES

Thus, the IEEE RA has assigned the three-byte identifier (OUI-24) 0x0050C2 to itself. (It has also assigned 0x001BC5 to itself but there don't seem to be any organizations assigned OUI-36's with these first three bytes.)

From iab.txt which can be downloaded from the website:

00-50-C2	(hex)		IEEE P1609 WG

4A4000-4A4FFF	(base 16)	IEEE P1609 WG

3800 N Fairfax Drive #207

Arlington VA 22203-1759

UNITED STATES

And it is important to note that ALL entries in this document have 00-50-C2 as the first three bytes.

Also, from

http://standards.ieee.org/regauth/registries.html

IEEE Registration Authority - Registries

Organizationally Unique Identifier (OUI)
An OUI / "company_id" is a 24-bit globally unique assigned number referenced by various standards. The OUI is usually concatenated with 24 or 40 bits by an Organization to create a 48-bit or 64-bit number that is unique to a particular piece of hardware. The OUI can be used to create MAC Addresses, Bluetooth Device Addresses or Ethernet Addresses. There are other uses of the OUI as well, such as its use as a company identifier in the SNAP protocol. 
Please review our Frequently Asked Questions and Tutorials for more information.

Individual Address Block (IAB)
An Individual Address Block is for people who need less than 4,097 unique 48-bit numbers (EUI-48) and thus find it hard to justify buying their own OUI. The IAB is a particular OUI belonging to the IEEE Registration Authority, concatenated with 12 additional IEEE-provided bits, leaving only 12 bits for the owner to assign to his (up to 4,096) individual devices. Please review our Frequently Asked Questions and Tutorials for more information.

OUI-36
An OUI-36 is a 36-bit identifier that can be used as an Individual Address Block or as an extended OUI. The OUI-36 may be appended with 4 organization-supplied bits to form a 40-bit Context Dependent Identifier (CDI-40), with 12 organization-supplied bits to form an EUI-48, or with organization-supplied 28 bits to form an EUI-64. Please review our Frequently Asked Questions and Tutorials for more information.

Thus, the aforementioned changes are completely backward compatible with all legacy devices.

Move to accept the above additions to P802.11p D5.0.

- Motion by: _____ _______________Date: _____________

- Second:  ________ ______________

| Approve: 0 | Disapprove: 0 | Abstain: 0 |
| --- | --- | --- |