IEEE P802.11
Wireless LANs

| LB140 STA Statistics Comment Resolutions Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-Jan-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |

- Abstract

This document contains normative text changes to resolve comment IDs 506 and 510.

The submission is referenced to draft D4.0 and augments to all editor instructions as well as the actual base draft modifications.

All new text relative to the draft D4.0 is identified in blue color and underlined and all deleted text is identified by strikethrough.

7.3.2.22.8 STA Statistics Report

The format of the Measurement Report field for RSNA Counters Group is shown in Figure 7-68l1.

|  | dot11RSNAStats CMACICVErrors | dot11RSNAStatsCMACReplays |  | dot11RSNAStats RobustMgmt CCMPReplays |
| --- | --- | --- | --- | --- |
| Octets: | 4 | 4 |  | 4 |
|  | dot11RSNAStats TKIPICVErrors | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays |
| Octets | 4 | 4 | 4 | 4 |
| Figure 7-68l1—Measurement Report field format for RSNA Counters Group |  |  |  |  |