IEEE P802.11
Wireless LANs

| Proxies and portals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32771 | +1 407-562-4050 | Guenael.Strutt@motorola.com |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr at siemens dot com |

Abstract

The submission is a resolution of comments 1116, 1922, 1631, 1732, 1918, 1029 and 996.

Baseline: D2.06

Instructions to the TGs editor are shown in red.  When the instruction “modify (the text) as shown” is used, red indicates a deletion and blue indicates a deletion.

| CID | Comment / Explanation | Recommended Change | Resolution Notes | What I did |
| --- | --- | --- | --- | --- |
| 1922 | "If a root receives a PU element with a add proxy information or if it detects a new proxied address in its DS (e.g. through bridge learning) and the root already has proxy information for one or more of these proxied address, it shall send a PU element with Bit 0 set to 1" Telepathy is at work here. Bridge learning occurs in a layer above the MAC. There is no clause 10 interface that communicates this data to the MLME. | Provide the clause 10 interface to support this behaviour, or remove the cited text. | BTW a STA is not in the DS yet it is a proxied entity. We should remove the "through bridge learning" text and the reference to the DS. In general, we simply want the MSTA to send information about a proxied address regardless of its origin | This comment was already resolved in D2.06 – the whole paragraph is gone. I did add a note explaining that proxied entities can be STAs associated with an AP or STAs behind a portal |
| 1631 | The PU is transmitted to "the" (not "a") destination MP. Furthermore, the PU is not a request. Use proper naming for MIB variables (dot11…) for PU_TIMEOUT and MAX_PU. | Change sentence into "The PU element is transmitted via unicast from the MP to the destionation MP. This PU element may be repeated after every dot11… for dot11… times until the MP receives a corresponding PUC element." | Group with a proxy cleanup submission. | Removed “from the mesh STA to a destination mesh STA.” |
| 1732 | It is never defined what the proxy information comprises. | Define proxy information similar to forwarding information in clause 11B.9.4.4 | Submission needed. | Modelled definition of proxy information on forwarding information. Included in the Proxy Protocol clause, just before the Proxy Update. |
| 1116 | An MAP is really an MPP with a special case of being able to use the same medium as the MPs. We can have a small section describing this subtlety, explain that DS bits are different, MAC addresses are different and that in all other respect MAPs are just MPPs. We should note that these MPPs should not use a PANN because it is an optimization that is useful only if the MPP is connected to "the wire" | Maybe create a special section in 11B.7 after the overview. Can also use this as a spot to emphasize the fact that an AP and an MP cannot share the same physical address | Change "11B.7.5 Proxy protocol" and explain what we use proxies for (portals, A Mesh access point is not a portal. Lump with proxy work | Provided examples of proxied MAC addresses |
| 996 | The portal announcement message is not well defined. 802.11-2007 provides a definition for the portal ("The logical point at which the integration service is provided" and "integration service: The service that enables delivery of medium access control (MAC) service data units (MSDUs) between the distribution system (DS) and a non-IEEE-802.11 local area network (LAN) (via a portal)"). However, the portal announcement message indicates a "a live connection to an external network." What does that mean? If a portal has a connection to an 802.3 (Ethernet) it should issue the PANN message. What happens if a higher layer protocol (e.g. Spanning Tree) has blocked the port? What is a "live connection"? "Live" with respect to which layer? Is a PHY link sufficient? Should there be IP connectivity? | Clarify the meaning of the PANN. | submission needed | The information element clause is no place to discuss those issues. |
| 1918 | "Enabling the announcement protocol in a given MPP is beyond the scope of this standard." True, but it should be possible for the MLME to discover if it is an MPP, in order to perform the normative behaviour described for it in this subclause. | Add a mib variable that is true for an MPP. Then define an MPP as a STA for which this mib variable exists and is set to true. | Lump with work on portal. Create MIB variable that drives the transmission of the PANN. | Added dot11MeshPortalAnnouncementProtocol is set to 1 |
| 1029 | "The combination of MP functionality and the IEEE 802.1D bridging functionality is called an MPP." that goes way beyond what an 802.11-2007 portal. In section M.4 802.11-2007 states "There are a number of differences between the IEEE 802.11 integration service and the service provided by an IEEE 802.1 bridge. In the IEEE 802.11 architecture, a portal provides the minimum connectivity between an IEEE 802.11 WLAN system and a non-IEEE-802.11 LAN. Requiring an IEEE 802.1D bridge in order to be compliant with IEEE Std 802.11 would unnecessarily render some implementations noncompliant. The most important distinction is that a portal has only one “port” (in the sense of IEEE Std 802.1D, for example) through which it accesses the DS. This renders it unnecessary to update bridging tables inside a portal each time a STA changes its association status. In other words, the details of distributing MSDUs inside the IEEE 802.11 WLAN need not be exposed to the portal. Another difference is that the DS is not an IEEE 802 LAN (although it carries IEEE 802 LLC SDUs). Requiring that the DS implements all behaviors of an IEEE 802 LAN places an undue burden on the architecture. Finally, it is an explicit intent of this standard to permit transparent integration of an IEEE 802.11 WLAN into another non-IEEE-802.11 LAN, including passing bridge PDUs through a portal. While an implementer may wish to attach an IEEE 802.1D bridge to the portal (note that the non-IEEE-802.11 LAN interface on the bridge need not be any particular type of LAN), it is not an architectural requirement of this standard to do so." Chapter 3 already defines the MPP. | Delete sentence "The combination of MP functionality and the IEEE 802.1D bridging functionality is called an MPP." | Good idea. Similar to comments 996 and 1918 | Removed all text that reiterates what a portal is. The standard already explains what a portal is. |

Modify 11B.7 as shown

- Interworking

- Overview of interworking in a mesh BSS

An MBSS functions like an IEEE 802 LAN segment that is compatible with IEEE 802.1D. The MBSS appears as a single access domain.

The combination of mesh STA functionality and the IEEE 802.1D bridging functionality is called a portal.

The mesh MAC entity appears as a single port to an IEEE 802.1D MAC Relay entity in IEEE 802.1D (to which it presents the Internal Sublayer Service interface) or any other higher-layer entity, such as a Spanning Tree Protocol entity.

An MBSS may have zero or more portals which may be connected to one or more LAN segments. In case two portals connect the MBSS to one external LAN segment, broadcast loops may occur and the IEEE 802.1D bridging protocol may cause the LAN ports of one of the portals to be closed. These cases can be prevented by proper configuration measures.

The default path selection protocol supports the transfer of “proxy information” towards the portal (see clause 11B.7.5 Proxy protocol). The portal shall support the Internal Sublayer Service interface to transfer this information to the MAC Relay Entity as defined in IEEE 802.1D. As a result, the bridge, or, more specifically, the Learning Process within the MAC Relay Entity will learn the addresses of all the mesh STAs and their attached stations.

- Portal announcement protocol

This subclause describes the function, generation and processing of the Portal Announcement (PANN).

Enabling the announcement protocol in a given portal is beyond the scope of this standard.

- Function

The Portal Announcement (PANN) element, described in Error: Reference source not found, is used to announce the presence of a mesh STA collocated with a configured as a portal in the mesh BSS. Portal Announcements allow mesh STAs to select the appropriate portal and build a path towards it. A mesh STA which receives a Portal Announcement message shall propagate the Portal Announcement as described below but may ignore its content.

The PANN sequence number of the Portal Announcement shall be incremented by the portal before each transmission.

- Conditions for generating and sending a PANN

A mesh STA shall propagate a PANN frame in the following cases:

Case A: Original transmission

All of the following applies:

- The mesh STA is configured as a portal

- dot11MeshPortalAnnouncementProtocol is set to 1

- At every dot11MeshPortalAnnouncementInterval

The content of a PANN element in Case A shall be as shown in Content of a PANN element in Case A.

| - Content of a PANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the PANN element |
| Length | 13 |
| Flags | Not used |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for the Portal Announcement |
| Mesh Portal Address | Mesh STA MAC address |
| PANN Sequence Number | A sequence number specific for the portal |

Case B:	Propagation

All of the following applies:

- The mesh STA has received a Portal Announcement

- The decremented time to live of the Portal Announcement is equal to or greater than 1

- dot11MeshForwarding is set to 1  NOTE TO EDITOR: THERE IS A CHANGE HERE!

The content of a PANN element in Case B shall be as shown in Content of a PANN element in Case B.

| - Content of a PANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the PANN element |
| Length | 13 |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Mesh Portal Address | As received |
| PANN Sequence Number | As received |

- PANN processing

A received Portal Announcement is subject to certain acceptance criteria. Processing depends on the contents of the Portal Announcement and the information available at the receiving mesh STA.

- Acceptance criteria

The Portal Announcement element shall not be accepted (and shall not be processed as described in Effect of receipt) if

- the PANN Sequence Number of the Portal Announcement is lower than the PANN Sequence Number of the recently received Portal Announcement from this portal.

- Effect of receipt

The following applies only to a Portal Announcement element that was accepted according to the acceptance criteria in Acceptance criteria (Acceptance criteria) above.

- The receiving mesh STA shall transmit a Portal Announcement as defined in Conditions for generating and sending a PANN, Case B

- Mmesh STA data forwarding behavior

When a mesh STA has a data message to send, it first follows the data forwarding procedures defined in Error: Reference source not found. If the mesh STA is not able to determine an intra-mesh path to the destination MAC address, the mesh STA shall assume that the destination is outside the mesh BSS and shall forward the message to one or more portals (see Handling of frames that leave the MBSS). If the destination appears to be outside the mesh BSS but there is no portal available, the mesh STA shall silently discard the frame.

- Pportal data forwarding behavior

Forwarding of frames by the portal into the mesh BSS follows the procedures given in Error: Reference source not found.

Portals can learn the addresses of the mesh STAs and of devices attached to these mesh STAs through the receipt of path selection messages and messages carrying proxy information (see Error: Reference source not found).

- Handling of frames that leave the MBSS

A frame sent by a mesh STA in the mesh BSS has the following final destinations:

- A mesh STA address or a proxied address that the portal knows is reachable through the mesh BSS

The portal forwards the frame to the destination mesh STA.

- An address that the portal knows is outside the mesh BSS

The portal forwards the frame on the external network.

- An address unknown to the portal

The portal forwards the frame on the external network.

- Handling of frames that enter the MBSS

A frame received by a portal in an MA_UNITDATA.request at the MAC service has two possible destinations:

- A mesh STA address or proxied address that the portal knows is inside the mesh BSS

The portal forwards the frame to the destination mesh STA.

- An address unknown to the portal

The portal has two options:

- To attempt to establish a path to the destination Error: Reference source not found

- To broadcast the frame within the mesh BSS using the Mesh Broadcast procedure (see Error: Reference source not found)

The criteria for making this choice are beyond the scope of this standard.

- Proxy protocol

Insert 11B.7.5.1 and 11B.7.5.2 as shown

11B.7.5.1 Proxy information

Forwarding information of mesh STAs only contains addresses that belong to the MBSS.  Mesh STAs forward frames to MAC addresses that are outside the MBSS by treating such addresses as proxied addresses.  The mesh STAs that are the recipients of the frames destined to proxied addresses are called proxy mesh STAs, and their MAC addresses are called proxy addresses.

Proxy information consists of at least a proxied MAC address, the corresponding destination proxy MAC address of the proxy mesh STA and the corresponding proxy lifetime.

Proxy information is updated in the following circumstances:

- A mesh STA receives and processes a Proxy Update (see clause 11B.7.5.1 	Proxy Update (PU))

- A mesh STA receives and processes a Mesh Path Selection information element (for example, clause 11B.9.5.3.2 Effect of receipt, items 3 and 4; clause 11B.9.6.3.2 Effect of receipt, items 3 and 4).

11B.7.5.2 Proxy usage

Proxied entities are entities whose MAC addresses cannot be discovered and reached using mesh services, i.e. they are not part of an MBSS.  Examples of such entities are

- STAs that are associated with an AP that is collocated with a mesh STA

- STAs that are behind a portal that is collocated with a mesh STA

Modify 11B.7.5.1 as shown (it should now be 11B.7.5.3)

- Proxy Update (PU)

This clause describes the function, generation and processing of the PU element.

- Function

A PU element is generated by a mesh STA to inform a destination mesh STA of its proxied addresses.

- Conditions for generating and sending a PU

A proxy mesh STA may transmit a PU when it adds or deletes a local proxy to its proxy information. A portal mesh STA may also transmit a PU at periodic intervals.

The PU element is transmitted in an individually addressed manner from the mesh STA to a destination mesh STA. This request may be repeated until the mesh STA receives a PUC element.

The content of a PU element shall be as shown in Content of a PU element.

| - Content of a PU element |  |
| --- | --- |
| Field | Value/description |
| ID | Value given in Error: Reference source not found for the PU element |
| Length | 9 + N * 6 |
| Flags | Bit 0: 0: add proxy information; 1: delete proxy information 1 – 7: Reserved |
| PU Sequence number | Sequence number of the PU |
| Proxy Address | MAC address of the proxy mesh STA |
| Number of Proxied Device Address (N) | Number of proxied addresses reported to the destination mesh STA N (N1) |
| Proxied Device MAC Address | MAC address of proxied entities to which the mesh STA is providing mesh services |

- Effect of receipt of PU processing

The destination mesh STA shall update the proxy address in its proxy information with the list of proxied addresses reported in the PU.  The lifetime of the proxy information is the same as the lifetime of the path to the proxy address.

A PU element in a PU frame is individually addressed from a mesh STA to a destination mesh STA. The destination mesh STA shall update the proxy address in its proxy information with the list of proxied addresses reported in the PU.

- Proxy Update Confirmation (PUC)

This clause describes the function, generation and processing of the PUC element.

- Function

A PUC IE is generated by a destination mesh STA to inform the sender of PU that the PU has been properly received.

- Conditions for generating and sending a PUC

A PUC is individually addressed from the destination mesh STA to the mesh STA that sent the PU.

The content of a PUC element shall be as shown in Content of a PUC element.

| - Content of a PUC element |  |
| --- | --- |
| Field | Value/description |
| ID | Value given in Error: Reference source not found for the PUC element |
| Length | 8 |
| Flags | Bit 0- 7: Reserved |
| PU Sequence number | PU Sequence number of the PU which is being confirmed |
| Destination mesh STA Address | MAC address of the recipient of the PU |

- Effect of receipt of PUC processing

On receiving a PUC element in a PUC frame, the mesh STA shall no longer send any PUs with the same PU sequence number.

Modify 7.3.2.96 as shown

- PANN information element

The Portal Announcement (PANN) element is used for announcing in the MBSS the presence of a mesh STA  collocated with a configured as a Mesh Portal (that has a live connection to an external network). Mesh STAs may use this information to increase the efficiency of communication with stations outside the MBSS.

The PANN element is transmitted in a beacon or a Mesh Path Selection action frame (see clause 7.4.14 Mesh Path Selection action frame details).

The format of the PANN element is shown in PANN  element.

| Element ID | Length | Flags | Hopcount | Time to Live | Mesh Portal Address | PANN Sequence Number |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 |
| - PANN element |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 13.

The Flags field is reserved for future use.

The Hop Count field is coded as an unsigned integer. When the Mesh Portal sends the PANN primitive, it sets the hop count to 0. The Hop Count field indicates the number of hops the PANN primitive has traversed.

The Time to Live field is coded as an unsigned integer and indicates the maximum number of hops allowed for this element.

The Mesh Portal Address is represented as a 48-bit MAC address and is set to the MAC address of the mesh STA that is collocated with the portal.

The PANN Sequence Number field is coded as an unsigned integer and is set to a PANN Sequence Number specific for the originating mesh portal.

Detailed usage of the PANN element is described in Interworking.

Modify Annex D as shown

dot11MeshPortalAnnouncementProtocol OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute shall specify whether or not the mesh portal is using the Portal Annoucement protocol.”

DEFVAL { 0}

::= { dot11MeshSTAConfigEntry X}