IEEE P802.11
Wireless LANs

| Some fixes to mesh power management (text) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |

Abstract

This document provides some fixes to the problem and simplification in mesh power management in D2.06.

## Summary of the intention of this document

- Resolve the problem regarding to the combination use of MDA and deep sleep mode. (Problem #1 in 11-09/121r0)

- Resolve the problem caused by the use of Request to trigger frame. (Problem #2 in 11-09/121r0)

- Provide the clarification text to differentiate DA=broadcast and RA=broadcast. (Question #1 in 11-09/121r0)

- Decouple the beaconing parameter and power modes. Provide some clarification text in Annex V4.3. (Question #2 in 11-09/121r0)

## Suggested changes to the draft spec

- Disallow to operate in deep sleep mode when a mesh STA enables MDA.

- Remove the Request to trigger frame and allow initiating forward link peer service period if it does not want to wait for the trigger frame from the receiver.

- Provide the clarification text to differentiate DA=broadcast and RA=broadcast.

- Decouple the beaconing parameter and power modes. Provide some clarification text in Annex V4.3.

Apply the following changes.
Corresponding changes to D2.06 is indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only. Some “comments” are inserted to the document to notify the deleted part.

- Information elements

Insert the following rows (ignoring the header row and footer note) in Element IDsinput.doc—Element IDs in the correct position to preserve ordering by the “Element ID” column and update the “Reserved” range of codes appropriately.

| - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| Information element | Element ID | Total length of element in octets including the Type and Length octets | Extensible |
| Mesh Configuration Error: Reference source not found |  | 22 |  |
| Mesh ID Error: Reference source not found |  | 2 to 34 |  |
| Link Metric Report Error: Reference source not found |  | 3 to 257 |  |
| Congestion Notification Error: Reference source not found |  | 10 |  |
| Peer Link Management Error: Reference source not found |  | 5 to 9 |  |
| Supported MBSS Regulatory Classes and Channels (see Error: Reference source not found) |  | 5 to 257 |  |
| Mesh Channel Switch Announcement Error: Reference source not found |  | 9 |  |
| Mesh TIM Error: Reference source not found |  | 6 to 256 |  |
| Awake Window Error: Reference source not found |  | 4 |  |
| Beacon Timing Error: Reference source not found |  | 7 to 257 |  |
| Error: Reference source not found |  |  |  |
| MDAOP Setup Request Error: Reference source not found |  | 7 |  |
| MDAOP Setup Reply Error: Reference source not found |  | 4 or 8 |  |
| MDAOP Advertisements Error: Reference source not found |  | 3 to 257 |  |
| MDAOP Reservation Teardown Error: Reference source not found |  | 3 or 9 |  |
| Portal Announcement (PANN) Error: Reference source not found |  | 15 |  |
| Root Announcement (RANN) Error: Reference source not found |  | 19 |  |
| Path Request (PREQ) Error: Reference source not found |  | 39 to 257 |  |
| Path Reply (PREP) Error: Reference source not found |  | 33 or 39 |  |
| Path Error (PERR) Error: Reference source not found |  | 14 to 254 |  |
| Proxy Update (PU) Error: Reference source not found |  | 11 to 251 |  |
| Proxy Update Confirmation (PUC) Error: Reference source not found |  | 10 |  |
| MSCIE Error: Reference source not found |  | 9 |  |
| MSAIE Error: Reference source not found |  | 97 to 257 |  |
| NOTE-The length of an element marked “See NOTE” is specified in this Table, however additional fields may be added in future revisions, with new fields appearing following the existing fields. |  |  |  |

Assignment of values for these information elements needs to be approved by IEEE 802.11 ANA. Until that time, these values are marked as <ANA>. Final values will be requested from IEEE 802.11 ANA once this amendment reaches the 75% approval threshold in Sponsor Ballot.

- Mesh Resource Coordination action frame details

Action frame formats for management of Mesh Resource Coordination are defined in this subclause.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Resource Coordination Action field valuesinput.doc.

| - Mesh Resource Coordination Action field valuesinput.doc |  |
| --- | --- |
| Action field value | Description |
|  | Congestion Control Notification |
|  | MDA Setup Request |
|  | MDA Setup Reply |
|  | MDAOP Advertisement Request |
|  | MDAOP Advertisements |
|  | MDAOP Reservation Teardown |
|  | TBTT Adjustment Request |
|  | Mesh Channel Switch Announcement |
| 9-255 | Reserved |

Error: Reference source not found.

- MDA

Mesh Deterministic Access (MDA) is an optional access method that allows mesh STAs that support MDA to access the WM at selected times with lower contention than would otherwise be possible. This standard does not require all mesh STAs to use MDA. MDA can be used by a subset of mesh STAs in a mesh. However, MDA connections can only be setup among mesh STAs in MDA enabled mode. The performance of MDA may be impacted by devices that do not respect MDA reservations.

MDA uses management frames to determine a series of target transmission starting times and durations, called MDAOPs, between an MDAOP owner and one or more MDAOP responders. Via MDA reports, these MDAOPs are advertised in a neighborhood around the MDAOP owner and responders. MDA supporting mesh STAs in this neighborhood that could cause interference to transmissions during these MDAOPs, or that would experience interference from them, shall refrain from accessing the WM during these MDAOPs. The MDAOP owner and the MDAOP responders access the WM during these MDAOPs using HCF contention-based channel access (EDCA) described in Error: Reference source not found.

In order to use the MDA method for access, a mesh STA shall maintain synchronization with its neighboring mesh STA. When a mesh STA enables MDA, mesh STA shall be in active mode or in light sleep mode and try to listen to Beacon frames from all of its neighboring STAs. The MDA method is described in detail below.

- MDA operation process

The mesh STA that decides to use MDA shall scan on the current operating channel for dot11MDAScanDuration. During the duration, the mesh STA shall not initiate or accept new MDAOPs, but the mesh STA shall set the MDA Enabled bit to 1 in Beacon and Probe Response frames it transmits. After the mesh STA has scanned the channel for dot11MDAScanDuration it may initiate and accept MDAOPs.

A mesh STA in MDA enabled mode may discover other neighboring mesh STAs in MDA enabled mode by receiving a Beacon, or Probe Response frames from the neighboring mesh STAs.

- Power save in a mesh BSS

A mesh STA is capable to buffer frames and track power mode of each peer mesh STA, A mesh STA uses peer service periods for individually addressed frame transmission to the mesh STA in power save mode, and transmits the group addressed frame after the DTIM Beacon when any of its peer mesh STA is in power save mode. These capabilities are referred as support for power save.

A mesh STA, which operates in power save mode or is transitioning to power save mode, is referred to as a power saving mesh STA. The operation in power save mode is optional.

Note: In this subclause, the frame whose RA field is set to individually address is referred to as individually addressed frame. DA field of the individually addressed frame in this context may not be set to an individually address necessarily, if the frame uses 4 address or 6 address format. Similarly, the frames whose RA is set to group address is referred to as group addressed frames, in this subclause.

- Link Specific Power Modes

A mesh STA is in one of two different power states:

- Awake: the mesh STA is able to transmit or receive frames and is fully powered.

- Doze: the mesh STA is not able to transmit or receive and consumes very low power.

The manner in which a mesh STA transitions between power states is determined by the combination of peer link specific power modes. A mesh STA shall be in Awake state if any of its peer links requires operation in Awake state.

A peer link is always associated with two mesh STAs and both mesh STAs have own power mode for the peer link. A mesh STA defines a power mode in which it operates for the peer link and maintains the power mode its peer mesh STA operates for the peer link. The power modes of peer links are independent and mesh STA may operate in different power modes for other peer links. A mesh STA maintains a power mode for non-peer mesh STAs which is described in Non-peer power modes. An example illustration of power modes use is shown in Error: Reference source not found.

There are three power modes defined: active mode, light sleep mode, and deep sleep mode. The used power mode shall be indicated by the Power Management field and Power Save Level field. The difference between modes is as follows

- Active mode: The mesh STA shall be in Awake state all the time.

- Light sleep mode: The mesh STA alternates between Awake and Doze states, as determined by the frame transmission and reception rules. The mesh STA shall listen to all the Beacons from its peer mesh STA

- Deep sleep mode: The mesh STA alternates between Awake and Doze states, as determined by the frame transmission and reception rules. The mesh STA may choose not to listen to the Beacons from its peer mesh STA

A power mode is defined as the combination of the Power Management field indicated in the Frame Control field and the Power Save Level as shown in the Power Mode Definition.

| - Power Mode Definition |  |  |  |
| --- | --- | --- | --- |
| Activity Level | Power Mode | Power Management Field | Power Save Level Field |
| Highest Lowest | Active Mode | 0 | Reserved |
|  | Light Sleep Mode | 1 | 0 |
|  | Deep Sleep Mode | 1 | 1 |

- Non-peer power modes

A mesh STA shall indicate its non-peer power mode with the Power Management field in the Frame Control field and Power Save Level field in the Mesh Capability field in Beacon and Probe Response frames. A mesh STA shall also indicate its non-peer power mode with the Power Management field in the Frame Control field and Power Save Level field in the Mesh Control field in group addressed mesh data frames. The non-peer power mode determines when non-peer mesh STAs may send Probe Request and Peer Link Open Request frames to the mesh STA. The non-peer power mode shall be equal to or lower than the lowest activity level in which the mesh STA is operating for peer mesh STAs.

- Power mode indications and transitions

When a mesh STA is operating in active mode for a link, the mesh STA shall set Power Management bit to 0 in all transmitted individually addressed Mesh Data or Mesh-Null frames on this link.

When a mesh STA is operating in light sleep mode for a link, the mesh STA shall set the Power Management bit to 1 and the Power Save Level field to 0 in all transmitted individually addressed Mesh Data or Mesh-Null frames on this link.

When a mesh STA is operating in deep sleep mode for a link, the mesh STA shall set the Power Management bit to 1 and the Power Save Level field to 1 in all transmitted individually addressed Mesh Data or Mesh-Null frames on this link.

The algorithm to trigger the change of power mode is beyond the scope of this standard.

- Transition to a higher activity level

A mesh STA may use group addressed or individually addressed frames to change its power mode to a higher activity level, i.e. from deep sleep to light sleep or to active mode; or from light sleep to active mode.

Individually addressed frames may be used temporarily raise the activity level of the mesh STA for a peer link. This may be useful in cases, when a link temporarily requires efficient data transmission with the peer mesh STA and the mesh STA desires to be able to transit back to lower activity level without performing the power management transition signaling with all peer mesh STAs.

- Transition to a lower activity level

A mesh STA shall use acknowledged individually addressed Mesh Data or Mesh-Null frames to change its power mode for a link to a lower activity level, i.e. from active mode to light or deep sleep mode; or from light sleep to deep sleep mode.

If a mesh STA is operating in deep sleep mode for a link or the mesh STA intends to change the power mode of at least one link to deep sleep mode, the mesh STA shall set the Power Management bit to 1 and Power Save Level field to 1 to all transmitted group addressed frames.

If a mesh STA is not operating in deep sleep mode for any link and the mesh STA is operating in light sleep mode for a link or the mesh STA intends to change its operation mode for at least one link to light sleep mode, the mesh STA shall set the Power Management bit to 1 and Power Save Level field to 0 in all transmitted group addressed frames.

- Mesh TIM transmissions

The Mesh TIM element shall identify the peer mesh STAs operating in power save mode for which traffic is pending and buffered in the reporting mesh STA. This information is coded in a partial virtual bitmap using AIDs. In addition, the Mesh TIM contains an indication whether group addressed traffic is pending. The mesh STA shall identify those peer mesh STAs for which it is prepared to deliver buffered MSDUs by setting bits in the partial virtual bitmap of the Mesh TIM that correspond to the appropriate AIDs.

- Mesh TIM types

There are two different Mesh TIM types: Mesh TIM and Mesh DTIM. A mesh STA shall transmit a Mesh TIM with every Beacon frame. Every DTIMPeriod, a Mesh TIM of type Mesh DTIM is transmitted with a Beacon frame. After a Mesh DTIM the mesh STA shall send the buffered group addrssed MSDUs, before transmitting any individually addressed frames. The More Data field of each group addrssed frame shall be set to indicate the presence of further buffered group addrssed  MSDUs. The mesh STA sets the More Data field to 0 in the last transmitted group addrssed frame following the transmission of the DTIM beacon.

If CCA is IDLE for the duration of PHY specific Group Delivery Idle Time when a mesh STA expects to receive a Mesh Beacon or a group addrssed frame, the receiving mesh STA may assume that no more frames destined to group addresses will be transmitted and return to Doze state. If PHY mode operates as defined in Clauses 15 and 18 or as defined in Clauses 17 and 19, the Group Delivery Idle Time is three TUs, otherwise the Group Delivery Idle Time is set to 0.

- Awake Window

A power saving mesh STA shall be in Awake state during its own Awake Window. An Awake Window is present after the Beacon and Probe Response frames containing an Awake Window element. An awake Window element shall be in the DTIM Beacon frame and may be in TIM Beacon and Probe Response frames. The start of the Awake Window is measured from the end of the Beacon or Probe Response transmission. The duration of the Awake Window period is defined by the value of the Awake Window field in the Awake Window element.

If the owner of the Awake Window transmits frames destinated to group addresses, the Awake Window is extended until the owner of Awake Window has transmitted all of its frames to group address and PHY specific PostAwakeTime from the transmission of the last group addressed frame has elapsed. The PostAwakeTime is PHY mode specific. If PHY mode operates as defined in Clauses 15 and 18 or as defined in Clauses 17 and 19, the PostAwakeTime is 5TUs, otherwise the PostAwakeTime is set to 0.

The peer mesh STAs may send trigger frames to the power saving mesh STA during the Awake Window of the power saving mesh STA in order to initiate a peer service period. A successfully transmitted trigger frame initiates a peer service period as described in Peer service period.

Non-peer mesh STAs may send Probe Request and Peer Link Open frames to the power saving mesh STA during its Awake Window. The power saving mesh STA shall operate in Awake state at least until the completion of the peer link setup procedure (i.e. success or failure) or the transmission of the Probe Response frame upon the reception of the Probe Request frame.

- Power save Support

As described in Link Specific Power Modes a mesh STA indicates its power mode for each peer link and obtains the power modes of its peer mesh STAs. A mesh STA shall not arbitrarily transmit frames to mesh STAs operating in a power save mode, but shall buffer frames and only transmit them at designated times.

As described in Mesh TIM transmissions a mesh STA indicates the presence of buffered traffic in TIM elements for all peer mesh STAs in power save mode. As described in Mesh TIM types, a mesh STA transmits its group addressst frames after its DTIM Beacon if any of its peer mesh STA is in power save mode..

As described in Peer service period peer service periods are used for frame transmissions towards a mesh STA which operates in power save mode. Peer Service Periods are not used in frame exchange towards active mode mesh STAs.

A mesh STA may initiate a peer service period with a peer mesh STA which operates in deep or light sleep mode by transmitting a trigger frame during the Awake Window of the peer mesh STA.

- Operation in link specific power mode

- Operation in the active mode

A mesh STA operating in active mode shall be in Awake state. Peer service periods are not used in frame exchange toward active mode mesh STAs.

- Operation in light sleep mode

A mesh STA operating in light sleep mode shall enter the Awake state prior to every TBTT that matches the Beacon timing of its own and peer mesh STA. The mesh STA shall remain in Awake state after its beacon transmission for the duration of the Awake Window and the duration of its group addressed frame transmissions, whichever occurs later. The mesh STA may return to the Doze state after the Beacon reception from a peer mesh STA, to which it indicated to operate in light sleep mode, if the peer mesh STA did not indicate buffered individually addressed or group addressed frames. If an indication of buffered frames is received, the light sleep mode mesh STA shall send a trigger frame to initiate a peer service period with the beaconing mesh STA.

If a light sleep mode mesh STA enters the Awake state in order to listen to DTIM Beacon and group addressed transmissions from its peer mesh STA, the mesh STA shall remain Awake until the More Data field of the group addressed MSDUs indicated the last group addressed MSDU.

A mesh STA in power save mode may receive peer trigger frames during its Awake Window as described in Awake Window.

If a mesh STA indicates buffered traffic using a Mesh TIM in Beacon frame for a peer mesh STA in light sleep for the link, it shall remain in Awake state until it completes the buffered traffic transmission to the peer mesh STA in light sleep mode or until its  next TBTT.

- Operation in deep sleep mode

A mesh STA operating in deep sleep mode shall enter the Awake state prior to every TBTT that matches its own DTIM Beacon timing and shall remain in Awake state after the Beacon transmission for the duration of the Awake Window and the duration of its group addressed frame transmissions, whichever occurs later. A mesh STA operating in deep sleep mode may not receive Beacon frames from its peer mesh STAs as far as it maintains synchronization with peer mesh STAs.

A mesh STA in power save mode may receive peer trigger frames during its Awake Window as described in Awake Window.

If a mesh STA indicates buffered traffic using a Mesh TIM in Beacon frame for a peer mesh STA in light sleep for the link, it shall remain in Awake state until it completes the buffered traffic transmission to the peer mesh STA in light sleep mode or until its next TBTT.

- Conditions for Doze State

A mesh STA may enter Doze state if all following conditions are fulfilled:

- the mesh STA operates in power save mode for all of the links

- the mesh STA has no peer service period ongoing

- the Awake Window indicated by the mesh STA has expired

- the mesh STA has terminated its group addressed frames delivery sequence after its DTIM Beacon

- the mesh STA has completed the buffered frames delivery to the peer mesh STA in light sleep mode, to which the mesh STA had indicated buffered traffic in Beacon frame

Guidance for the power save mechanisms use and default parameter values are given in Annex Power Save parameters selection .

- Peer service period

Peer service periods are used for frame exchange in a link in which at least one of the mesh STAs operates in power save mode. A peer service period is a contiguous period of time during which one or more individually addressed frames are transmitted between two peer mesh STAs. A peer service period is directional and may contain one or more TXOPs. One mesh STA transmits frames and initiates the termination of the peer service period. A mesh STA may have multiple peer service periods ongoing in parallel. At most one peer service period can be set up in each direction with each peer mesh STA. A peer service period is initiated by a peer trigger frame.

In addition, when a mesh STA enables MDA while in a power save mode, a scheduled service period begins at the each MDAOP start time as described in 11B.12.11. A mesh STA in a power save mode shall enter the Awake state prior to the start time of scheduled service period.

A data frame or a QoS-Null frame that requires acknowledgement may be used as a peer trigger frame. If the EOSP bit in the peer trigger frame is set to 0, the peer trigger frame initiates two independent peer service periods; one is owned by the transmitter of the peer trigger frame and the other is owned by the receiver of the peer trigger frame,. If the EOSP bit in the peer trigger frame is set to 1, one peer service period is initiated which is owned by the receiver of peer trigger frame. Peer service period shall not be used in frame exchange toward active mode mesh STA.

During the peer service period, the transmitter of the peer service period and peer mesh STA shall operate in Awake state. The peer service period may contain one or more TXOPs. During the peer service period, the transmitter is able to transmit frames from any AC, but a TXOP shall contain frames from a single AC only.

The peer service period is terminated after a successfully acknowledged QoS-Null or data frame with the EOSP bit set to 1 from the transmitter of the peer service period.

If the mesh STA does not receive an acknowledgement to a frame sent with the EOSP subfield set to 1, the mesh STA shall retransmit that frame at least once within the same peer service period - subject to applicable retry or lifetime limit. The maximum number of retransmissions within the same peer service period is the lesser of the Max Retry Limit and the MIB attribute dot11MeshSTAMissingAckRetryLimit. If an Ack to the retransmission of this last frame in the same peer service period is not received, the mesh STA may use the next peer service period to further retransmit that frame subject to the applicable retry or lifetime limit.

- MDA use by power saving mesh STA

When a mesh STA enables MDA, mesh STA shall be in active mode or in light sleep mode. When the mesh STA that is in a deep sleep mode with any of its neighbor peer mesh STA is enabling MDA access, it shall change power mode of all its links to light sleep mode or active mode, prior to use the MDA access.

A scheduled service period begins at the MDAOP start time, if the receiver mesh STA in MDAOP operates in power save mode for the transmitter mesh STA in MDAOP. The MDAOP transmitter is the transmitter during the scheduled service period. The MDAOP receiver is the receiver during the scheduled service period. Scheduled service periods are not used, if the receiver mesh STA in MDAOP operates in active mode for the transmitter mesh STA.

The scheduled service period continues until it is successfully terminated by acknowledged QoS-null or data frame with set EOSP bit from transmitter to receiver as described in Peer service period..

- Power Save parameters selection

Power save mechanisms enable mesh STA operation in Doze state. In Doze state a mesh STA may set the transceivers off. The power save mechanism may adjust the power mode of a mesh STA for various reasons which are beyond the scope of the standard including but not limited to traffic load or scanning and network maintenance needs.

The Annex specifies recommendations how to select a power mode for the mesh STA based on traffic load, recommendations how to do scanning in mesh BSSs which may contain mesh STAs in power save mode and recommended default values for power save related parameters.

- Selecting the power mode based on traffic load

A mesh STA may adjust its power mode based on the traffic load or the QoS requirements of the forwarded traffic. If a mesh STA has high traffic load or a mesh STA is congested, the mesh STA should operate in active mode to reduce delays or overhead that may be created by the power save mechanisms.

A mesh STA may consider operating in active mode for all peer mesh STAs, even if only a single link is congested. When mesh STA operates in active mode, the mesh STA may receive frames at any time and peer service periods may be triggered on demand, as in case of an AP that receives a trigger frame at times controlled by a non-AP STA.

If traffic load is moderate or low and best effort data is transmitted, a mesh STA may consider operating in light sleep mode for all or some peer links. The mesh STA operating in light sleep mode for a peer link receives beacons from the peer mesh STA with periodic indication of buffered traffic.

If traffic load is low or no traffic is transmitted, a mesh STA may consider operating in deep sleep mode for a peer link. The mesh STA does not need to wake-up to receive a beacon from the peer mesh STA relative to which it operates in deep sleep mode.

The mesh STA may use deep sleep mode to control the number of times it enters the Awake state to receive Beacon frame from a peer mesh STA. If the mesh STA operates in deep sleep mode for all of its peer link, the mesh STA needs only to operate in Awake state during its own beacon transmission and Awake Window.

The power saving mesh STA should avoid real time traffic (AC3 or AC2) transmissions with EDCA when both mesh STAs in a peer link are in power save mode. In this case the delays in frames forwarding will increase and the forwarded data frames have inappropriate QoS.

- Scanning of mesh BSSs

A mesh BSS may have mesh STAs in power save and active mode. With active scanning only devices in Awake state at the transmission time of Probe Request frame may be found. However, with passive scanning also mesh STAs in power save mode can be found.

Since mesh STAs in power save mode may transmit Beacons at long intervals, a mesh STA seeking for candidate mesh STAs for a new peer links should do passive scanning relatively longer time compared to passive scanning in BSS infrastructure mode. mesh STAs in power save mode with long Mesh DTIM interval may not be discovered with short scanning durations. mesh STAs in power save mode may use short Mesh DTIM interval, if it intends to establish new peer links.

- Default Parameters

The following are the recommended default values for power save related parameters for mesh STAs:

- Beacon Period: 200TU

- Mesh DTIM period: 5

- Awake Window: 10TU

|  | For moderate power save | For aggressive power save |
| --- | --- | --- |
| Beacon Period | 200 TU | 800 TU |
| Mesh DTIM period | 4 | 1 |
| Awake Window | 10 TU | 10 TU |

A mesh STA that is eager to conserve power and likely to operate in deep sleep mode with most of the links should utilize the value from aggressive power save parameters.

Although mesh STAs may utilize individual parameters regardless of the parameters used by neighbor mesh STAs or peer mesh STAs, each implementer should recognize balance between the power save efficiency and delay in the service initiation.

- Frames forwarding in MBSS containing mesh STAs in power save mode

The battery powered mesh STAs should avoid forwarding frames, to avoid power consumption and possible additional delays and inefficiencies that power save mechanisms may cause. If a power saving mesh STA forwards frames, it should select its power mode based on the traffic load and the traffic type as described in the Clause Selecting the power mode based on traffic load.

The mesh STAs that desire to operate in power save mode may select the power save mode:

- mesh STAs that are “mains powered” may apply power save if they do not have frames to forward

- mesh STAs, that are “battery powered” and desire to minimize the power consumption, may freely use all power save modes

Mesh STAs may degrade the link metric value. The use of worse metric values make reduces the probability of a power saving mesh STA being used for forwarding frames. If the mesh STA will operate in active mode for the link in forwarding path it should apply the link metric value without degradation.

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D2.06, January 2009.

[2] “some fixes to mesh power management”,  11-09/0121r0, January 2009.