IEEE P802.11
Wireless LANs

| LB140 Multicast Diagnostics Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |

Abstract

This document contains proposed normative text changes to draft 4 of P802.11v to resolve comments received in letter ballot 140 for comments CID 115, 466, 504, 532 and 533.

Modify sub-clause 7.3.2.21.12 as indicated

7.3.2.21.12 Multicast Diagnostics Request

The Measurement Request field corresponding to a Multicast Diagnostics Request is shown in Figure 7-62q1. The response to a Multicast Diagnostics Request is a Multicast Diagnostics Report.

|  | Randomization Interval | Measurement Duration | Group MAC Address | Multicast Triggered Reporting (optional) | Optional subelements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 0 or 35 | Variable |
| Figure 7-62q1—Measurement Request field format for a Multicast Diagnostics Request |  |  |  |  |  |

EDITORIAL NOTE—Figure 7-62r is defined in IEEE 802.11k-2008.

The Randomization Interval field is set to the desired upper limit of random delay before the measurement begins, expressed in TUs. The use of the Randomization Interval is described in 11.11.3. When requesting a triggered multicast diagnostic report, the Randomization Interval field is reserved.

The Measurement Duration field is set to the duration of the requested measurement, expressed in TUs. When requesting a triggered multicast diagnostic report, the Measurement Duration field is reserved.

The Group MAC Address field contains the MAC address of the group addressed frames to which the measurement request relates when the LSB of the first octet set to one. A Group MAC Address field with the LSB of the first octet set to zero indicates that all group addressed frames, apart from the broadcast MAC address, are requested.

The Multicast Triggered Reporting field is used to specify trigger conditions and thresholds. It is only present when requesting triggered multicast diagnostic reporting. The Multicast Triggered Reporting field is as shown in Figure 7-62r2.

The Multicast Trigger Condition field specifies reporting triggers for triggered management diagnostic reporting. The format of the Multicast Trigger Condition field is shown in Figure 7-62r3.

|  | Subelement ID | Length | Multicast Trigger Condition | Report Timeout | Trigger Timeout |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |  |  |
|  |  |  |  | Figure 7-62r2—Multicast Triggered Reporting field subelement |  |  |  |

|  | B0 | B1 | B7 |
| --- | --- | --- | --- |
|  | Report Timeout Request | Reserved |  |
| Bits: | 1 | 7 |  |
| Figure 7-62r3—Multicast Trigger Condition field |  |  |  |

The Report Timeout Request field is set to 1 to request that a Multicast Triggered Report be generated when no multicast frames with the monitored multicast or broadcast address are received in a time equal to the value given in the Report Timeout field. The Report Timeout Request field is set to 0 when a multicast reception timeout is not requested.

The Report Timeout field contains a time value in units of 100 TU to be used as the threshold value for the Report Timeout trigger condition. The Report Timeout field is not used when the Report Timeout Request field is set to 0.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA does not generate further Multicast Triggered Reports after a trigger condition has been met.

The Optional Subelement field format contains zero or more Subelements, each consisting of a one octet Subelement ID field, a one octet Length field and a variable length Data field, as shown in Figure 7-95p. The optional subelements are ordered by non-decreasing Subelement ID.

The Subelement ID field values for the defined optional subelements are shown in Table 7-<TBD>. A Yes in the Extensible column of a subelement listed in Table 7-57b indicates that the Length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is set to Subelements, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.14.2.

| Table 7-<TBD>—Optional Subelement IDs for Multicast Diagnostics Request |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Multicast Triggered Reporting | 5 |  |
| 02-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 239 |  |
| 222-255 | Reserved |  |  |

EDITORIAL NOTE—The Figure 7-95p is defined in the IEEE 802.11k-2008.

The use of Multicast Diagnostics Request is defined in 11.20.2.

References: