IEEE P802.11
Wireless LANs

| LB140 Multicast Diagnostics Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |

Abstract

This document contains proposed normative text changes to draft 4 of P802.11v to resolve comments received in letter ballot 140 for comments CID 115, 466, 467, 468, 504, 532, 533, 534, 536 and 537.

Modify sub-clause 7.3.2.21.12 as indicated

7.3.2.21.12 Multicast Diagnostics Request

The Measurement Request field corresponding to a Multicast Diagnostics Request is shown in Figure 7-62q1. The response to a Multicast Diagnostics Request is a Multicast Diagnostics Report.

|  | Randomization Interval | Measurement Duration | Group MAC Address | Multicast Triggered Reporting (optional) | Optional subelements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 0 or 35 | Variable |
| Figure 7-62q1—Measurement Request field format for a Multicast Diagnostics Request |  |  |  |  |  |

EDITORIAL NOTE—Figure 7-62r is defined in IEEE 802.11k-2008.

The Randomization Interval field is set to the desired upper limit of random delay before the measurement begins, expressed in TUs. The use of the Randomization Interval is described in 11.11.3. When requesting a triggered multicast diagnostic report, the Randomization Interval field is reserved.

The Measurement Duration field is set to the duration of the requested measurement, expressed in TUs. When requesting a triggered multicast diagnostic report, the Measurement Duration field is reserved.

A Group MAC Address field with the LSB of the first octet set to one contains the MAC address of the group addressed frames to which the measurement request relates. A Group MAC Address field with the LSB of the first octet set to zero indicates that all group addressed frames, apart from the broadcast MAC address, are requested.

The Multicast Triggered Reporting field is used to specify trigger conditions and thresholds. It is only present when requesting triggered multicast diagnostic reporting. The Multicast Triggered Reporting field is as shown in Figure 7-62r2.

The Multicast Trigger Condition field specifies reporting triggers for triggered management diagnostic reporting. The format of the Multicast Trigger Condition field is shown in Figure 7-62r3.

|  | Subelement ID | Length | Multicast Trigger Condition | Report Timeout | Trigger Timeout |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |  |  |
|  |  |  |  | Figure 7-62r2—Multicast Triggered Reporting field subelement |  |  |  |

|  | B0 | B1 | B7 |
| --- | --- | --- | --- |
|  | Report Timeout Request | Reserved |  |
| Bits: | 1 | 7 |  |
| Figure 7-62r3—Multicast Trigger Condition field |  |  |  |

The Report Timeout Request field is set to 1 to request that a Multicast Triggered Report be generated when no multicast frames with the monitored multicast or broadcast address are received in a time equal to the value given in the Report Timeout field. The Report Timeout Request field is set to 0 when a multicast reception timeout is not requested.

The Report Timeout field contains a time value in units of 100 TU to be used as the threshold value for the Report Timeout trigger condition. The Report Timeout field is not used when the Report Timeout Request field is set to 0.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA does not generate further Multicast Triggered Reports after a trigger condition has been met.

The Optional Subelement field format contains zero or more Subelements, each consisting of a one octet Subelement ID field, a one octet Length field and a variable length Data field, as shown in Figure 7-95p. The optional subelements are ordered by non-decreasing Subelement ID.

The Subelement ID field values for the defined optional subelements are shown in Table 7-<TBD-1>. A Yes in the Extensible column of a subelement listed in Table 7-<TBD-1> indicates that the Length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is set to Subelements, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.14.2.

| Table 7-<TBD-1>—Optional Subelement IDs for Multicast Diagnostics Request |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | Multicast Triggered Reporting | 5 |  |
| 2-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 239 |  |
| 222-255 | Reserved |  |  |

EDITORIAL NOTE—The Figure 7-95p is defined in the IEEE 802.11k-2008.

The use of Multicast Diagnostics Request is defined in 11.20.2.

Modify the following clause as shown

7.3.2.22.11Multicast Diagnostics Report

The format of the Measurement Report field of a Multicast Diagnostics report is shown in Figure 7-68q1.

|  | Measurement Time | Measurement Duration | Group MAC Address | Multicast Reporting Reason |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 2 | 6 | 1 |  |
|  | Multicast Received MSDU Count | First Sequence Number | Last Sequence Number | Multicast Rate | Optional subelements |
| Octets: | 4 | 2 | 2 | 2 | Variable |
| Figure 7-68q1—Measurement Report field format for a Multicast Diagnostics Report |  |  |  |  |  |

The Measurement Time field is set to the value of the STA TSF timer at the time the measurement started. For a triggered Multicast Diagnostics report, this is the TSF value at the reporting STA when the trigger condition was met. When the reason for sending the report is Performance Measurement and the Multicast Received MSDU Count is nonzero, the Measurement Time field is set to the value of the STA TSF timer at the time of the first multicast MSDU received during the measurement interval.

When the Measurement Result Performance Measurement field in the Multicast Reporting Reason field is set to one, the Measurement Duration field specifies the period over which the Multicast Diagnostic Report was generated. When the Performance Measurement field in the Multicast Reporting Reason field is set to zero, the Measurement Duration field is not used.

The Group MAC Address field contains the value from the Group MAC Address field from the Multicast Diagnostics Request to which the report relates.

The Multicast Reporting Reason field indicates the reason why the measuring STA sent the Multicast Diagnostics report. The Multicast Reporting Reason field is shown in Figure 7-68q2.

|  | B0 | B1 | B2-B7 |
| --- | --- | --- | --- |
|  | Report Timeout Trigger | Measurement Result | Reserved |
| Bits: | 1 | 1 | 6 |
| Figure 7-68q2—Multicast Reporting Reason field |  |  |  |

EDITORIAL NOTE—Figure 7-68q is defined in IEEE 802.11k-2008.

The subfields of the Multicast Reporting Reason field are defined as follows:

—	The Report Timeout Trigger field set to 1 indicates that Multicast Diagnostics Report was generated as a triggered report due to the timeout of the multicast diagnostic timer.

—	The Measurement Result field set to 1 indicates that the Multicast Diagnostic Report contains the result of a multicast diagnostic request. The report may be generated unsolicited by a non-AP STA or in response to a request for performance measurement of a multicast stream.

All the bits in the Multicast Reporting Reason field can be independently set.

The Multicast Received MSDU Count field contains the total number of multicast MSDUs with the indicated Multicast MAC Address that were received during the Measurement Duration. For a triggered multicast diagnostics measurement this is the total number of frames received with the indicated Multicast MAC Address. the Multicast Received MSDU Count field contains the total number of multicast MSDUs with the indicated Multicast MAC Address that were received between the acceptance of the multicast diagnostics measurement request and the occurance of the timeout trigger condition.

The twelve least significant bits of the First Sequence Number field contain the sequence number of the first frame received with destination address equal to the value in the Multicast MAC address field during the measurement period. When the LSB of the first octet of the Multicast MAC address field in the multicast diagnostic request is set to 1, the twelve LSBs of the First Sequence Number field contains the sequence number of the first frame received with destination address equal to the value in the Multicast MAC address field during the measurement period. When the LSB of the first octet of the Multicast MAC address field in the multicast diagnostic request is set to 0, the twelve LSBs of the First Sequence Number field contains the sequence number of the first group addressed frame, that does not have the broadcast MAC address as its destination, received during the measurement period. The four most significant bits of the First Sequence Number field are set to zero. This field is used only if the Measurement Result field is set to 1; otherwise, it is set to 0.

The twelve least significant bits of the Last Sequence Number field is the sequence number of the last frame received with destination address equal to the value in the Multicast MAC address field during the measurement period. When the LSB of the first octet of the Multicast MAC address field in the multicast diagnostic request is set to 1, the twelve LSBs of the Last Sequence Number field contains the sequence number of the last frame received with destination address equal to the value in the Multicast MAC address field during the measurement period. When the LSB of the first octet of the Multicast MAC address field in the multicast diagnostic request is set to 0, the twelve LSBs of the Last Sequence Number field contains the sequence number of the last group addressed frame, that does not have the broadcast MAC address as its destination, received during the measurement period. The four most significant bits of the Last Sequence Number field are set to zero. This field is used only if the Measurement Result field is set to 1; otherwise, it is set to 0.

The First Sequence Number field and the Last Sequence Number field are set to 0 if the Multicast Received MSDU Count is 0.

The Multicast Rate field specifies the highest data rate, in 0.5 Mb/s units, at which the STA has received a group addressed frame with a valid FCS during the measurement period.The Multicast Rate field is encoded with the MSB set to 1 to indicate that the data rate is in the basic rate set, and set to 0 to indicate that the data rate is not in the basic rate set. The remaining 15 bit value is multiplied by 0.5 Mb/s to indicate the data rate. The Multicast Rate field is set to 0 by the STA to indicate that it has not received a group addressed frame with a valid FCS during the measurement period.

The Optional Subelement field format contains zero or more Subelements, each consisting of a one octet Subelement ID field, a one octet Length field and a variable length Data field, as shown in Figure 7-95p. The optional subelements are ordered by non-decreasing Subelement ID.

The Subelement ID field values for the defined optional subelements are shown in Table 7-<TBD-2>. A Yes in the Extensible column of a subelement listed in Table 7-<TBD-2> indicates that the Length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is set to Subelements, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.14.2.

| Table 7-<TBD-2>—Optional Subelement IDs for Multicast Diagnostics Report |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 239 |  |
| 222-255 | Reserved |  |  |

EDITORIAL NOTE—The Figure 7-95p is defined in the IEEE 802.11k-2008.

The use of Multicast Diagnostics Report is defined in 11.20.2.

References: