IEEE P802.11
Wireless LANs

| TGp LB 141 Comment Resolution Clauses 10.3a, b, c and 11.a.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Susan R Dickey | California PATH (affiliation Caltrans) | 1357 S 46th St, UC Berkeley, Richmond, California | 510-665-3664 | dickey@path.berkeley.edu |

Abstract

This submission proposes resolutions to comments from Letter Ballot 141 on clauses 10.3a, b, c and 11.a.1 in TGP Draft D5.0.

|  | LB141 Comment Resolution |
| --- | --- |

- COMMENTS:

| CID | Commenter | Clause | Pg | Ln | Comment | Suggested Remedy | Comment resolution. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 125 | Emeott, Stephen | 10.3.a | 11 | 43 | The Timestamp field is not used or required by any feature in this amendment. Therefore, there is no need to get, set or advance the timer. Delete these three MLME primitives. | Make the suggested change | Accept. |
| 127 | Kenney, John | 10.3.a.2.4 | 12 | 50 | There is a sentence explaining that if the result is successful the value of the TSF timer is included in the primitive. There is no explanation of whether the parameter is included if the result is failure. | Append "If the result of an MLME-SETTSFTIME.request is failure the TSFtime parameter is not included in the MLME-SETTSFTIME.confirm primitive." Also, in 10.3.a.2.2, in the Description column of the table listing the primitive parameters, add ", if ResultCode is SUCCESS" after "Value of the TSF timer" in the Description column. | Counter. By accepting comment 125, this clause will be deleted. |
| 128 | Emeott, Stephen | 10.3.b | 13 | 1 | The Timestamp field is not used or required by any feature in this amendment. Therefore, there is no need to get, set or advance the timer. Delete these three MLME primitives. | Make the suggested change | Accept. |
| 130 | Emeott, Stephen | 10.3.c | 14 | 39 | The Timestamp field is not used or required by any feature in this amendment. Therefore, there is no need to get, set or advance the timer. Delete these three MLME primitives. | Make the suggested change | Accept. |
| 159 | Chu, Liwen | 11.a.1 | 21 | 6 | .11 baseline standard defines several rules for accepting Beacon, Probe, other management frames, and data frames. .11p draft defines how to accept data frames. Inside the context of a BSS, the BSSID is set by the SME, but outside the context of a BSS, it is not specified how to verify the BSSID field in order to accept the "Timing and information" frame and other management frames. How does the STA know whether the "Timing and information frame" are associated with an overlapping BSS or whether it was sent outside the contest of a BSS and is OK to accept? | Define the rules about accepting management frames outside the context of BSS. | Accept in principle. With the changes to clause 7 in 11-09/58, Address 3 will be set to the wildcard BSSID when sent outside the context of a BSS, resolving the ambiguity. |
| 160 | Roy, Richard | 11.a.1 | 21 | 6 | This subclause should describe how the timing information is used in addition to how the timestamp is generated. | "Remove subclasue 11.a.1 and replace it with the following subclause: "" 11.6.2a Use of the Timing and Information management frame for distributing time from an external clock The main purpose of the Timing and Information management frame is to provide a mechanism whereby a STA can send to other STAs sufficient information in a single management frame to allow the receiving STAs to estimate the time being kept by an external clock (other than the TSF timer) on the transmitting STA. This is accomplished using the timestamp parameter in the transmitted frame in conjunction with the Timing Information Element (TIE) which contains the information necessary to adjust the TSF timer of the transmitting STA to match the output of the transmitting STAs external clock. A STA that is transmitting a Timing and Information frame shall set the value of the timestamp parameter to the value of the STA's TSF timer at the time that the data symbol containing the first bit of the timestamp is transmitted to the PHY plus the transmitting STAs delays through its local PHY from the MAC-PHY interface to its interface with the WM. A Timing and Information frame is generated by a STA's MLME in response to receiving an MLME-TIMING_INFO.request from the STA's SME. After generating the frame, the MLME returns an MLME- TIMING_INFO.confirm to the SME.""" | Counter. By accepting CID 125 we remove the TSF primitives from the MLME and expect that higher layers will be able to synchronize clocks using MLME-HL-SYNC if requird. |
| 161 | Kenney, John | 11.a.1 | 21 | 8 | The Timing and Information frame always includes a timestamp, so the first sentence of 11.a.1 applies whether or not the STA sending this frame is communicating outside the context of a BSS. | Delete "communicating outside the context of a BSS" from the first sentence of 11.a.1 | Counter. 11.a.1 changes as a result of accepting CID 125 to have no reference to timestamp. |

- Background

Commenters have said that no need has been demonstrated for timestamp handling or TSF MLME primitives by TGp. This is a “devil’s advocate” proposal that we accept their comments. Are existing mechanisms in 802.11-2007/11.6.2 adequate for higher level synchronization? If this proposal is accepted, should the name of the Timing Advertisement Frame be changed, e.g., to Parameter Advertisement Frame?

- Recommended Resolution of the Comments:

The following changes to TGp Draft 5.0 should be made to resolve the comments as listed in Section 1 above.

Delete clauses 10.3.a, 10.3.b and 10.3.c, renaming 10.3.d to 10.3.a.

Change clause 11.a.1, lines 6-13, page 21, from

:

11.a.1 Generation and timestamping of a Timing and information frame

A STA communicating outside the context of a BSS that is sending a Timing and information frame sets the value of the timestamp to the value of the STA's TSF timer at the time that the data symbol containing the first bit of the timestamp is transmitted to the PHY plus the transmitting STA’s delays through its local PHY from the MAC-PHY interface to its interface with the WM (see 11.a.2). A Timing and information frame is generated by a STA's MLME in response to receiving an MLME-TIMING_INFO.request from the STA's SME. After generating the frame, the MLME generates an MLME-TIMING_INFO.confirm to the SME.

to:

11.a.1 Generation of a Timing Advertisement Frame

When dot11OCBEnabled is true, a STA may send a Timing Advertisement Frame to let other STAs know parameters that may be used to communicate with it on the current channel. A Timing Advertisement Frame is generated by a STA's MLME in response to receiving an MLME-TIMING_ADVERTISEMENT.request from the STA's SME. After generating the frame, the MLME generates an MLME-TIMING_ADVERTISEMENT.confirm to the SME.

- Motion (if technical and/or significant):

Move to accept the Recommended Resolutions to these comments, based on the changes made to the D5.0 as a specified in Section 3 above:

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |