IEEE P802.11
Wireless LANs

| LB-137: Proposal to Resolve Interworking Nomenclature |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-1-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

The term “interworking capable” is used in several places in 802.11u-d4.0 and is deemed to be less than optimal specification language.  The commenters prefer tying behavior to a specific MIB variable to make the amendment more readable.

This proposal addresses the following CIDs: 4324, 4343 and 4338.

This document is based on IEEE 802.11-2007 and 802.11u-d4.02.

5	General description

5.2	Components of the IEEE 802.11 architecture

Change the heading text as shown below:

5.2.10	SSPN Interface

Change the text as shown below:

An AP can interact with external networks using a logical SSPN interface for the purpose of authenticating users and provisioning services, as shown in Figure 5-6a (IEEE 802.11 Interworking infrastructure architecture).

5.4	Overview of the services

5.4.8	Interworking with external networks

Change the text as shown below:

The Interworking service allows non-AP STAs to access services provided by an external network according to the subscription or other characteristics of that external network. An IEEE 802.11 non-AP STA may have a subscription relationship with an external network, e.g., with an SSPN.

11	MLME

11.1	Synchronization

    - Acquiring synchronization, scanning

11.1.3.2	Active scanning

11.1.3.2.1	Sending a probe response

Change the first paragraph of 11.1.3.2.1 as shown below:

STAs, subject to criteria below, receiving Probe Request frames not containing an Interworking field in the Extended capabilities element set to 1, shall respond with a Probe Response only if

Change the following text after the first paragraph (bulleted list) of 11.1.3.2.1 as shown below:

STAs having dot11InterworkingServicesEnable set to TRUE, subject to criteria below, receiving Probe Request frames containing an Interworking field in the Extended capabilities element set to 1,  shall respond with a Probe Response only if:

11	MLME

11.18	WLAN Interworking with external networks procedures

11.18.1	Interworking capabilities and Information

Change the text as shown below:

STAs indicate their support for Interworking Service by setting the dot11InterworkingServiceEnabled MIB variable to true. When dot11InterworkingServiceEnabled is true, APs include the Interworking element in Beacon and Probe Response frames and non-AP STAs include the Interworking element in Probe Request frames.

11.18.2	Interworking procedures: Generic Advertisement Services

Change the text as shown below:

This subclause describes the actions and procedures that are used to invoke Generic Advertisement Services (GAS). GAS may be used to enable network selection for STAs having dot11InterworkingServiceEnabled set to TRUE.