IEEE P802.11
Wireless LANs

| Action frames and addressing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32771 | +1 407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

The submission is a resolution of comments.

Baseline: D2.06

Instructions to the TGs editor are shown in red.  When the instruction “modify (the text) as shown” is used, red indicates a deletion and blue indicates a deletion.

| CID | Comment / Explanation | Recommended Change | Resolution Notes | What I did |
| --- | --- | --- | --- | --- |
| 1096 | How to propagate Portal Announcement is not clear | Add PANN IE to beacon, describe options (probes) | Good idea. thought and submission needed We could have a section that explains how mesh IEs can be sent. We should also consider unifying all mesh management frames under one umbrella. JKR: propagation rules can be copied/derived from the RANN | 11-09-0115 describes how a PANN is propagated. This submission further specifies how the IE is sent |
| 1130 | I don't think it says anywhere whom the PREQ and the PREP are addressed to. | Add the DA for PREQ and PREP (in the frame format section? Somewhere else?) | Submission needed. Check also other cases. Add "Note: the Address 1 (DA) of the action frame carrying the Root Path Confirmation PREQ element is set to the address of the TA of the RANN that triggere the Root Path Confirmation PREQ." at the end of "Case C: Root Path Confirmation" clause 11B.9.5.2 Conditions for generating and sending a PREQ" -- there are other occurrences that need to be fixed -- there will be a submission later. Suggest lumping with the frame format reorganization | A full table is provided on how to send HWMP action frames. I also deleted sporadic references in the spec so that the information is centralized. |

Modify 7.3.2.98 as shown

- PREQ information element

The Path Request (PREQ) element is used for discovering a path to one or more target mesh STAs, building a proactive (reverse) path selection tree to the root mesh STA, and confirming a path to a target mesh STA (optional).

The format of the PREQ element is shown in PREQ element.

| Element ID | Length | Flags | Hopcount | Time to Live | PREQ ID | Originator mesh STA Address | Originator HWMP Sequence Number | Originator Proxied Address | Lifetime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets:1 | 1 | 1 | 1 | 1 | 4 | 6 | 4 | 0 or 6 | 4 |
| Metric | Target Count | Per Target Flags #1 | Target Address #1 | Target HWMP Sequence Number #1 | ... | Per Target Flags #N | Target Address #N | Target Sequence Number #N |  |
| 4 | 1 | 1 | 6 | 4 | ... | 1 | 6 | 4 |  |
| - PREQ element |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The length is set to 36 or 42 37 to 255 octets.

The format of the Flags field is shown in PREQ Flags field format.

| B0 | B1 | B2 | B3 B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- |
| Portal Role | Addressing Mode | Proactive PREP | Reserved | AE | Reserved |
| Bits: 1 | 1 | 1 | 3 | 1 | 1 |
| - PREQ Flags field format |  |  |  |  |  |

The Flags field is set as follows.

Bit 0: Portal Role (0 = non-portal, 1 = portal).

Bit 1: Addressing Mode (0 = group addressed, 1 = individually addressed).

Bit 2: Proactive PREP (0 = off, 1 = on).

Bit 3-5: Reserved.

Bit 6: Address Extension (AE) (1= proxied device address present, 0 = otherwise).

Bit 7: Reserved.

The Hop Count field is coded as an unsigned integer and is set to the number of hops from the originator to the mesh STA transmitting the request.

The Time to Live field is coded as an unsigned integer and is set to the maximum number of hops allowed for this element.

The PREQ ID field is coded as an unsigned integer and is set to some unique ID for this PREQ.

The Originator mesh STA Address field is represented as a 48-bit MAC address and is set to the originator MAC address.

The Originator HWMP Sequence Number is coded as an unsigned integer and is set to the HWMP sequence number specific to the originator.

The Originator Proxied Address field is the MAC address of an entity proxied by the Originator.(e.g. STA) This field is only present if the AE flag is set to 1 and is represented as a 48-bit MAC address.

The Lifetime field is coded as an unsigned integer and is set to the time for which mesh STAs receiving the PREQ consider the forwarding information to be valid. The lifetime is measured in TUs.

The Metric field is coded as an unsigned integer and is set to the cumulative metric from the originator to the mesh STA transmitting the PREQ.

The Target Count N field is coded as an unsigned integer and gives the number of targets (N) contained in this PREQ.

The format of the Per Target Flags field is shown in PREQ Per Target Flags field format.

| B0 | B1 | B2 | B3 B7 |
| --- | --- | --- | --- |
| TO | RF | USN | Reserved |
| Bits: 1 | 1 | 1 | 5 |
| - PREQ Per Target Flags field format |  |  |  |

Per Target Flags are set as follows.

Bit 0: TO (Target Only): If TO=0, an intermediate mesh STA with active forwarding information to the corresponding target responds to the PREQ with a individually addressed PREP; if TO=1, only the target mesh STA can respond with a individually addressed PREP. The default value is 1.

Bit 1: RF (Reply-and-Forward): The RF flag controls the forwarding of PREQ at intermediate mesh STAs. When TO=0 and the intermediate mesh STA has active forwarding information to the corresponding target, the PREQ is not forwarded if RF=0 and forwarded if RF=1. The default value is 1. When TO=1, the RF flag has no effect.

Bit 2: USN (Unknown Target HWMP Sequence Number): The USN flag indicates whether the Target HWMP Sequence Number field of the corresponding target can be interpreted as HWMP sequence number (USN=0) or not (USN=1), the latter meaning that a target HWMP sequence number is unknown at the originator mesh STA.

Bit 3-7: Reserved

The Target Address is represented as a 48-bit MAC address.

The Target HWMP Sequence Number field is coded as an unsigned integer and is the latest known HWMP sequence number received in the past by the originator mesh STA for any path towards the target. If such a target HWMP sequence number is now known, the USN bit is set to 1 and the contents of this field is ignored.

Detailed usage of the PREQ element is described in Path Request (PREQ).

The PREQ element may be transmitted to a neighbor peer mesh STA via either individually addressed or group addressed. A “individually addressed PREQ” is a PREQ element contained in a management frame that is individually addressed to a neighbor peer mesh STA. A “broadcast group addressed PREQ” is a PREQ element contained in a management frame that is broadcast group addressed to all neighbor peer mesh STAs.  Detailed addressing information is provided in clause 11B.9.4 Addressing of Mesh Path Selection action frame.

Modify 7.4.14 as shown

- Mesh Path Selection action frame details

The Action frame formats for management of Mesh Path Selection are is defined in this subclause.  Detailed addressing information for this action frame is provided in clause 11B.9.4 Addressing of Mesh Path Selection action frame.

An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in Mesh Path Selection Action field values.

| - Mesh Path Selection Action field values |  |
| --- | --- |
| Action field value | Description |
|  | Path Request Mesh Path Selection |
|  | Path Reply |
|  | Path Error |
|  | Root Announcement |
| 14-255 | Reserved |

Add 7.4.14.1 as shown

- Mesh Path Selection frame format

The Mesh Path Selection frame is transmitted by a mesh STA for the purpose of establishing paths to other mesh STAs using the HWMP protocol defined in Error: Reference source not found. Frame body of a Path Request frame contains the information shown in Path Request frame body.  The Mesh Path Selection frame may contain any number of information elements defined in Table s21.

| - Mesh Path Selection frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
| - – a | Root Announcement element (optional) |
| a+1 – b | Path Request element (optional) |
| b+1 – c | Path Reply element (optional) |
| c+1 – d | Path Error element (optional) |

The Category field is set to the value in Table 7-24 for category Mesh Path Selection.

The Action field is set to the value in Table s20 for this action frame type.

The Root Announcement element is set as described in 7.3.2.97.

The Path Request element is set as described in 7.3.2.98.

The Path Reply element is set as described in 7.3.2.99.

The Path Error element is set as described in 7.3.2.100.

Delete 7.4.14.1 through 7.4.14.4 as shown

- Path Request frame format

The Path Request frame is transmitted by an originator mesh STA to discover the path to the target using the HWMP protocol defined in Error: Reference source not found. This frame may be transmitted using group addresses or individual addresses. The frame body of a Path Request frame contains the information shown in Path Request frame body.

| - Path Request frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | Path Request element |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Mesh Path Selection Action field values for this action frame type.

The Path Request element is set as described in PREQ information element.

- Path Reply frame format

The Path Reply frame is transmitted by a target mesh STA to a originator mesh STA in the mesh to determine the path between the source and destination mesh STA using the HWMP protocol defined in Error: Reference source not found. This frame is transmitted using individual addresses. The frame body of a Path Reply frame contains the information shown in Path Reply frame body.

| - Path Reply frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | Path Reply element |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Mesh Path Selection Action field values for this action frame type.

The Path Reply element is set as described in Error: Reference source not found.

- Path Error frame format

The Path Error frame is transmitted by a mesh STA that detected a link error on a mesh path to the precursor mesh STA(s) using the HWMP protocol defined in Error: Reference source not found. This frame may be transmitted using group addresses or individual addresses. The frame body of a Path Error frame contains the information shown in Path Error frame body.

| - Path Error frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | Path Error element |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Mesh Path Selection Action field values for this action frame type.

The Path Error element is set as described in Error: Reference source not found.

- Root Announcement frame format

The Root Announcement frame is transmitted by a Root mesh STA using the HWMP protocol defined in Error: Reference source not found. This frame may be transmitted using group addresses or individual addresses. The frame body of a Root Announcement frame contains the information shown in Root Announcement frame body.

| - Root Announcement frame body |  |
| --- | --- |
| Order | Information |
|  | Category |
|  | Action Value |
|  | Root Announcement element |

The Category field is set to the value in Error: Reference source not found for category Mesh Path Selection.

The Action field is set to the value in Mesh Path Selection Action field values for this action frame type.

The Root Announcement element is set as described in Error: Reference source not found.

Add 11B.9.4 as shown

11B.9.4 Addressing of Mesh Path Selection action frame

All HWMP information elements are sent in a Mesh Path Selection action frame (see clause 7.4.14 Mesh Path Selection action frame details).  The RANN information element may also be sent in a beacon frame.

| Information element sent | Address 1 | Address 2 | Address 3 | Note |
| --- | --- | --- | --- | --- |
| PREQ Case C (individual address) | Address 2 of the frame containing the RANN information element that triggered the PREQ | Address of the mesh STA sending the PREQ | Same as Address 2 | Address 2 does not have to be the same as the “Originator mesh STA Address” in the PREQ information element |
| PREQ Case D (if previous PREQ received was individually addressed), Case B | Next hop MAC address to the Mesh STA identified as the Target MAC address in the PREQ information element | Address of the mesh STA sending the PREQ | Same as Address 2 | Address 2 does not have to be the same as the “Originator mesh STA Address” in the PREQ information element |
| PREQ Case D (if previous PREQ received was group addressed), Case A, Case B, Case E | Group address | Address of the mesh STA sending the PREQ | Same as Address 2 | Address 2 does not have to be the same as the “Originator mesh STA Address” in the PREQ information element |
| PREP Case A, B | address of the next hop to the Originator mesh STA Address of the PREQ that triggered the PREP | Address of the mesh STA sending the PREP | Same as Address 2 |  |
| RANN | Group address | Address of the mesh STA sending the RANN | Same as Address 2 | Address 2 does not have to be the same as “Root mesh STA Address” in the RANN information element |
| PERR (individual address) | Address | Address of the mesh STA sending the PERR | Same as Address 2 |  |
| PERR | Group address | Address of the mesh STA sending the PERR | Same as Address 2 |  |

Multiple HWMP information elements may be sent in the same Mesh Path Selection action frame as long as they share the same intended Address 1.

Delete the following note:

Note: the Address 1 (DA) of the action frame carrying the PREP element is set to the address of the next hop to the Originator mesh STA Address of the PREQ that triggered the PREP.

Delete the following note:

Note: the DA of the action frame carrying the PREP element is set to the address of the next hop to the Originator mesh STA Address of the PREQ that triggered the PREP.

Update 11B.9.5. Path request as follows:

- Path Request (PREQ)

This subclause describes the function, generation and processing of the Path Request information element.

- Function

The Path Request (PREQ) element, described in PREQ information element, is used for three purposes:

- Discovering a path to one or more targets

- Building a proactive (reverse) path selection tree to the root mesh STA

- Path maintenance (optional)

- Conditions for generating and sending a PREQ

When an STA implements support for one or more of the procedures described in this clause, it shall set dot11MeshEnabled attribute to true.

A mesh STA shall send a PREQ element in a PREQ frame in the following cases:

Case A: Original Transmission (Path Discovery)

All of the following applies:

- The mesh STA needs to establish an on-demand path to one or more target mesh STAs or target proxy mesh STA for which there is no ongoing path discovery initiated by this mesh STA.

- The mesh STA has not sent a PREQ element less than dot11MeshHWMPpreqMinInterval TUs ago. If this is the case, the transmission of the PREQ has to be postponed until this condition becomes true.

- The mesh STA has not made more than (dot11MeshHWMPmaxPREQretries – 1) repeated attempts at path discovery towards the target of the PREQ.

The content of a PREQ element in Case A shall be as shown in Contents of a PREQ in Case Einput.doc.

| - Content of a PREQ element in Case Ainput.doc |  |  |
| --- | --- | --- |
| Field |  | Value |
| ID |  | Value given in Error: Reference source not found for the PREQ element |
| Length |  | 26 + N*11 (if Bit 6 in Flags (AE) = 0) 32 + N*11 (if Bit 6 in Flags (AE) = 1) |
| Flags |  | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: Address Extension (AE) (1 – if proxied address present, 0 – otherwise) Bit 7: Reserved |
| Hop Count |  | 0 |
| Time to Live |  | Maximum number of hops allowed for this information element, e.g., dot11MeshHWMPnetDiameter. |
| PREQ ID |  | Previous PREQ ID + 1 |
| Originator mesh STA Address |  | MAC address of the originator of the PREQ |
| Originator HWMP Sequence Number |  | Previous Originator HWMP SN + 1. See Error: Reference source not found |
| Proxied Address |  | Present only if Bit 6 in Flags = 1. This value is set to the proxied address which is the source of the frame. |
| Lifetime |  | The time for which mesh STAs receiving the PREQ consider the forwarding information to be valid, e.g. dot11MeshHWMPactivePathTimeout. |
| Metric |  | Initial value of active path selection metric |
| Target Count |  | N (N>=1) |
| Per Target | Flags | The TO flag is set to dot11MeshHWMPTargetOnly and the RF flag is set to dot11MeshHWMPReplyAndForward Bit 0 (TO): dot11MeshHWMPTargetOnly Bit 1 (RF): dot11MeshHWMPReplyAndForward Bit 2 (USN): 0 if forwarding information for Target Address with valid HWMP sequence number exists, 1 otherwise. |
|  | Target Address | MAC address of requested target mesh STA |
|  | Target HWMP Sequence Number | The latest HWMP sequence number stored by the originator mesh STA for the target mesh STA from the forwarding information (seeError: Reference source not found Forwarding information). |

Case B:	Original Transmission (Path Maintenance) (optional)

All of the following applies:

- the mesh STA has a path to a given target that is not a Root mesh STA

- the last PREQ to this target was sent dot11MeshHWMPmaintenanceInterval TUs (or more) ago.

The content of a PREQ in Case B shall be as shown in Content of a PREQ element in Case B.

| - Content of a PREQ element in Case B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field |  | Value |  |  |
| ID |  | Value given in Error: Reference source not found for the PREQ element |  |  |
| Length |  | 26 + N*11 |  |  |
| Flags |  | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |  |  |
| Hop Count |  | 0 |  |  |
| Time to Live |  | Maximum number of hops allowed for this information element = dot11HWMPNetDiameter. |  |  |
| PREQ ID |  | Previous PREQ ID +1 |  |  |
| Originator mesh STA Address |  | MAC address of the originator of the PREQ |  |  |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Error: Reference source not found |  |  |
| Proxied Address |  | Field not present in this element |  |  |
| Lifetime |  | The time for which mesh STAs receiving the PREQ consider the forwarding information to be valid, e.g. dot11MeshHWMPactivePathTimeout. |  |  |
| Metric |  | Initial value of active path selection metric |  |  |
| Target Count |  | N (N>=1) |  |  |
| Per Target | Flags |  | TO flag = 1 RF flag = 0 Bit 0 (TO): 1 Bit 1 (RF): 0 Bit 2 (USN): 0 if forwarding information for Target Address with valid HWMP sequence number exists, 1 otherwise. |  |
|  | Target Address |  | MAC Address of target mesh STA |  |
|  | Target HWMP Sequence Number |  | The latest HWMP sequence number for this target known to the originator mesh STA |  |

Case C: Root Path Confirmation.

One of the following applies:

- the mesh STA has received a RANN from a root mesh STA and does not have a path to a root mesh STA

- the mesh STA has a path to a root mesh STA and the last PREQ to the root mesh STA was sent dot11MeshHWMPconfirmationInterval TUs (or more) ago.

The content of a PREQ element in Case C shall be as shown in Content of a PREQ element in Case C.

| - Content of a PREQ element in Case C |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Error: Reference source not found for the PREQ element |  |
| Length |  | As required |  |
| Flags |  | Bit 0: 0 (no portal role) Bit 1: 1 (individually addressed) Bit 2: 0 (no proactive PREP applicable) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |  |
| Hop Count |  | 0 |  |
| Time to Live |  | Maximum number of hops allowed for this information element, e.g., dot11MeshHWMPNetDiameter |  |
| PREQ ID |  | Not used |  |
| Originator mesh STA Address |  | MAC address of the originator mesh STA |  |
| Originator HWMP Sequence Number |  | Originator HWMP SN + 1. See Note 2 under Case A. |  |
| Proxied Address |  | Field not present in this element |  |
| Lifetime |  | The time for which mesh STAs receiving the PREQ consider the forwarding information to be valid, e.g. dot11MeshHWMPactiveRootTimeout. |  |
| Metric |  | Initial value of active path selection metric |  |
| Target Count |  | 1 |  |
| Per Target | Flags | TO flag = 1, RF flag = 0 Bit 0 (TO): 1 Bit 1 (RF): 0 Bit 2 (USN): 0 |  |
|  | Target Address | Root mesh STA MAC Address |  |
|  | Target HWMP Sequence Number | The latest HWMP sequence number for this target known to the originator mesh STA |  |

Case D:	PREQ Propagation

Case D1 (target count = 1, no PREP generation):

All of the following applies:

- the mesh STA has received and accepted a PREQ – See Acceptance criteria

- the HWMP TTL field is greater than 1 – See Error: Reference source not found

- Target Count = 1

- the mesh STA is not the target of the PREQ OR the target of the PREQ is the MAC broadcast address (all 1’s)

- the mesh STA is not the proxy of the target address

- [Target Only flag of the target in the PREQ is ON (TO = 1)] 
OR
[{Target Only flag of the target in the PREQ is OFF (TO = 0)} AND {mesh STA has no active forwarding information for the requested target}]

The content of a PREQ element in Case D1 shall be as shown in Content of a PREQ element in Case D1.

| - Content of a PREQ element in Case D1 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Error: Reference source not found for the PREQ element |  |
| Length |  | As received |  |
| Flags |  | As received |  |
| Hop Count |  | As received + 1 |  |
| Time to Live |  | As received – 1 |  |
| PREQ ID |  | As received |  |
| Originator mesh STA Address |  | As received |  |
| Originator HWMP Sequence Number |  | As received |  |
| Proxied Address |  | As received. This field is only present if Bit 6 of the Flags field (AE flag) is set to 1. |  |
| Lifetime |  | As received |  |
| Metric |  | As receivedown metric toward transmitter of received PREQ |  |
| Target Count |  | 1 |  |
| Per Target | Flags | As received |  |
|  | Target MAC Address | As received |  |
|  | Target HWMP Sequence Number | As received |  |

Case D2 (target count = 1, PREP generation as intermediate mesh STA):

All of the following applies:

- the mesh STA has received and accepted a PREQ – See Acceptance criteria

- the HWMP TTL field is greater than 1 – See Error: Reference source not found

- Target Count = 1

- the mesh STA is not the target of the PREQ

- the mesh STA is not the proxy of the target address

- the mesh STA has active forwarding information for the requested target

- Target Only flag of the target in the PREQ is OFF (DO = 0)

- Reply and Forward flag of the target in the PREQ is ON (RF = 1)

The contents of a PREQ element in Case D2 shall be as shownContents of a PREQ element in Case D2.

| - Contents of a PREQ element in Case D2 |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Error: Reference source not found for the PREQ element |  |
| Length |  | As received |  |
| Flags |  | As received |  |
| Hop Count |  | As received + 1 |  |
| Time to Live |  | As received – 1 |  |
| PREQ ID |  | As received |  |
| Originator mesh STA Address |  | As received |  |
| Originator HWMP Sequence Number |  | As received |  |
| Proxied Address |  | As received. This field is only present if Bit 6 of the Flags field (AE flag) is set to 1. |  |
| Lifetime |  | As received |  |
| Metric |  | As receivedown metric toward transmitter of received PREQ |  |
| Target Count |  | 1 |  |
| Per Target | Flags | Bit 0 (TO): 1 (set to 1 before forwarding because mesh STA sent a PREP) Bit 1 (RF): 0 Bit 2 (USN): As received |  |
|  | Target MAC Address | As received |  |
|  | Target HWMP Sequence Number | As received |  |

Case D3 (target count > 1): All of the following applies

- the mesh STA has received and accepted a PREQ – See Acceptance criteria

- the HWMP TTL field is greater than 1 – See Error: Reference source not found

- there is at least one requested target left after processing the PREQ according to PREQ processing.

The contents of a PREQ element in Case D3 shall be as shown in Contents of a PREQ element in Case D3input.doc.

| - Contents of a PREQ element in Case D3input.doc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field |  | Value |  |  |
| ID |  | Value given in Error: Reference source not found for the PREQ element |  |  |
| Length |  | 26 + N * 11 |  |  |
| Flags |  | As received |  |  |
| Hop Count |  | As received + 1 |  |  |
| Time to Live |  | As received – 1 |  |  |
| PREQ ID |  | As received |  |  |
| Originator mesh STA Address |  | As received |  |  |
| Originator HWMP Sequence Number |  | As received |  |  |
| Proxied Address |  | As received. This field is only present if Bit 6 of the Flags field (AE flag) is set to 1. |  |  |
| Lifetime |  | As received |  |  |
| Metric |  | As receivedown metric toward the transmitter of the received PREQ |  |  |
| Target Count |  |  | 1 ≤ target count ≤ received target count received target count less the number of requested destinations, for which the processing mesh STA — is the target mesh STA or — is the target proxy mesh STA or — has active forwarding information for the requested target and the corresponding Target Only flag is off (TO=0) and Reply and Forward Flag is off (RF=0) |  |
| Per Target #A | Flags #A | As received |  |  |
|  | Target MAC Address #A | As received |  |  |
|  | Target HWMP Sequence Number #A | As received |  |  |
| Per Target #B | Flags #B | Bit 0 (TO): 1 (set to 1 because mesh STA sent PREP) Bit 1 (RF): 0 Bit 2 (USN): As received |  |  |
|  | Target MAC Address #B | As received |  |  |
|  | Target HWMP Sequence Number #B | As received |  |  |

For the per target fields (per target flags, target mesh STA MAC address, target HWMP sequence number) assume the following:

- target #A: If target A would have been the only requested target, it would generate a PREQ for propagation according to case D1.

- target #B: If target B would have been the only requested target, it would generate a PREQ for propagation according to case D2.

Case E:	Proactive PREQ (original transmission)

All of the following applies:

- The Root mesh STA is configured to send proactive root PREQs

- The Root Announcement interval has expired

The contents of a PREQ in Case E shall be as shown in Contents of a PREQ in Case Einput.doc.

| - Contents of a PREQ in Case Einput.doc |  |  |  |
| --- | --- | --- | --- |
| Field |  | Value |  |
| ID |  | Value given in Error: Reference source not found for the PREQ element |  |
| Length |  | 37 |  |
| Flags |  | Bit 0: As needed (portal role) Bit 1: 0 (broadcast) Bit 2: As needed (proactive PREP) Bit 3 – 5: Reserved Bit 6: 0 (no address extension) Bit 7: Reserved |  |
| Hop Count |  | 0 |  |
| Time to Live |  | Maximum number of hops allowed for this information element, e.g. dot11HWMPNetDiameter. |  |
| PREQ ID |  | Previous PREQ ID + 1 |  |
| Originator mesh STA Address |  | root mesh STA MAC address |  |
| Originator HWMP Sequence Number |  | Previous HWMP sequence number of root mesh STA + 1 |  |
| Proxied Address |  | Field not present in this element |  |
| Lifetime |  | dot11MeshHWMPpathToRootInterval |  |
| Metric |  | Initial value of active path selection metric |  |
| Target Count |  | 1 |  |
| Per Target | Flags | TO = 1, RF = 1 |  |
|  | Target Address | Broadcast address |  |
|  | Target HWMP Sequence Number | 0 |  |

- PREQ processing

Received PREQ elements are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the PREQ and the information available to the receiving mesh STA.

See also Error: Reference source not found: General rules for message processing.

- Acceptance criteria

The PREQ element shall not be accepted (and shall not be processed as described in Effect of receipt) if any of the following is true:

- The Originator HWMP sequence number < HWMP sequence number in forwarding information for Originator mesh STA

- (The Originator HWMP sequence number = HWMP sequence number in forwarding information for Originator mesh STA) AND (PREQ ID has already been received in a PREQ from the same originator) AND (the updated path metric is worse than path metric in forwarding information for Originator mesh STA)

- (the target address of the PREQ is neither the recipient MAC address nor a MAC address proxied by the recipient) AND (dot11MeshForwarding is set to 0)

Otherwise, the PREQ information element is accepted.

See also Error: Reference source not found: General rules for message processing.

- Effect of receipt

The following applies only to a PREQ element that was accepted according to the acceptance criteria in Acceptance criteria:

- The receiving mesh STA shall record the PREQ ID and the Originator mesh STA Address.

- If the originator HWMP sequence number in PREQ is greater than the previous originator HWMP sequence number or the originator HWMP sequence number in PREQ is equal to the previous originator HWMP sequence number and updated path metric is better than the previous path metric, the receiving mesh STA shall update the active forwarding information it maintains for the originator mesh STA and previous hop mesh STA of the PREQ (see Error: Reference source not found)

- If the mesh STA is addressed by the PREQ or is the proxy of the target MAC address it shall initiate the transmission of a PREP to the originator mesh STA (Error: Reference source not found Case A). If the PREQ carries a proxied address (indicated by the AE flag), the mesh STA shall update its proxy information with the proxied address and set the PREQ originator mesh STA address as the corresponding proxy.

- If step 3 was not applicable for the mesh STA and the AE flag in the PREQ is set to 1, the mesh STA may update its proxy information with the proxied address and set the PREQ originator mesh STA address as the corresponding proxy.

- If the mesh STA has valid forwarding information to any of the requested targets and the TO flag for such a target is OFF (TO=0), it initiates the transmission of a PREP for each of these targets (see Error: Reference source not found Case A)

- If there are targets in the PREQ that have been not processed in steps 3 or 5 or that have been processed in step 5 but the corresponding Reply and Forward Flag is ON (RF = 1), the receiving mesh STA shall forward the PREQ as defined in Conditions for generating and sending a PREQ, Case D.

- If the mesh STA is initiating a PREP transmission on behalf of another target (intermediate reply), it should update its forwarding information by placing the last hop mesh STA (from which it received the PREQ) into the precursor list for the forward path entry for the target. In addition, this intermediate mesh STA also updates its forwarding information for the originator mesh STA by placing the next hop toward the target in the precursor list for the reverse path entry.