IEEE P802.11
Wireless LANs

| Resolutions for Transmit beamforming comments of TGn Sponsor Ballot |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hongyuan Zhang | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408 222 1837 | hongyuan@marvell.com |
| Harish Ramamurthy | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408 222 9683 | harishr@marvell.com |

Abstract

This submission proposed resolutions to the beamforming CIDs 133~139 for the PHY comment subgroup.

The changes proposed in this document are based on 802.11n D7.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

CID 133, 134, 135

| 133 | 9.19.3 | 177.22 | "shall transmit an immediate or aggregated feedback..."by definition, aggregated feedback is one case of immediate feedback. It is better to differentiate SIFS immediate feedback, and aggregated feedback. Line 46 also has the same issue | Either redefine the categories of the feedback, or replace the text by "shall transmit an immdiate SIFS feedback, or aggregated feedback" | Agree in principle, replace the text as suggested |
| --- | --- | --- | --- | --- | --- |
| 134 | 9.19.3 | 177.21 | "A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field of its HT Capabilities element to either 2 or 3 shall transmit an immediate or aggregated feedback as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence as follows:" Are these rules not applicable for compressed and non-compressed feedback? Change text as suggested | A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field or Explicit Non-compressed Beamforming Feedback Capable or Explicit Compressed Beamforming Feedback Capable of its HT Capabilities element to either 2 or 3 shall transmit an immediate or aggregated feedback as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence as follows: | Agree |
| 135 | 9.19.3 | 177.43 | "A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field of its HT Capabilities element to either 2 or 3 shall transmit immediate or aggregated feedback as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence as follows:" Are these rules not applicable for compressed and non-compressed feedback? Change text as suggested | A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field or Explicit Non-compressed Beamforming Feedback Capable or Explicit Compressed Beamforming Feedback Capable of its HT Capabilities element to either 2 or 3 shall tranmsit immediate or aggregated feedback as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence as follows: | Agree |

TGn Editor: on line 21 page 177 of D7.0, make the following change:

“A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field of its HT Capabilities element to either 2 or 3 shall transmit Explicit CSI feedback after SIFS or later in the beamformer’s TxOP an immediate or aggregated feedback as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table XX: Rules for beamformee immediate feedback transmission responding to Non-NDP sounding. as follows:

A beamformee that sets the Explicit Non-compressed Beamforming Feedback Capable field of its HT Capabilities element to either 2 or 3 shall transmit Explicit Non-compressed Beamforming feedback after SIFS or later in the beamformer’s TxOP as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table XX: Rules for beamformee immediate feedback transmission responding to Non-NDP sounding.

A beamformee that sets the Explicit Compressed Beamforming Feedback Capable field of its HT Capabilities element to either 2 or 3 shall transmit Explicit Compressed Beamforming feedback after SIFS or later in the beamformer’s TxOP as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table XX: Rules for beamformee immediate feedback transmission responding to Non-NDP sounding.

Table XX: Rules for beamformee immediate feedback transmission responding to Non-NDP sounding ” (editorial note: move the bullets below the mentioned paragraph into a table)

TGn Editor: on line 43 page 177 of D7.0, make the following change:

" A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field of its HT Capabilities element to either 2 or 3 shall transmit the Explicit CSI feedback after SIFS or later in the beamformer’s TxOP an immediate or aggregated feedback as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table XX: Rules for beamformee immediate feedback transmission responding to NDP sounding. as follows:

A beamformee that sets the Explicit Non-compressed Beamforming Feedback field of its HT Capabilities element to either 2 or 3 shall transmit the Explicit Non-compressed Beamforming feedback after SIFS or later in the beamformer’s TxOP as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table XX: Rules for beamformee immediate feedback transmission responding to NDP sounding.

A beamformee that sets the Explicit Compressed Beamforming Feedback field of its HT Capabilities element to either 2 or 3 shall transmit the Explicit Compressed Beamforming feedback after SIFS or later in the beamformer’s TxOP as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table XX: Rules for beamformee immediate feedback transmission responding to NDP sounding.

Table XX: Rules for beamformee immediate feedback transmission responding to NDP sounding "

(editorial note: move the bullets below the mentioned paragraph into a table)

CID 138, 139

| 138 | 9.19.3 | 178.17 | "A beamformee transmitting either immediate or aggregated feedback response shall use an Action No Ack +HTC frame (defined in 7.2.3.13)." First, agregated feedback is included in immediate feedback as in current spec. Secondly, it does not make sense that +HTC is mandatory for immediate feedback, but optional for delayed feedback (see the next comment). Better to make them unified. | Replace it with "A beamformee transmitting immediate feedback response shall use an Action No Ack frame, or Action No Ack +HTC frame (defined in 7.2.3.13)" | Agree |
| --- | --- | --- | --- | --- | --- |
| 139 | 9.19.3 | 178.20 | "A beamformee transmitting delayed feedback response shall use an Action frame to send this information within a separate TXOP." The beamformee may send unsolicted MCS feedback and hence it may use Action + HTC frame | Replace it with "A beamformee transmitting delayed feedback response shall use an Action frame or Action frame+HTC to send this information within a separate TXOP." | Agree |

Discussions: The feedback frame may or may not contain MCS feedback and hence it is appropriate to make it optional.

TGn Editor: on line 17 page 178 of D7.0, make the following changes:

“A beamformee transmitting either feedback response after SIFS or later in the beamformer’s TxOP  immediate or aggregated feedback response shall use an Action No Ack frame, or Action No Ack +HTC frame (defined in 7.2.3.13).”.

TGn Editor: on line 20 page 178 of D7.0, make the following changes:

“A beamformee transmitting delayed feedback response shall use an Action frame or an Action +HTC frame to send this information within a separate TXOP.”

CID 136

| 136 | 9.19.3 | 177.58 | "If the feedback response frame is not transmitted a SIFS after the reception of the NDP, and is not transmitted as part of an aggregated ACK or BlockAck response in the same TXOP, then the feedback response frame is delayed until the beamformee's next transmission within the TXOP." Take the scenario when beamformer uses NDP. How does the beamformer distinguish whether the beamformee will send feedback response SIFS after receiving NDP or will send it as an aggregated frame in beamformer's TXOP. Say beamformee always uses the latter method, the beamformer would have to wait for PIFS after NDP transmission always? Given that both options need to be supported, some additionally signalling may need to be introduced. Similar complications arise with "Immediate and Delayed" option. | Choice #1-Modify the capability field: Change "Immediate" option to "Immediate and non-aggregated". Change "Immediate and Delayed" option to "Immediate and aggregated".Choice #2-Mandate the Beamformer's behavior on receiving the immediate feedback, e.g. beamformer needs to wait SIFS time after sending an NDP to a beamformee supporting immediate or immediate and delayed feedback | Agree in principle |
| --- | --- | --- | --- | --- | --- |

Discussions: If the beamformee is capable of generating immediate feedback, there is a possibility of collision if the BFMer sends a packet SIFS/RIFS after sending an NDP or Non-NDP sounding packet with No ACK. It is better to constrain the BFMer’s behavior rather than redefine the categories of the feedback.

TGn Editor: insert a paragraph at line 10 page 178 of D7.0 as below:

“............…

--If the feedback response frame is not transmitted a SIFS after the reception of the NDP, and is not transmitted as part of an aggregated ACK or BlockAck response in the same TXOP, then the feedback response frame is delayed until the beamformee’s next transmission within the TXOP.

When the beamformee sets the Explicit Transmit Beamforming CSI Feedback field of its HT Capabilities element to either 2 or 3, and the beamformer has transmitted an NDP, or an non-NDP Explicit Beamforming CSI feedback request in a frame that does not require immediate control response, the beamformer shall not transmit the next packet to the beamformee, until PIFS after transmitting the sounding request.

When the beamformee sets the Explicit Non-compressed Beamforming Feedback Capable field of its HT Capabilities element to either 2 or 3, and the beamformer has transmitted an NDP, or an non-NDP Explicit Non-Compressed Beamforming feedback request in a frame that does not require immediate control response, the beamformer shall not transmit the next packet to the beamformee, until PIFS after the sounding request.

When the beamformee sets the Explicit Compressed Beamforming Feedback Capable field of its HT Capabilities element to either 2 or 3, and the beamformer has transmitted an NDP, or an non-NDP Explicit Compressed Beamforming feedback request in a frame that does not require immediate control response, the beamformer shall not transmit the next packet to the beamformee, until PIFS after transmitting the sounding request.

A beamformee shall not transmit a CSI, Compressed Beamforming or Non-compressed Beamforming frame except in response to a request for feedback.

………“

CID 137

| 137 | 9.19.3 | 177.30 | "If the transmission of an ACK or BlockAck control response frame is required in response to the non-NDP request for feedback, both the feedback response frame and the control response frame may be aggregated in an A-MPDU." For non-NDP sounding sending a feedback SIFS after a control response is not allowed. This forces the beamformee to transmit the feedback frame as part of an aggregate for immediate feedback. This option is allowed for NDP sounding and there is no reason why it shouldn't be allowed for non-NDP sounding. Change text as suggested | "If the transmission of an ACK or BlockAck control response frame is required in response to the non-NDP request for feedback, both the feedback response frame and the control response frame may be aggregated in an A-MPDU, or the feedback response frame may be transmitted SIFS after transmitting the control response." | Disagree: withdrawn by the commenter per the email from the commenter ( From: Hongyuan Zhang Sent: Monday, January 19, 2009 2:50 PM To: 'Vinko Erceg'; Bruce Kraemer Cc: Harish Ramamurthy Subject: Withdraw CID 137 Hi Vinko, Bruce, I would like to withdraw comment #137 (for beamforming). Thanks Hongyuan ) |
| --- | --- | --- | --- | --- | --- |