IEEE P802.11
Wireless LANs

| TGp LB 141 Annex J Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-02-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Alastair Malarky | Mark IV Industries | 6030 Ambler Drive, Mississaugua, Ontario, Canada L4W 2P1 | 905-624-3020 | amalarky@ivhs.com |

Abstract

This document addresses CIDs numbers 209-210, 212-215 all of which apply to Annex J of P802.11pD5.

Some of the comment resolution results in changes to Annex I as well as Annex J.

The changes to the draft 802.11p amendment are provided and a motion prepared to incorporate them and accept the comment resolutions proposed.

|  | LB141 Comment Resolution |
| --- | --- |

- COMMENTS with Proposed Resolutions:

| CID | Commenter | Clause | Comment | Suggested Remedy | Comment resolution. |
| --- | --- | --- | --- | --- | --- |
| 209 | Roy, Richard | J.2 | Europe currently does not allow 44.8 dBm EIRP. | Change 44.8 dBm to 33dBm in Table J.2 where it occurs. | Counter – Change is accepted. Also EN 302571-1 also does not specify transmit power levels in mW (at the antenna input) so the entries in the mW transmit column are to be deleted. |
| 210 | Roy Richard | Annex J | The Europeans have requested addition of 10MHz and 40MHz "regulatory classes" in the 5.5GHz band for ITS use. | Add two more regulatory classes to table J-2, one for 10MHz channels (99-141) and the other for 40MHz channels (102-134) both with ch starting freq 5GHz. | Conuter - Accept in principle. Results in need to update Table I.2 and add new ITS 20 MHz entry also. |
| 212 | Roy, Richard | J.2.2 | The statement: "STAs shall be classified for operation in this band by their Maximum TX Power capability, as listed in Table I.5a in I.2, at which power level they shall be compliant to the spectral emission requirements listed in I.2.3" is ambiguous. The question is "whcih power level" is to be used for compliance testing, the max for the class, or the max the unit is capable of. It makes a big difference potentially. Consider that a 1.01mW output device is a Class B device. Does it get to meet Class B power mask at full class B power of 10mW, or must it meet the Clas B mask scaled down to 1.01 mW. It should be the former but that's not clear from the text. | Clarify the text. | Accept in principle. Text is clarified. |
| 213 | Roebuck, Randy | Annex J.2 | Should "dot11WAVEEnabled" to be added to the elements set to true? | Add "- dot11WAVEEnabled" | Declined – dot11WAVEEnabled has been deleted |
| 214 | Roy, Richard | J.2.2 | There is no reason to restrict: "STAs shall have the following element set to "false": — dot11RadioMeasurementEnabled … The radio measurement functionality may prove to be extremely useful (eg., in congestion control) in some circumstances. Whether or not to set this to FALSE is a system design issue not a standards issue. | Remove this restriction. | Accepted |
| 215 | Roy, Richard | J.2.3 | Specifications for 5.9 operation in Europe are missing. | Add J.2.3 stating that: Band specific operating requirements in Europe are defined in EN 302 571 for the ITS frequency band EN 301 893 for the RLAN frequency band. Furthermore: STAs shall have the following elements set to "true": — dot11RegulatoryClassesRequired, — dot11SpectrumManagementRequired, —dot11OCBEnabled. Also add dot115.9GHzEnabled if that MIB variable is added to 11p. | Counter. Accept in principle. Separate sections defined for each band. Also see comment 214. |

- Recommended Resolution of the Comments:

For comments 209 and 213, see recommended resolution in column “Comment Resolution” in table in section 1.

For comment 210, these additional regulatory classes are being added herein as requested. However the behaviours allowed in the 5.5 GHz band are different to those allowed in the 5.9 GHz band, it is necessary to add EN 301 893 to the emission limit set for ITS operations in Table I-2.  This results in an additional regulatory class for 20 MHz in the 5.5 GHz band for ITS operations.

For comment 212, it is agreed that the text is not fully clear.  The FCC defines four transmit power classes and for each class the emission mask is specified for that class without regard to the actual transmit power of that device.  Therefore a device classified under a particular class must meet the emission mask for that class as specified even if the maximum output power of the device is less than the maximum power permitted in the class.  This is consistent with the base emission mask specification of 802.11 which must be met even if a device is not capable of the maximum transmit power or EIRP permitted under the standard.  Therefore the text has been clarified accordingly.

For comment 214, the original reasoning was provided in 11-08/0907r7 and essentially the reson for setting this false was that requests could be used for denial of service attacks since they were not autheticated when operating with dot11OCBEnabled set true.  However since the management frames have provision for vendor specific information elements, then higher layer authentication can be included in these to authenticate these frames.  Hence the concern over denial of service is no longer an issue.

For comment 215, there needs to be separate sections for each band, in particular because the 5.5 GHz band is also already specified under existing 802.11 amendments and the restriction of dot11OCBEnabled=true can only apply to the ITS specific regulatory classes.

- Proposed Modifications to P802.11pD5

The revised text for the P802.11p amendment is shown below.  Text in red indicates new or modified text in the body of P802.11D5.  For visual impact, where text has been modified in any manner (deleted, changed or added) the local text is also highlighted.  Instructions in green are identification to the P802.11 editor of what changes are required from the baseline of P802.11pD5.  Strikeout and underline identify the changes from the amendment base (IEEE 802.11-2007 and subsequent amendments in the queue prior to P802.11p).  Thus the P802.11p editor can, where applicable, just remove the text color and highlighting from items following the green instructions and then copy and paste them into the amendment document.

Add EN 301 893to the European entry for ITS Radio Service in Table I.2:

- Table I.2—Emissions limits sets

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| <ANA> ITS radio service | FCC 47 CFR [B8], Sections 90.375, 90.377, 90.379, 95.639 and 95.1511 | ETSI EN 302 571 ETSI EN 310 893 | Reserved |
| 7 <ANA+1> - 255 | Reserved | Reserved | Reserved |

Modify instruction, modify TX power limits (mW) and add 3 new entries to Table J.2 Regulatory classes in Europe as shown in following :

Insert 7 new entries and the footnote to Table J.2, before the last row and change the numbering of the last row accordingly as shown. Note that <ANA > means that the number is to be assigned by ANA, <ANAE> refers to the ANA number assigned to the new entry in Table I.2 through this amendment, while <ANAD> refers to the ANA numbers assigned to the new entries in Table I.3 through this amendment :

Table J.2—Regulatory classes for 5 GHz bands in Europe.

| Regulatory Class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <ANA>1 | 5.0025 | 5 | 170-184 | - | 33 dBm | <ANAE> | <ANAD>, <ANAD+1> |
| <ANA+1>1 | 5 | 10 | 171-184 | - | 33 dBm | <ANAE> | <ANAD>, <ANAD+1> |
| <ANA+2>1 | 5 | 20 | 172-183 | - | 23 dBm | <ANAE> | <ANAD>, <ANAD+1> |
| <ANA+3>1 | 5 | 40 | 174-181 | - | 23 dBm | <ANAE> | <ANAD>, <ANAD+1> |
| <ANA+4> | 5 | 10 | 99-141 | - | 30 dBm | <ANAE> | 1, 3, 4 |
| <ANA+5> | 5 | 20 | 100-140 | - | 30 dBm | <ANAE> | 1, 3, 4 |
| <ANA+6> | 5 | 40 | 102-138 | - | 30 dBm | <ANAE> | 1, 3, 4 |
| 16<ANA+7>-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

This regulatory class specifies a list of channels in the 5.9GHz band.  Current regulations may only permit a subset of these channels

Change paragraph beginning “STAs shall be classified…” in J.2.2 to read

STAs shall be classified for operation in this band by their Maximum TX Power capability, as listed in Table I.5a in I.2.  STAs shall be compliant to the spectral emission requirements for their class listed in I.2.3.

Delete the paragraph beginning “STAs shall have the following element set to "false":…” in J.2.2

Add instruction and 2 new paragraph J.2.3 and J.2.4

- Add the following new sections J.2.3 and J.2.4, noting <ANAR+x> refers to the <ANA+x> numbers assigned in Table J.2 through this amendment.

- J.2.3 5.470 to 5.725 GHz in Europe

STAs in Regulatory Classes <ANAR+4> through <ANAR+6> shall have the following elements set to “true”:

- dot11MultiDomainCapabilityEnabled,

- dot11RegulatoryClassesRequired,

- dot11SpectrumManagementRequired

- dot11OCBEnabled

- J.2.4 5.850 to 5.925 GHz in Europe

STAs shall have the following elements set to “true”:

- dot11MultiDomainCapabilityEnabled,

- dot11RegulatoryClassesRequired,

- dot11SpectrumManagementRequired,

- dot11OCBEnabled.

- Motion (if technical and/or significant):

Move to accept the Recommended Resolutions to these comments, based on the changes made to the D5.0 as a specified in Section 3 above:

- Motion by: ____________________Date: _________________

- Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

- References:

The following documents have been considered in the assessment herein:

- IEEE Std 802.11-2007

IEEE Std 802.11k-2008

IEEE Std 802.11y-2008

IEEE P802.11p-D5

- FCC 47 CFR Part 90

- FCC 47 CFR Part 95

- ASTM E2213-03

EN 301 893 v1.5.1

11-08/0907r7

11-08/1452r1

1