IEEE P802.11
Wireless LANs

| Proposed Resolutions for Adjustment of SA Query Transaction Identifer Field |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 264 3759 | Kapil.sood@intel.com |

Abstract

This document contains proposed resolutions to the following comments from SB on D7.0

- CID # 41

- CID # 57

All changes apply to TGw draft D7.0, unless specified otherwise.

# CID #41 and 57

Proposed Resolution to CID 57 as “Resolved by the resolution of Comment Number 41”

Resolution for CID 41 is given below:

Change 7.4.9.1, page 21, line 37

Change “Transaction Identifier” octets to “2”.

Change 7.4.9.1, page 21, line 49

The Transaction Identifier field is a 16-bit non-negative counter value set by the STA sending the SA Query Request frame to identify any outstanding request/response transaction.

Change 7.4.9.2, page 21, line 64

Change “Transaction Identifier” octets to “2”.

Change 10.3.39.1.2, page 54, line 33

Change 10.3.39.2.2, page 55, line 14

Change 10.3.39.3.3, page 56, line 8

Change 10.3.39.4.2, page 56, line 47

The “Type” field for “TransactionIdentifier” as

2 octets

Move table in 10.3.39.3.3. to 10.3.39.3.2.

Change 11.3.2.2, page 57, line 43-53

Following this, if the AP is not already engaging in an SA Query with the STA, the AP shall issue one MLME-SAQuery.request primitive to the STA every dot11AssociationSAQueryRetryTimeout TUs until a matching MLME-SAQUERY.confirm is received or dot11AssociationSAQueryMaximumTimeout TUs from the beginning of the SA Query procedure has passed.  The STA shall insert the TransactionIdentifier in SA Query Request, and increment this by 1 for each subsequent SA Query Request, and rolling over to 0 after maximum allowed value in this field.  An AP may interpret reception of a valid protected frame as an indication of a successfully completed SA Query. If an MLME-SAQUERY.confirm with an outstanding transaction identifier is not received within dot11AssociationSAQueryTimeout period, the AP shall allow the association process to be started without starting additional SA Query procedure.

Change 11.3.2.4, page 58, line 11-21

Following this, if the AP is not in an ongoing SA Query with the STA, the AP shall issue one MLME-SAQuery.request primitive to the STA every dot11AssociationSAQueryRetryTimeout TUs until a matching MLMESAQUERY. confirm is received or dot11AssociationSAQueryMaximumTimeout TUs from the beginning of the SA Query procedure has passed. The STA shall insert the TransactionIdentifier in SA Query Request, and increment this by 1 for each subsequent SA Query Request, and rolling over to 0 after maximum allowed value in this field.  An AP may interpret reception of a valid protected frame as an indication of a successfully completed SA Query. If an MLMESAQUERY. confirm with an outstanding transaction identifier is not received within dot11AssociationSAQueryTimeout period, the AP shall allow the association process to be started without starting additional SA Query procedure.

Insert in IEEE802.11-2007, 8.5.3.2, page213, as step (C) on reception of Message 2

(c) If Management Frame protection is being negotiated, the AP initializes the SA Query TransactionIdentifier to an implementation specific non-negative integer value, valid for the current pairwise security association.