IEEE P802.11
Wireless LANs

| Normative Text Proposal for LB#140 Comment Resolution on Co-Located Interference Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-02-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jing Zhu | Intel | 2111, NE 25th Ave., Hillsboro, OR 97124 | +1 (503) 264 7073 | jing.z.zhu@intel.com |

Abstract

This submission contains the normative text to resolve IEEE 802.11v LB#140 comments with CID 244 and 245.

The text is aligned with P802.11v-D4.0

7.3.2 Information Elements

Add a new raw into Table 7-26 as follows:

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| Collocated Interference Request (see 7.3.2.83) | <ANA> | 4 |  |

Insert the new section 7.3.2.83 into 7.3.2 as follows:

7.3.2.83 Collocated Interference Request element

The Collocated Interference Request element defines information about how the collocated interference report is used. The Co-located Interference Request element is defined in Figure vx1.

|  | Element ID | Length | Diaglog Token | Request Info |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure vx1 – Collocated Interference Request element format

The Element ID field is equal to the Collocated Interference Request value in Table 7-26.

The Length field is set to 2.

The Dialog Token field is set to any nonzero value to identify the request/response transaction.

The Request Info field is shown in Figure vx2.

Move Figure v91 of 7.4.11.13 to here:

|  | Automatic Response Enabled | Report Timeout |
| --- | --- | --- |
| Octets: | 1 | 7 |

Figure vx2 –Request Info field format

Move the last two paragraphs of 7.4.11.13 to here, and make changes as follows:

The Automatic Response Enabled field set to 1 indicates that the requesting STA requests the receiving STA

to send the Collocated Interference Response frames automatically with the Report Period interval, as

defined in 7.4.11.14, or when the STA knows of a change in the collocated interference.

The Report Timeout field contains a value in units of 100TU and indicates the minimum duration between

two consecutive Collocated Interference Response frames from the reporting STA.

7.2.3.5 Association Response frame format

Insert order 20 information fields into Table 7-11:

| Order | Information | Notes |
| --- | --- | --- |
| 20 | CollocatedInterferenceRequest | CollocatedInterferenceRequest element is optionally present if both the sending STA and the receiving STA of the Associate Response frame have set the Collocated Interference Reporting field in the Extended Capability element to 1 |

7.2.3.7 Reassociation Response frame format

Insert order 22 information fields into Table 7-13:

| Order | Information | Notes |
| --- | --- | --- |
| 22 | CollocatedInterferenceRequest | CollocatedInterferenceRequest element is optionally present if both the sending STA and the receiving STA of the Reassociate Response frame have set the sets the Collocated Interference Reporting field in the Extended Capability element to 1 |

7.4.11.13 Collocated Interference Request frame format

Change Fig. v90 of 7.4.11.13 as follows:

|  | Category | Action | Dialog Token | Request Info Collocated Interference Request Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 |

Figure v90 – Collocated Interference Request frame format

Remove the last four paragraphs of 7.4.11.13

Add a new paragraph to the end of 7.4.11.13 as follows:

The Collocated Interference Request element field is defined in 7.3.2.83.

7.4.11.14 Collocated Interference Response

Change the 4th paragraph as follows:

The Dialog Token field is set to the nonzero value received in the Collocated Interference Request frame element to identify the request/response transaction. If the Collocated Interference Response frame is being transmitted due to automatic responses enabled, the Dialog Token field is set to the nonzero value received in the Collocated Interference Request frame which was used to enable automatic responses to identify the request/response transaction.

10.3.6.4 MLME-ASSOCIATE.response

10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response (

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

Content of FT Authentication Information Elements,

SupportedRegulatoryClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

CollocatedInterferenceRequest,

VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSSMaxIdlePeriod | As defined in BSS Max Idle Period element | As defined in BSS Max Idle Period element | Indicates the BSS Max Idle Period parameters of the AP. This parameter is present if the MIB attribute dot11WirelessManagedmentImplemented is set to true, and is not present otherwise |
| TIM BroadcastResponse | As defined in TIM Broadcast Response element | As defined in TIM Broadcast Response element | Specifies the service parameters for TIM Broad-cast. This parameter is optional present if dot11MgmtOptionTIMBroadcastEnabled is set to true and the TIM Broadcast Request element is present in corresponding Assoication Request frame, and is not present otherwise |
| CollocatedInterferenceRequest | As defined in Collocated Interference Request element | As defined in Collocated Interference Request element | This parameter is optionally present if both the sending STA and the receiving STA of the Associate Response frame have set the Collocated Interference bit in the extended Capability field to 1 |

10.3.7.4 MLME-REASSOCIATE.response

10.3.7.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response (

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

Content of FT Authentication Information Elements,

SupportedRegulatoryClasses,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

CollocatedInterferenceRequest,

VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSSMaxIdlePeriod | As defined in BSS Max Idle Period element | As defined in BSS Max Idle Period element | Indicates the BSS Max Idle Period parameters of the AP. This parameter is present if the MIB attribute dot11WirelessManagedmentImplemented is set to true, and is not present otherwise |
| TIM BroadcastResponse | As defined in TIM Broadcast Response element | As defined in TIM Broadcast Response element | Specifies the service parameters for TIM Broad-cast. This parameter is optional present if dot11MgmtOptionTIMBroadcastEnabled is set to true and the TIM Broadcast Request element is present in corresponding Assoication Request frame, and is not present otherwise |
| CollocatedInterferenceRequest | As defined in Collocated Interference Request element | As defined in Collocated Interference Request element | This parameter is optionally present if both the sending STA and the receiving STA of the Reassociate Response frame has set the Collocated Interference bit in the extended Capability field to 1 |

11.20.8 Collocated Interference reporting

Change the 3rd paragraph to the end of 11.20.8 as follows:

A STA that supports collocated interference reporting shall only request collocated interference reporting from a STA within the same infrastructure BSS or the same IBSS whose last received Extended Capabilities element contained a value of 1 for the Collocated Interference bit in the Capabilities field. A STA gathers information from another STA by sending a Collocated Interference Request frame containing a unique Dialog Token to another STA. The STA, if it accepts the request, shall return a Collocated Interference Response frame that includes the Dialog Token that matches the one in the Collocated Interference Request frame element. An AP that has advertised collocated interference reporting capability may include Collocateded Interference Request element in the Association / Reassociation response to a STA that also advertised collocated interference reporting capability.

Change the 6th paragraph to the end of 11.20.8 as follows:

A STA that has not received any Collocated Interference Request frame may send a Collocated Interference Response frame autonomously with the Dialog Token field set to zero. The Dialog Token field is set to the nonzero value received in the Collocated Interference Request frame element which was used to enable automatic responses. If collocated interference is present, the reporting STA shall send Collocated Interference Response frames with interval indicated by the Report Period field or if the level of the interference caused by collocated interferer is significantly changing or if the time characteristics of the interference is significantly changing. If automatic response is enabled by the requesting STA, the reporting STA shall send Collocated Interference Response frames periodically with interval lower-bounded by the Report Period field. Also, if the time characteristics of the interference is significantly changing, the reporting STA shall send Collocated Interference Response frames to the requesting STA. The Report Period timer shall restart whenever the reporting STA sends out a Collocated Interference Response frame. The criteria a reporting STA uses for determining significant changes are internal to the reporting STA and outside the scope of this standard.