IEEE P802.11
Wireless LANs

| Normative Text Proposal for LB#140 Comment Resolution on Co-Located Interference Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jing Zhu | Intel | 2111, NE 25th Ave., Hillsboro, OR 97124 | +1 (503) 264 7073 | jing.z.zhu@intel.com |

Abstract

This submission contains the normative text to resolve IEEE 802.11v LB#140 comments with CID 244, 245.

The text is aligned with P802.11v-D4.0

11.20.8 Collocated Interference reporting

Change the 6th paragraph to the end of 11.20.8 as follows:

A STA that has not received any Collocated Interference Request frame may send a Collocated Interference Response frame autonomously with the Dialog Token field set to zero.  A STA may send the co-located interference request frame to another STA immediately after they are associated, so that the reporting STA can send the co-located interference response frame as soon as it detects performance degradation due to co-located interference. The Dialog Token field is set to the nonzero value received in the Collocated Interference Request frame which was used to enable automatic responses. If collocated interference is present, the reporting STA shall send Collocated Interference Response frames with interval indicated by the Report Period field or if the level of the interference caused by collocated interferer is significantly changing or if the time characteristics of the interference is significantly changing. If automatic response is enabled by the requesting STA, the reporting STA shall send Collocated Interference Response frames periodically with interval greater than or equal to the Report Period field. Also, if the temporal characteristics of the interference or the level of the interference caused by collocated interferer are significantly changing, the reporting STA shall send Collocated Interference Response frames to the requesting STA providing updated information to the requesting STA. The Report Period timer shall restart whenever the reporting STA sends out a Collocated Interference Response frame. The criteria a reporting STA uses for determining significant changes are internal to the reporting STA and outside the scope of this standard.