IEEE P802.11
Wireless LANs

| TGp LB 141 Resolution of Comments for Clauses 10 and 11 Assuming a Timing IE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-02-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x 175 | justin.mcnew@kapsch.net |

Abstract

This submission proposes the resolutions of comments for Clauses 10 and 11, assuming a Timing IE similar to that proposed in 09-0274/r2.

|  | LB141 Comment Resolution |
| --- | --- |

- COMMENT:

| CID | Commenter | Clause | Pg | Ln | Comment | Suggested Remedy | Comment resolution. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 125 | Emeott, Stephen | 10.3.a | 11 | 43 | The Timestamp field is not used or required by any feature in this amendment. Therefore, there is no need to get, set or advance the timer. Delete these three MLME primitives. | Make the suggested change | Counter. The Timestamp field and its relationship to the TSF timer are discussed in 11.6a. The Get primitive is needed, but not primitives for set or advance, which will be deleted. |
| 127 | Kenney, John | 10.3.a.2.4 | 12 | 50 | There is a sentence explaining that if the result is successful the value of the TSF timer is included in the primitive. There is no explanation of whether the parameter is included if the result is failure. | Append "If the result of an MLME-SETTSFTIME.request is failure the TSFtime parameter is not included in the MLME-SETTSFTIME.confirm primitive." Also, in 10.3.a.2.2, in the Description column of the table listing the primitive parameters, add ", if ResultCode is SUCCESS" after "Value of the TSF timer" in the Description column. | Accept. |
| 128 | Emeott, Stephen | 10.3.b | 13 | 1 | The Timestamp field is not used or required by any feature in this amendment. Therefore, there is no need to get, set or advance the timer. Delete these three MLME primitives. | Make the suggested change | Counter. The Timestamp field and its relationship to the TSF timer are discussed in 11.6a. The Get primitive is needed, but not primitives for set or advance, which will be deleted. |
| 130 | Emeott, Stephen | 10.3.c | 14 | 39 | The Timestamp field is not used or required by any feature in this amendment. Therefore, there is no need to get, set or advance the timer. Delete these three MLME primitives. | Make the suggested change | Counter. The Timestamp field and its relationship to the TSF timer are discussed in 11.6a. The Get primitive is needed, but not primitives for set or advance, which will be deleted. |
| 159 | Chu, Liwen | 11.a.1 | 21 | 6 | .11 baseline standard defines several rules for accepting Beacon, Probe, other management frames, and data frames. .11p draft defines how to accept data frames. Inside the context of a BSS, the BSSID is set by the SME, but outside the context of a BSS, it is not specified how to verify the BSSID field in order to accept the "Timing and information" frame and other management frames. How does the STA know whether the "Timing and information frame" are associated with an overlapping BSS or whether it was sent outside the contest of a BSS and is OK to accept? | Define the rules about accepting management frames outside the context of BSS. | Accept in principle. With the changes to clause 7 in 11-09/58, Address 3 will be set to the wildcard BSSID when sent outside the context of a BSS, resolving the ambiguity. |
| 160 | Roy, Richard | 11.a.1 | 21 | 6 | This subclause should describe how the timing information is used in addition to how the timestamp is generated. | "Remove subclasue 11.a.1 and replace it with the following subclause: "" 11.6.2a Use of the Timing and Information management frame for distributing time from an external clock The main purpose of the Timing and Information management frame is to provide a mechanism whereby a STA can send to other STAs sufficient information in a single management frame to allow the receiving STAs to estimate the time being kept by an external clock (other than the TSF timer) on the transmitting STA. This is accomplished using the timestamp parameter in the transmitted frame in conjunction with the Timing Information Element (TIE) which contains the information necessary to adjust the TSF timer of the transmitting STA to match the output of the transmitting STAs external clock. A STA that is transmitting a Timing and Information frame shall set the value of the timestamp parameter to the value of the STA's TSF timer at the time that the data symbol containing the first bit of the timestamp is transmitted to the PHY plus the transmitting STAs delays through its local PHY from the MAC-PHY interface to its interface with the WM. A Timing and Information frame is generated by a STA's MLME in response to receiving an MLME-TIMING_INFO.request from the STA's SME. After generating the frame, the MLME returns an MLME- TIMING_INFO.confirm to the SME.""" | Counter. Remove subclause 11.a.1. But setting the Timestamp is discussed in 11.6a. |
| 161 | Kenney, John | 11.a.1 | 21 | 8 | The Timing and Information frame always includes a timestamp, so the first sentence of 11.a.1 applies whether or not the STA sending this frame is communicating outside the context of a BSS. | Delete "communicating outside the context of a BSS" from the first sentence of 11.a.1 | Counter. Remove subclause 11.a.1. |
| 163 | Emeott, Stephen | 11.a.2 | 21 | 15 | Because the value of the TSF Timer is not used or relied upon by any feature in this amendment, the need to standardize how the TSF Timer is managed goes away. Delete this section. | Make the suggested change | Counter. Remove subclause 11.a.2. But managing the TSF timer is discussed in 11.6a. |
| 164 | Roy, Richard | 11.a.2 | 21 | 15 | This text is not necessary as it is a repeat of earlier clasue 11 material. | Remove this subclause. | Accept. |
| 165 | Zhang, Hongyuan | 11.a.2 | 21 | 15 | "Maintaining TSF seems to be made mandatory here. It is not described of how the TSF will be used in this amendment. Section 10.3.a.2.4 has some mention of an optional use-case to synchronize UTC etc. It is difficult to understand why this feature needs to be mandatory if there is no Layer 2 or below use. I understand that TSF is used in HLIE but again the end-benefit is not clear.2. There is already TSF functionality in base standard. Can't that be used?" | Make support for TSF optional or show a Layer-2 use-case. | Counter. Remove subclause 11.a.2. Use-case is described in 11.6a. |
| 166 | Braskich, Tony | 11.a.2 | 21 | 17 | In 802.11-2007, "TSF" is defined as a function to keep the "timers for all STAs in the same BSS synchronized." This draft defines communication outside the context of a BSS, where "MAC sublayer synchronization is not required." The requirement, then, that each STA "outside the context of a BSS shall maintain a TSF timer..." is illogical. What is the justification for this normative requirement? | Remove normative requirements for maintaining a TSF timer when outside a BSS, since there appears to be no use for the TSF timer in the draft. | Counter. Remove subclause 11.a.2. But the use of the TSF timer for STAs with dot11OCBEnabled set to true is discussed in 11.6a. The TSF function described in the base document is different from the usage of the TSF timer discussed in 11.6a. |

- Background

This submission proposes the resolutions to comments for Clauses 10 and 11, assuming a Timing IE similar to that originally proposed in 08-1165/r5, for which: (1) the MLME-GETTSFTIME.request and MLME-GETTSFTIME.confirm primitives are required, but not those for MLME-SETTSFTIME and MLME-INCTSFTIME; and (2) the use of the TSF timer to set the Timestamp field of the Timing Advertisement frame is discussed in subclause 11.6a of the proposal.

- Recommended Resolution of the Comments:

10.3.a.2 MLME-GETTSFTIME.confirm

10.3.a.2.2 Semantics of the service primitive

Editor to change (deletions, if any, in yellow background; insertions, if any, in green background) row 2, column 4 of the table in 10.3.2.2 as follows (CID 127):

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, FAILURE | Reports the outcome of GETTSFTIME request if ResultCode is SUCCESS. |

10.3.a.2.4 Effect of receipt

Editor to change (deletions, if any, in yellow background; insertions, if any, in green background) the first sentence of 10.3.a.2.4 as follows (CID 127):

The SME is notified of the result of an MLME-GETTSFTIME.request and, if successful, has the value of the TSF timer at the instant the MLME-GETTSFTIME.request was received by the MLME.  If the result of an MLME-GETTSFTIME.request is failure the TSFtime parameter is not included in the MLME-GETTSFTIME.confirm primitive.

Editor to remove 10.3.b.1 (“MLME-SETTSFTIME.request”) and all subclauses (CID 125, 128).

Editor to remove 10.3.b.2 (“MLME-SETTSFTIME.confirm”) and all subclauses (CID 125, 128).

Editor to remove 10.3.b.2 (“MLME-SETTSFTIME.indication”) and all subclauses (CID 125, 128).

Editor to remove 10.3.c.1 (“MLME-INCTSFTIME.request”) and all subclauses (CID 125, 130).

Editor to remove 10.3.c.2 (“MLME-INCTSFTIME.confirm”) and all subclauses (CID 125, 130).

Editor to remove 11.a.1 (“Generation and timestamping of a Timing Advertisement frame”) and all subclauses (CID 160, 161).

Editor to remove 11.a.2 (“TSF”) and all subclauses (CID 163, 164, 165, 166).

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D5.01. noted above and instruct the editor to make these changes to P802.11p D5.01.

- Motion by: ___Justin McNew_________________Date: _________________

- Second:  ______Wayne Fisher________________

| Approve: 12 | Disapprove: 1 | Abstain: 1 |
| --- | --- | --- |

- References: