IEEE P802.11
Wireless LANs

| Timing Information Element Text Alternative |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-02-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Justin McNew | Kapsch TrafficCom ¦ TechnoCom Mobility Solutions | 2035 Corte del Nogal, Suite 105, Carlsbad, CA 92011 | 760-438-5115 x175 | justin.mcnew@kapsch.net |

Abstract

Note: This text alternative is derived from IEEE P802.11p 08-1165r5 submission; Richard Roy, Connexis

This document contains a description of the timing information element which is optionally included in Timing Advertisement management frames to distribute information to receiving STAs that allows receiving STAs to estimate the value of time associated with an external clock in the transmitting STA.  For example, a STA with a GPS module that outputs UTC time can send information to receiving STAs that allow those STAs to estimate UTC time without having to have a GPS device present.

Insert the following new subclause  7.3.2.b at the end of subclause 7.3.2:

7.3.2.b Timing information element

The Timing information element, shown in Figure 7-95a2, contains fields describing the source of time associated with an external clock (external time source), an estimate of the offset between that external clock and the TSF timer, and the standard deviation of the error in the offset estimate. This information may be used by a STA to align its own external clock to that of another STA.

| Element ID | Length | Timing capabilities | TTOE | TTOE-STD | reserved |
| --- | --- | --- | --- | --- | --- |

Octets		1	          	1		        1		     10		      5	          0 - 239

Figure 7-95a2—Timing information element format

- The Timing capabilities field, shown in Figure 7-95a3, is used to indicate a STA’s source of external time and the current condition of its timing estimator.

| bits |  |  |  |  |  |  |  | description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| b0 | b1 | b2 | b3 | b4 | b5 | b6 | b7 |  |
| 0 | 0 | 0 |  |  |  |  |  | No standardized external time source |
| 0 | 0 | 1 |  |  |  |  |  | External time source is UTC [ed: add this reference: ITU-R TF.460-4, and insert the associated pointer here] |
| 010 - 111 |  |  |  |  |  |  |  | Reserved |
|  |  |  | 0 |  |  |  |  | External time source is not currently available (time may continue to be estimated based on this source, e.g. if UTC is being derived from GPS the coverage may be sporadic) |
|  |  |  | 1 |  |  |  |  | External time source is available and is currently being used to estimate time |
|  |  |  |  | 0000 – 1111 |  |  |  | Reserved |

Figure 7-95a3—Timing capabilities field format

- The TSF timer offset estimate (TTOE) contains a two’s complement integer in nanoseconds which, when added to the Timestamp present in the same transmitted frame gives the receiving STA an estimate of the transmitting STA’s external time source.  The Timestamp is derived from the TSF timer as defined in 11.6a.

- The TSF timer standard deviation (TTOE-STD) contains an unsigned integer in nanoseconds that defines the standard deviation of the error in the TTOE.  A STA sets the TTOE-STD field to the maximum value (240-1) to indicate that it has no meaningful error information to provide for the estimate of the external time source.

Insert the following new subclause 11.6a after subclause 11.6 and renumber as appropriate:

11.6a  Time advertisement

According 11.1.2 each STA maintains a TSF Timer. The Timestamp field is included in the Timing Advertisement frame, and when a STA transmits the Timing Advertisement frame the Timestamp shall be set to the value of the STA’s TSF timer at the time that the data symbol containing the first bit of the Timestamp is transmitted to the PHY plus the transmitting STA’s delays through its local PHY from the MAC-PHY interface to its interface with the WM [e.g., antenna, light emitting diode (LED) emission surface].

11.6a.1 Advertising an external time source using the TSF Timer, Timing Advertisement frame and Timing information element (informative)

A STA may advertise an external time source when transmitting the Timing Advertisement frame by including the Timing information element.  As defined in clause 7.3.2.b the Timing information element contains an estimate of the difference between its external time source and the Timestamp included in the frame (using the TTOE field) as well as the standard deviation of the error in that estimate (using the TTOE-STD field).   A STA with an external time source can implement an estimator in a variety of ways, which are beyond the scope of this standard.  The SME provides the Timing information element to the MLME when it requests the MLME to send a Timing Advertisement frame.  When a Timing Advertisement frame is received by a STA its MLME reports the timestamp and Timing information element to the SME.

In order to support this Timing Advertisement frame and Timing information element, the MLME-GETTSFTIME.request and MLME-GETTSFTIME.confirm primitives were added to clause 10.  (In order to accurately estimate the TTOE the MLME must be able to provide the current value of its TSF timer to the SME in a known or reasonably bounded time between the instant the SME requests the current value of the TSF timer and the instant the MLME reports it.)  In order for the SME to request the transmission of a Timing Advertisement frame the MLME-TIMING_ADVERTISEMENT.request primitive was also added to clause 10 and includes the Timing information element.  Finally, the  MLME-TIMING_ADVERTISEMENT.indication primitive was added to clause 10 for reception of the frame and includes both the timestamp and Timing information element.

Motion: Move to accept these changes to P802.11p D5.01 and instruct the editor to update the latest 11p draft to include these changes.

Moved:

Seconded:

YES:

NO:

ABSTAIN: