IEEE P802.11
Wireless LANs

| Date and Time zone configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 525 3693 | gabor.bajko@nokia.com |

Abstract

This proposal updates section 7.3.4.9 of TGu version D5.0 to address CIDs# 5070, 5071, 5116, 5117, 5118, 5170, 5171, 5172, 5173, 5174, 5303, 5367.

This proposal is based on draft TGu version D5.0

Insert the following text into 7.3.2.63.1 under Table v6:

When the MSB of the milliseconds field is set to 1, it indicates the date and time locally at the AP. When that bit is set to 0, the date and time is indicated in UTC time.

When all 15 LSB of the milliseconds field is set to one, then the Milliseconds sub-field of the Timestamp field shall be ignored by the non-AP STA (the time is only being reported to the nearest second).

- Replace figure 7-95o132 with the following figure:

| Info ID | Length | Timestamp | Time Zone |
| --- | --- | --- | --- |

Octets:	2	     2		     9	               variable

7.3.4.9 AP Date, Time and Time Zone Information

Make the following changes to paragraph 7.3.4.9:

The Info ID field is equal to the value in Table 7-43bz corresponding to the AP Date, Time and Time Zone Information.

The Length is a 2-octet field whose value is equal to 9 plus the number of octets in the Time Zone field. The minimum value of the Length field is 14-octet.

- The Status Code is a 2-octet field whose value is drawn from Table 7-23.

- The format of the Date field is as described in Figure 7-95o133, and represents the correct date at the APs location.

| - Year | - Month | - Day |
| --- | --- | --- |

- Octets:		1				1					2

- Figure 7-95o133—Date field format

The Year sub-field is a 2-octet field whose value is 0 to 65535., encoded in little endian format.

The Month sub-field is a one octet field whose value is 1 to 12.

The Day sub-field is a one octet field whose value is 1 to 31.

Example: November 13, 1984 is encoded as 0x0d 0x0b 0xc0 0x07..

- The format of the Time field as described in Figure 7-95o134 and represents the time known by the AP at the time of the query.

| - Hours | - Minutes | - Seconds |
| --- | --- | --- |

- Octets: 		1		1		1

Figure 7-95o134—Time field format

The Hours sub-field is a one octet field whose value is 1 to 23.

The Minutes sub-field is a one octet field whose value is 0 to 59.

The Seconds sub-field is a one octet field whose value is 0 to 59.

Example: 11:45:37 (11:45:37am) is encoded as 0x0b 0x2d 0x25, 23:45:37 (11:45:37pm) is encoded as 0x17 0x2d 0x25

The format of the Timestamp field is described in table v6, section 7.3.2.63.1.

The format of the Time Zone field is as specified in section 8.3 of IEEE Std 1003.1-2004:

stdoffset[dst[offset][,start[/time],end[/time]]]

The length of the field is no less than 4-octet and no more than TZNAME_MAX, as defined in IEEE Std 1003.1-2004. The Time Zone field represents the correct time zone at the APs location. The encoding of the field is in ASCII character as shown in the following Example-1.

Example-1: EST5

Example-2: EST5EDT4,M3.2.0/02:00,M11.1.0/02:00

In the Example-2 above, the string is interpreted as a time zone that is normally five hours behind UTC, and four hours behind UTC during DST, which runs from the second Sunday in March at 02:00 local time through the first Sunday in November at 02:00 local time. Normally, the time zone is abbreviated “EST” but during DST it is abbreviated “EDT”.

See Annex V.1.5 for informative text on usage of fields contained within this information element.