IEEE P802.11
Wireless LANs

| Dropped Octet Count update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 525 3693 | gabor.bajko@nokia.com |

Abstract

This proposal addresses CID#5248.

This proposal is based on draft TGu version D5.0

- Make the following changes to the text in section 6.2.1.1.4:

6.2.1.1.4 Effect of receipt

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data and subtypes Data, Data+CF-ACK, DATA+CF-Poll, and Data+CF-Ack+CF-Poll, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBestEffortRate in the dot11InterworkingEntry identified by the source MAC address of the received frame. The specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, the following operations shall be performed by the AP’s MAC sublayer:

—dot11NonAPStationBestEffortMSDUCount shall be incremented by 1.

—dot11NonAPStationBestEffortOctetCount shall be incremented by the number of octets in the MSDU.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data and subtypes QoS Data + CF-Ack + CF-Poll and the priority is an integer in the range of 0 to 7, inclusive, then the AP’s MAC sublayer shall derive the access category from the priority using the mapping in Table 9-1. The AP’s MAC sublayer shall retrieve the MIB variables listed below from the dot11InterworkingEntry identified by the source MAC address of the received frame and perform the following operations:

—If the access category is AC_VO, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthVoiceRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate limiting mechanism does not discard the frame, then dot11NonAPStationVoiceMSDUCount shall be incremented by 1 and dot11NonAPStationVoiceOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedVoiceMSDUCount shall be incremented by 1 and dot11NonAPStationDroppedVoiceOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedVoiceMSDUCount shall be incremented by 1.

—If the access category is AC_VI, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthVideoRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationVideoMSDUCount shall be incremented by 1 and dot11NonAPStationVideoOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedVideoMSDUCount shall be incremented by 1 and dot11NonAPStationDroppedVideoOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedVideoMSDUCount shall be incremented by 1.

—If the access category is AC_BE, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBestEffortRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationBestEffortMSDUCount shall be incremented by 1 and dot11NonAPStationBestEffortOctetCount shall be incremented by the number of octets in the

MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedBestEffortMSDUCount shall be incremented by 1 and dot11NonAPStationDroppedBestEffortOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedBesteffortMSDUCount shall be incremented by 1.

—If the access category is AC_BK, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthBackgroundRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationBackgroundMSDUCount shall be incremented by 1 and dot11NonAPStationBackgroundOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedBackgroundMSDUCount shall be incremented by 1 and dot11NonAPStationDroppedBackgroundOctetCount shall be incremented by the number of octets in the MSDU. If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedHCCAMSDUCount shall be incremented by 1.

At an AP for which dot11SSPNInterfaceEnabled is true, upon receipt of an individually addressed frame of type data whose priority is an integer in the range of 8 to 15, inclusive, then the AP’s MAC sublayer shall perform rate limiting to enforce the resource utilization limit in dot11NonAPStationMaxAuthHccaRate; the specific mechanism to perform rate limiting is outside the scope of this specification. If the rate-limiting mechanism does not discard the frame, then dot11NonAPStationHCCAMSDUCount shall be incremented by 1, and dot11NonAPStationHCCAOctetCount shall be incremented by the number of octets in the MSDU.If the rate limiting mechanism does discard the frame, then dot11NonAPStationDroppedHCCAMMSDUCount shall be incremented by 1 and dot11NonAPStationDroppedHCCAOctetCount shall be incremented by the number of octets in the MSDU.