IEEE P802.11
Wireless LANs

| SAE Vector |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA, USA | +1 408 227 4500 | dharkins@arubanetworks.com |

Abstract

This document provides a hexadecimal representation of a single run of the SAE authentication protocol from the perspective on one side. It addresses CID 956.

Instruct the Editor to append the following to Appendix H.

H.10 SAE Test Vector

group 19

Password: 'thisisreallysecret'

Peers' MAC addresses: 00:0b:6b:d9:02:46 and 00:09:5b:66:ec:1e

-----------------------------------------------------------------------

H() of password 'thisisreallysecret' and counter 1:

d3b02e34 46609a19 aea55340 e69b8eb8 22d1d5bd 87cfffc0 8bb66dcf c5a2624b

candidate x value:

bad1c03a e442064e c9940b07 a30b781f d24ef3c8 4573db4b 1b5881a6 591a88bd

produces secret point P (x,y):

bad1c03a e442064e c9940b07 a30b781f d24ef3c8 4573db4b 1b5881a6 591a88bd

c8be8981 ce3fca26 5629b067 2784d8ad c8c86324 a905d0c3 2091526d 0ecc1e1d

H() of 00:0b:6b:d9:02:46 followed by 00:09:5b:66:ec:1e

bf9a9852 2502d0d8 88d43de1 31a06509 f7412d12 91a23f26 89590af1 9c080a03

PWE (x,y):

d3a7752d 41832cb3 78898d0f 52833215 1bbd8fb0 a1daa004 466c67bf ee591fa1

9e416870 57395a14 621ca0d5 4d4d57c4 46a5ea58 a072c3aa ee16aa79 a4f978ca

local private value:

d929e58f 2228c86b e41254d7 8c914272 f3aeac34 1b4786e5 e9f980f5 0262e06b

local mask value:

3d61717c f0cf64bd a677d1d2 d2d22f55 686a29cb ffa9b43c bef71967 66c54ff6

local commit:

168b570d 12f82d28 8a8a26aa 5f6371c8 9f31db52 73d99c9d b536cf99 6cc50b10

bf455487 a4b549a5 cf6f16cb 3e46c9ba f0221d93 27a4a2c7 8b1c14a0 6ddd5b8f

4ee800b4 017ef3d8 a6530f55 6cb3d430 e7fd3f3b b50a2f45 2cbad684 192b588b

peer's commit:

3752fb8a 341d9215 1d88d3e0 f863af07 1b485062 42f5b71e 57b935b1 6dc68b57

c9b73c2f 446280e1 90d8acf7 ffebb38f c9d5adc9 a932f39f 92425bfd fb74fcee

abec2893 325b68a9 9cab38c5 15bcb634 5137b20a ed546fa8 9d70679c c2bef983

k:

f5faaed4 1bfbf69b 0fd37c2b 8c2965f9 a875baec 79cb34c4 62a1ce6a 6ef2175c

local confirm:

996bf696 dfaccb61 0d996dc7 48c34e6e dc9c604b 5deb8277 1c639769 d696178e

peer's confirm:

0824d2c8 63f67c90 86df7038 cd906859 cad805b3 da18fd05 0ba1c80b 1401e572

PMK:

629a47b9 e84c16fa 99816af2 01882e8f 8b4c2955 34b3a77b 9f07e7e3 afab062a

References:

“authsae” project on www.sourceforge.com