IEEE P802.11
Wireless LANs

| TGad Evaluation Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

TGad evaluation methodology

# Contributors

(This will grow to reflect those providing explicit contributions / review comments of this document. Please let Eldad know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |
| Minyoung Park | Intel Corporation |  |  | minyoung.park@intel.com |
| Robert Stacey | Intel Corporation |  |  | robert.j.stacey@intel.com |
| Hongyuan Zhang | Marvell |  |  | hongyuan@marvell.com |
| James Yee | MediaTek |  |  | james.yee@mediatek.com |
| Vish Ponnampalam | MediaTek |  |  | vish.ponnampalam@mediatek.com |
| Vinko Erceg | Broadcom |  |  | verceg@broadcom.com |
| Andre Bourdoux | IMEC |  |  | bourdoux@imec.be |
| Carlos Cordeiro | Intel Corporation |  |  | carlos.cordeiro@intel.com |
| Roman Maslennikov | University of Nizhny Novgorod |  |  | roman.maslennikov@wcc.unn.ru |
| Sai Shankar N | Broadcom Corp |  |  | nsai@broadcom.com |
| Alexander Maltsev | Intel Corporation |  |  | alexander.maltsev@intel.com |
| Artyom Lomayev | Intel Corporation |  |  | artyom.lomayev@intel.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | EP initial contribution | 09 March 2009 |
| R1 | Added a few more hardware impairments | 11 March 2009 |
| R2 | Removed ADC/DAC hardware impairments | 12 March 2009 |
| R3 | Additional contributors, added uncompressed video requirement, added more configuration detail | 29 April 2009 |
| R4 | In conference room environment, added logical link between two laptops | 08 May 2009 |
| R5 | Two editorial modifications | 11 May 2009 |
| R6 | Added reference | 12 May 2009 |
| R7 | Added conference room detail; added lightly compressed video model as per 09/709 | 07 July 2009 |
| R8 | Changed | 16 July 2009 |
| R9 | Modified Local file transfer and added web browsing model as per 09/1216 ; added hard disk file transfer as per 09/1222; added PHY impairments as per 09/1213 | 18 Nov 2009 |
| R10 | added SNR vs PER methodology as per 09/1229; updated home living room scenario parameters; update to PA model; update to local file transfer and web browsing model | 18 Nov 2009 |

# Introduction

The evaluation methodology defines conditions for functional requirements compliance, PHY performance, and a limited set of simulation scenarios and comparison criteria for evaluating proposals.

# Conditions for Functional Requirement Compliance

## Point-to-point link simulation

Synthetic test case to demonstrate compliance with requirements in [functional requirements Section 2.1].

- Two stations

  - STA 1 is source

  - STA 2 is sink

- traffic from STA1 to STA2

  - protocol: UDP

  - offered load: infinite

  - MSDU size: TBD

- PHY channel impulse response and pathloss model

  - TBD

- Meet requirements in [functional requirements Sections 2.1.1 and 2.1.2]

## Link budget parameters for FR Section 2.1.2 (range requirement – Req03)

| Parameters | Units | Value | Notes |
| --- | --- | --- | --- |
| Max Tx Power | dBm | 10 |  |
| Noise Figure | dB | 10 |  |
| Max Tx antenna gain | dBi | 14 |  |
| Max Rx antenna gain | dBi | 14 |  |
| Min Pathloss | dB | 98 | 10 meters LOS + 10 dB additional pathloss for NLOS |

# PHY Performance

The criterion for comparison of PHY characteristics are PER vs. SNR curves for different operation modes and different modulation and coding schemes of the system.

## PHY Channel Impulse Response

In order to calculate PER vs. SNR curves, decoupling of the channel impulse response and path loss characteristics of the general channel model is required as follows:

  - channel impulse response (CIR) is to be normalized on an instantaneous basis (packet-by-packet). Instantaneous normalization of the CIRs is performed after application of beamforming

  - standardized antennas

- Isotropic radiator (as defined in [3])

- Basic steerable antenna model with a directional antenna pattern defined as a Gaussian function with 300 mainlobe beamwidth and -20 dB backlobe, refer to [3]

  - To steer directional antennas, algorithm adjusting TX and RX antenna patterns towards the most power channel ray is suggested as the standard beamforming algorithm. (The description of the algorithm is provided in [3]).

  - Antenna TX/RX combinations

  - Omni TX to omni RX

  - Omni TX to directional RX

  - Directional TX to directional RX

## Hardware impairments

- phase noise:

- model

- parameters

- PSD(0) =  -80 dBc/Hz

- Pole frequency fp1 = 0.75 MHz

- Pole frequency fp2 = 2 MHz

- Zero frequency fz =  100 MHz

- r1 = 3.7; r2 = 1.65; r3 = 2.05

- PSD(infinity) = -150 dBc/Hz

- PA non-linearity model:

- Rapp AM-AM

- Modified Rapp AM-PM

- GaAs PA model parameters

  - AM-AM

    - g = 19

    - Asat = 1.4

    - s = 0.81

  - AM-PM

    -  = - 48000

    -  = 0.123

    - q1 = 3.8

    - q2 = 3.7

- CMOS PA model parameters

  - AM-AM

    - g = 5

    - Asat = 0.6

    - s = 0.71

  - AM-PM

    -  = 2560

    -  = 0.114

    - q1 = 2.4

    - q2 = 2.3

- Calculate backoff as the output power backoff from full saturation:

    - PA Backoff = 10 log10(Average TX Power/Psat)

    - Disclose: (a) EIRP and how it was calculated, (b) PA Backoff

    - Note: a PA Backoff equal to 8 dB for OFDM and 0.5 dB for single carrier is recommended.

- carrier frequency offset and symbol clock:

- fixed carrier frequency offset of  –13.675 ppm at the receiver, relative to the transmitter

- The symbol clock shall have the same relative offset as the carrier frequency offset

- Transmitter I/Q imbalance:

- Imbalance model

- distortion coefficients

## Comparison Criteria

    - PER vs. SNR curves

- all MCS’s

- channel impulse responses

    - AWGN

    - home living room

    - office conference room

    - enterprise cubicle

- antenna combinations for each channel model:

    - Omni TX to omni RX; LOS

    - Omni TX to directional RX; NLOS

    - Directional TX to directional RX; NLOS

- simulations must include:

    - the following hardware impairments:

    - one of the PA non-linearity models (Note: CMOS recommended)

    - phase noise

    - carrier frequency offset and symbol clock

    - timing acquisition on a per-packet basis

    - preamble detection on a per-packet basis

- System Evaluation

## Traffic Models

- Uncompressed video

  - Parameters

    - Constant Bit Rate (CBR)

    - 3 Gbps (1080p, (RGB): 1920x1080 pixels, 24bits/pixels, 60frames/s)

  - Requirements given in [functional requirements Section 2.1.3]

- lightly compressed video

  - Requirements

    - PLR: 1e-8

    - Delay: 10 ms

  - Parameters

    - Slice inter-arrival time (IAT) = 1/4080 seconds

    - µ = 15.798 Kbytes

    - σ = 1.350 Kbytes

    - b = 515 Mbps

  - Algorithm for each video source – Input: target bit rate in Mbps (p); Output: slice size in Kbytes (L)

    - At each IAT, generate a slice size L with the following distribution: Normal(µ*(p/b), σ*(p/b))

      - If L > 92.160 Kbytes, set L = 92.160 Kbytes

- Local file transfer

  - protocol: TCP

  - offered load: infinite

  - MSDU size: 64 bytes for TCP connection establishment (3 way handshake) and 1500 bytes

  - Hurst Parameter: 0.875

  - The inter arrival times of TCP request are modelled by Poission process with mean 10 connections per minute. The traffic generation of data once the connection is established is given below.

    - Simulate or generate ten ON/OFF sources. The ON and OFF times are distributed according to heavy tailed Pareto distribution, with α 1 and α 2 as their ON and OFF times respectively. The Hurst parameter H is related to ON and OFF times by the following formula: H = (3 - min (α1, α2))/2

- Web browsing

  - Protocol: HTTP over TCP

  - Offered load: infinite

  - MSDU size: 64 and 1500 bytes

  - Hurst Parameter: 0.75

  - The inter arrival times of TCP request are modelled by Poission process with mean 10 connections per minute. The traffic generation of data once the connection is established is given below.

    - Simulate or generate ten ON/OFF sources. The ON and OFF times are distributed according to heavy tailed Pareto distribution, with α 1 and α 2 as their ON and OFF times respectively. The Hurst parameter H is related to ON and OFF times by the following formula: H = (3 - min (α1, α2))/2

- Hard disk file transfer

  - Transaction Model

    - A transaction consists of a READ request from host to drive for a specific block of data

    - Followed by the data transfer from drive to host

  - Algorithm

    - Compute corresponding sequence of transaction data sizes

  - Parameters

    - READ request is a short (256B) packet sent from host to drive

    - fixed 1ms delay between receipt of READ request and data offered

    - Compute sequence of inter-arrival times of transaction requests with following discrete random variable distribution

    - Compute corresponding sequence of transaction data sizes with following discrete random variable distribution

## PHY Model

A PHY model to be used for system evaluation should include a PHY abstraction mechanism and path loss models.

PHY vs. SNR curves obtained for PHY performance evaluation as described in Section 3.1 may be used for the PHY abstraction. Alternatively, a different PHY abstraction mechanism could be used, a description for which should be provided.

Path loss models developed in [3, Section 7] should be used for system evaluation.

As explained in Section 3.1, a PHY model (PER vs SNR curves and path loss models) may be derived if parameters of the antenna and beamforming algorithm are fixed. Section 3.1 defines the standard set of antenna and beamforming parameters.  However, TGad proposals may also include system evaluation results based on their proposed antenna and beamforming algorithm.

## Simulation Scenarios

### Home living room

Set top box transmitting uncompressed video, and TV receiving uncompressed video

- configuration

- TBD (minimum number of channel realizations;)

- 7m x 7m x 3m

    - STB 4 meter separated from TV

    - NLOS

    - TV & STB at height of 1.5m

    - TV on front wall

    - Couch, arm chair

- traffic type

- Uncompressed video

- PHY channel impulse and pathloss model

- Home living room

- NLOS

### Office conference room

The office conference room floor plan is shown in Figure 1.

Figure 1. Office conference room floor plan

Mix of uses:  Laptop transmitting lightly compressed video to projector.  Multiple laptops connected to AP.  Laptop connected to device performing sync-and-go file transfer.  Laptops connected to other laptops performing local file transfer.  Links between devices are logical, e.g. STA 3 and STA 5 are performing local file transfer between each other but the physical link could be direct or through the AP.

- configuration

- Room dimensions (length, width, height) in meters is 3.0 x 4.5 x 3

- Devices (coordinates of devices are calculated using coordinate axes shown in Figure 1)

    - AP: location (x = 1.50 m, y = 0.50 m, z = 2.90 m – in ceiling)

    - STA 1:

      - Projector

      - location: ( x = 1.75 m,  y =2.30 m, z fixed at 1 m)

      - Traffic type: receiving lightly compressed video from STA 2 (LOS link)

    - STA 2:

      - Laptop

      - location:  (x = 1.90 m, y = 1.50 m,  z fixed at 1m)

      - Traffic type:

        - transmitting lightly compressed video to STA 1 (LOS link) with target bit rate (p) equal to TBD Mbps

        - Local file transfer from AP

    - STA 3:

      - Laptop

      - location:  (x = 1.35 m, y = 3.00 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to/from STA 5  (NLOS link)

        - web browsing

    - STA 4:

      - Laptop

      - location:  (x = 1.30 m, y = 2.40 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to AP

        - web browsing

    - STA 5:

      - Laptop

      - location:  (x = 1.25 m,y = 1.40 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to/from STA 3 (NLOS link)

        - web browsing

    - STA 6:

      - Laptop

      - location:  (x = 1.55 m, y = 1.20 m, z fixed at 1m)

      - Traffic type:  web browsing

    - STA 7:

      - Laptop

      - location:  (x = 1.85 ,y = 3.10, z fixed at 1m)

      - Traffic type:

        - local file transfer to STA 8 (LOS link)

        - Local file transfer from AP

    - STA 8:

      - mobile device

      - location: (x = 1.60, y = 3.25, , z fixed at 1m)

      - Traffic type: Local file transfer from STA 7 (LOS link)

- PHY channel impulse and pathloss model

- office conference room

- All links to the AP are LOS links that may be blocked by people. Model of the human blockage is TBD. Type of links (LOS or NLOS) between two STAs is specified above.

- TBD definition of interference depending on topology

### Enterprise cubicle

Mix of uses:  Laptop transmitting lightly compressed video to monitor.  Laptop connected to AP.  Laptop connected to hard drive.

- configuration

  - cubicle layout

    - single cubicle (length, width) in meters: 2.5m x 1.8 m

    - 8 cubicles in 4 rows, 2 columns

    - ceiling height 3m

    - TBD ( Randomly or fixed) populate TBD (3) cubes using same frequency channel

  - Floor dimension: 25m x 25m

  - AP: TBD (x,y, z fixed in ceiling) location (AP located in the ceiling in the middle of the group of cubicles)

  - Devices in each of the populated cubes

    - STA 1:

      - monitor

      - location: (x,y) random within cube, z fixed at 1m

      - Traffic type: receiving lightly compressed video from STA 2

    - STA 2:

      - Laptop

      - location: (x,y) random within cube, z fixed at 1m

      - Traffic type:

        - transmitting lightly compressed video to STA 1 with target bit rate (p) equal to TBD Mbps

        - Local file transfer to/from AP

        - Local file transfer to/from STA 3

        - web browsing to/from AP

    - STA 3:

      - hard drive

      - location: (x,y) random, z random between 0m and 1.5m

      - Traffic type: Hard disk file transfer to/from STA 2

- PHY channel and pathloss model

- Enterprise cubicle

- TBD mix of LOS and NLOS in cubicle, NLOS between cubicles, TBD mix of LOS/NLOS to AP

- TBD definition of interference depending on topology

## Comparison Criteria

- goodput (aggregate and per flow)

  - average

- delay (per flow)

  - average

  - # of packets that exceed delay requirement

- packet loss rate (per flow)

- Provide description of PHY abstraction & antenna model

- Provide description of scheduling algorithm

# References

- 11-08-0806-07-0vht-60-ghz-par-nescom-form-plus-5Cs.doc

- 11-07-2988-04-0000-liaison-from-wi-fi-alliance-to-802-11-regarding-wfa-vht-study-group-consolidation-of-usage-models.ppt

- TBD channel impulse model and pathloss model document

- 11-09-0583-00-00ad-tgad-usage-model.ppt

- 11-09-0709-02-00ad-lightly-compressed-video-traffic-modeling.ppt

- 11-09-1216-01-00ad-internet-traffic-modeling.ppt

- 11-09-1213-00-00ad-60ghz-impairments-modeling.ppt

- 11-09-1222-01-00ad-harddrive-traffic-model.ppt

- 11-09-1229-00-Application-of-60-GHz-Channel-Models-for Comparison-of-TGad-Proposals.ppt