IEEE P802.11
Wireless LANs

| TGad Evaluation Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

TGad evaluation methodology

# Contributors

(This will grow to reflect those providing explicit contributions / review comments of this document. Please let Eldad know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |
| Minyoung Park | Intel Corporation |  |  | minyoung.park@intel.com |
| Robert Stacey | Intel Corporation |  |  | robert.j.stacey@intel.com |
| Hongyuan Zhang | Marvell |  |  | hongyuan@marvell.com |
| James Yee | MediaTek |  |  | james.yee@mediatek.com |
| Vish Ponnampalam | MediaTek |  |  | vish.ponnampalam@mediatek.com |
| Vinko Erceg | Broadcom |  |  | verceg@broadcom.com |
| Andre Bourdoux | IMEC |  |  | bourdoux@imec.be |
| Carlos Cordeiro | Intel Corporation |  |  | carlos.cordeiro@intel.com |
| Roman Maslennikov | University of Nizhny Novgorod |  |  | roman.maslennikov@wcc.unn.ru |
| Sai Shankar N | Broadcom Corp |  |  | nsai@broadcom.com |
| Alexander Maltsev | Intel Corporation |  |  | alexander.maltsev@intel.com |
| Artyom Lomayev | Intel Corporation |  |  | artyom.lomayev@intel.com |
| Chang-Soon Choi | IHP Microelectronics |  |  | choi@ihp-microelectronics.com |
| Avinash Jain | Qualcomm |  |  | avinashj@qualcomm.com |
| M. Hossein Taghavi | Qualcomm |  |  | mtaghavi@qualcomm.com |
| Hemanth Sampath | Qualcomm |  |  | hsampath@qualcomm.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | EP initial contribution | 09 March 2009 |
| R1 | Added a few more hardware impairments | 11 March 2009 |
| R2 | Removed ADC/DAC hardware impairments | 12 March 2009 |
| R3 | Additional contributors, added uncompressed video requirement, added more configuration detail | 29 April 2009 |
| R4 | In conference room environment, added logical link between two laptops | 08 May 2009 |
| R5 | Two editorial modifications | 11 May 2009 |
| R6 | Added reference | 12 May 2009 |
| R7 | Added conference room detail; added lightly compressed video model as per 09/709 | 07 July 2009 |
| R8 | Changed | 16 July 2009 |
| R9 | Modified Local file transfer and added web browsing model as per 09/1216 ; added hard disk file transfer as per 09/1222; added PHY impairments as per 09/1213 | 18 Nov 2009 |
| R10 | added SNR vs PER methodology as per 09/1229; updated home living room scenario parameters; update to PA model; update to local file transfer and web browsing model | 18 Nov 2009 |
| R11 | Modified phase noise model to single pole | 19 Nov 2009 |
| R12 | Modified CMOS PA model | 19 November 2009 |
| R13 | Deleted GaAs PA model; clarified phase noise and I/Q; modified local file transfer and web browsing model; added home living room environment figure and configuration; updated TBDs in 2.1; fixed cross references to Channel Model and Functional Requirements doc; updated lightly compressed video traffic rate TBD in 4.3.2 and 4.3.3 | 18 January 2010 |
| R14 | Added interference model per 10/0067r0; modified office conference room per 10/0046r0 | 19 January 2010 |
| R15 | Added coexistence language per 10/0149r0 | 20 January 2010 |

# Introduction

The evaluation methodology defines conditions for functional requirements compliance, PHY performance, and a limited set of simulation scenarios and comparison criteria for evaluating proposals.

# Conditions for Functional Requirement Compliance

## Point-to-point link simulation

Synthetic test case to demonstrate compliance with requirements in [11, functional requirements Section 2.1].

- Two stations

  - STA 1 is source

  - STA 2 is sink

- traffic from STA1 to STA2

  - protocol: UDP

  - offered load: infinite

  - MSDU size: 8 Kbytes

- PHY channel impulse response and pathloss model

  - home living room

- Meet requirements in [11, functional requirements Sections 2.1.1 and 2.1.2]

## Link budget parameters for FR Section 2.1.2 (range requirement – Req03)

| Parameters | Units | Value | Notes |
| --- | --- | --- | --- |
| Max Tx Power | dBm | 10 |  |
| Noise Figure | dB | 10 |  |
| Max Tx antenna gain | dBi | 14 |  |
| Max Rx antenna gain | dBi | 14 |  |
| Min Pathloss | dB | 98 | 10 meters LOS + 10 dB additional pathloss for NLOS |

## Coexistence for FR Section 2.3 (Req08)

Proposers shall describe mechanisms by which coexistence of an 802.11ad AP and other 60 GHz systems and standards in the band is covered by the proposal. This should include coexistence with IEEE 802.15.3c. Those mechanisms may include energy detection, preamble detection or the use of some universal mode to communicate between the 802.11ad AP and the devices of the other 60GHz systems and standards.  Once an interferer is detected, the 802.11ad AP should avoid interference through the use of beamforming, use of alternate available channel or other means to coexist. If the 802.11ad AP and the devices of the other 60GHz systems and standards operate co-channel, the performance of the coexistence mechanism can be measured by the effective throughput of the device in the presence of the interferer operating within the band.

# PHY Performance

The criterion for comparison of PHY characteristics are PER vs. SNR curves for different operation modes and different modulation and coding schemes of the system.

## PHY Channel Impulse Response

In order to calculate PER vs. SNR curves, decoupling of the channel impulse response and path loss characteristics of the general channel model is required as follows:

  - channel impulse response (CIR) is to be normalized on an instantaneous basis (packet-by-packet). Instantaneous normalization of the CIRs is performed after application of beamforming

  - standardized antennas

- Isotropic radiator (as defined in [3])

- Basic steerable antenna model with a directional antenna pattern defined as a Gaussian function with 300 mainlobe beamwidth and -20 dB backlobe, refer to [3]

  - To steer directional antennas, algorithm adjusting TX and RX antenna patterns towards the most power channel ray is suggested as the standard beamforming algorithm. (The description of the algorithm is provided in [3]).

  - Antenna TX/RX combinations

  - Omni TX to omni RX

  - Omni TX to directional RX

  - Directional TX to directional RX

## Hardware impairments

- phase noise:

- model

- parameters

- PSD(0) =  -90 dBc/Hz

- Pole frequency fp = 1 MHz

- Zero frequency fz =  100 MHz

- Corresponding PSD(infinity) = -130 dBc/Hz

- impairment is modeled at both transmitter and receiver

- PA non-linearity model:

- Rapp AM-AM

- Modified Rapp AM-PM

in degrees

- CMOS PA model parameters

  - AM-AM

    - g = 4.65

    - Asat = 0.58

    - s = 0.81

  - AM-PM

    -  = 2560

    -  = 0.114

    - q1 = 2.4

    - q2 = 2.3

- Calculate backoff as the output power backoff from full saturation:

    - PA Backoff = 10 log10(Average TX Power/Psat)

    - Disclose: (a) EIRP and how it was calculated, (b) PA Backoff

    - Note: a PA Backoff equal to 8 dB for OFDM and 0.5 dB for single carrier is recommended.

- carrier frequency offset and symbol clock:

- fixed carrier frequency offset of  –13.675 ppm at the receiver, relative to the transmitter

- The symbol clock shall have the same relative offset as the carrier frequency offset

- Transmitter and receiver I/Q imbalance:

- Imbalance model

- distortion coefficients

## Comparison Criteria

    - PER vs. SNR curves

- all MCS’s

- channel impulse responses

    - AWGN

    - home living room

    - office conference room

    - enterprise cubicle

- antenna combinations for each channel model:

    - Omni TX to omni RX; LOS

    - Omni TX to directional RX; NLOS

    - Directional TX to directional RX; NLOS

- simulations must include:

    - the following hardware impairments:

    - PA non-linearity

    - phase noise

    - carrier frequency offset and symbol clock

    - timing acquisition on a per-packet basis

    - preamble detection on a per-packet basis

# System Evaluation

## Traffic Models

- Uncompressed video

  - Parameters

    - Constant Bit Rate (CBR)

    - 3 Gbps (1080p, (RGB): 1920x1080 pixels, 24bits/pixels, 60frames/s)

  - Requirements given in [functional requirements Section 2.1.3]

- lightly compressed video

  - Requirements

    - PLR: 1e-8

    - Delay: 10 ms

  - Parameters

    - Slice inter-arrival time (IAT) = 1/4080 seconds

    - µ = 15.798 Kbytes

    - σ = 1.350 Kbytes

    - b = 515 Mbps

  - Algorithm for each video source – Input: target bit rate in Mbps (p); Output: slice size in Kbytes (L)

    - At each IAT, generate a slice size L with the following distribution: Normal(µ*(p/b), σ*(p/b))

      - If L > 92.160 Kbytes, set L = 92.160 Kbytes

- Local file transfer

  - protocol: TCP (Reno)

  - offered load: infinite

  - MSDU sizes: 64 bytes for TCP connection establishment (3-way handshake) and 1500 bytes for payload data.

  - Algorithm: at the start of simulation, generate a TCP connection establishment with the following TCP parameter configuration (as appropriate for the simulation platform):

| TCP Model Parameters |  |
| --- | --- |
| MSS | Ethernet (1500) |
| Receive Buffer (bytes) | 65535 |
| Receive Buffer Adjustment | None |
| Delayed ACK Mechanism | Segment/Clock based |
| Maximum ACK Delay (sec) | 0.05 |
| Slow-Start Initial Count (MSS) | 1 |
| Fast Retransmit | Enabled |
| Duplicate ACK Threshold | 3 |
| Fast Recovery | Reno |
| Window Scaling | Enabled |
| Selective ACK (SACK) | Disabled |
| ECN Capability | Disabled |
| Segment Send Threshold | Byte Boundary |
| Active Connection Threshold | Unlimited |
| Karn's Algorithm | Enabled |
| Nagle Algorithm | Disabled |
| Initial Sequence Number | Auto Complete |
| Initial RTO (sec) | 3.0 |
| Min RTO (sec) | 1.0 |
| Max RTO (sec) | 64.0 |
| RTT Gain | 0.125 |
| Deviation gain | 0.25 |
| RTT Deviation Coefficient | 4.0 |
| Timer Granularity | 0.5 |

- Web browsing

  - Protocol: HTTP (version 1.0 or above)

  - MSDU sizes: 350 bytes for HTTP requests and 1500 bytes for payload data

  - Algorithm: After each reading time the new requests for pages are generated by the user (mean of 31 seconds), generate a HTTP request with the following parameters enlisted below. The parsing time is the time taken by the HTTP page to fill in all subpage requests which appear from the master page. After going through few of the subpages the user quits the session which is indicated by the last packet of the session. This is shown in Figure 1.

Reading Time

First Packet of Session

Last Packet of Session

Reading Time

Parsing Time

Figure 1: HTTP traffic pattern

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Main object size (SM) | Truncated Lognormal | Mean = 10710 bytes SD = 25032 bytes Min = 100 bytes Max = 2 Mbytes (before truncation) | σ=1.37,μ=8.37 if x>max or x<min, discard and generate a new value for x |
| Embedded object size (SE) | Truncated Lognormal | Mean = 7758 bytes SD = 126168 bytes Min = 50 bytes Max = 2 Mbytes (before truncation) | σ=2.36,μ=6.17 f x>max or x<min, discard and generate a new value for x |
| Number of embedded objects per page (Nd) | Truncated Pareto | Mean = 5.64 Max. = 53 (before truncation) | Subtract k from the generated random value to obtain Nd if x>max, discard and regenerate a new value for x |
| Reading time (Dpc) | Exponential | Mean = 30 sec |  = 0.033 |
| Parsing time (Tp) | Exponential | Mean = 0.13 sec | fx=λe−λx,x≥0 λ=7.69 |

- Hard disk file transfer

  - Transaction Model

    - A transaction consists of a READ request from host to drive for a specific block of data

    - Followed by the data transfer from drive to host

  - Algorithm

    - Compute sequence of inter-arrival times

    - Compute corresponding sequence of transaction data sizes

  - Parameters

    - READ request is a short (256B) packet sent from host to drive

    - fixed 1ms delay between receipt of READ request and data offered

    - Compute sequence of inter-arrival times of transaction requests with following discrete random variable distribution

    - Compute corresponding sequence of transaction data sizes with following discrete random variable distribution

## PHY Model

PHY abstraction and path loss models should be used for system level simulations.

PER vs. SNR curves obtained for PHY performance evaluation as described in Section 3.1 may be used for the PHY abstraction. Alternatively, a different PHY abstraction mechanism could be used, a description for which should be provided.

Path loss models developed in [3, Section 7] should be used for system evaluation.

As explained in Section 3.1, a PHY model (PER vs SNR curves and path loss models) may be derived if parameters of the antenna and beamforming algorithm are fixed. Section 3.1 defines the standard set of antenna and beamforming parameters.  However, TGad proposals may also include system evaluation results based on their proposed antenna and beamforming algorithm.

## Simulation Scenarios

### Home living room

An example of a home living room floor plan is show in Figure 2.

Figure 2: Example of home living room floor plan

Set top box transmitting uncompressed video, and TV receiving uncompressed video

- configuration as show in Figure 2, with 3 meter separation between STB and TV (Note: PHY channel model was developed with STB in the rectangular sector as show in Figure 2)

- traffic type

- Uncompressed video

- PHY channel impulse and pathloss model

- Home living room

- NLOS

### Office conference room

The office conference room floor plan is shown in Figure 3.

Figure 3: Office conference room floor plan

Mix of uses:  Laptop transmitting lightly compressed video to projector.  Multiple laptops connected to an AP that may or may not have a 60 GHz radio.  Laptop connected to device performing sync-and-go file transfer.  Laptops connected to other laptops performing local file transfer.  Links between devices are logical, e.g. STA 3 and STA 5 are performing local file transfer between each other but the physical link could be direct or through the AP.

Note: If the AP does not have a 60 GHz radio, then all client-to-AP communication is assumed to occur using other bands such as the 2.4 or 5 GHz band.

- configuration

- Room dimensions (length, width, height) in meters is 3.0 x 4.5 x 3

- Devices (coordinates of devices are calculated using coordinate axes shown in Figure 3)

    - AP (may or may not have a 60 GHz radio): location (x = 1.50 m, y = 0.50 m, z = 2.90 m – in ceiling)

    - STA 1:

      - Projector

      - location: ( x = 1.75 m,  y =2.30 m, z fixed at 1 m)

      - Traffic type: receiving lightly compressed video from STA 2 (LOS link)

    - STA 2:

      - Mobile Device

      - location:  (x = 1.90 m, y = 1.50 m,  z fixed at 1m)

      - Traffic type:

        - transmitting lightly compressed video to STA 1 (LOS link) with target bit rate (p) equal to 600 Mbps

        - Local file transfer from AP

      - Antenna capability: >=60 degree HPBW pointed towards STA 1

    - STA 3:

      - Laptop

      - location:  (x = 1.35 m, y = 3.00 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to/from STA 5  (NLOS link)

        - web browsing

    - STA 4:

      - Laptop

      - location:  (x = 1.30 m, y = 2.40 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to AP

        - web browsing

    - STA 5:

      - Laptop

      - location:  (x = 1.25 m,y = 1.40 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to/from STA 3 (NLOS link)

        - web browsing (if AP has a 60 GHz radio) or file transfer to STA 7 (if the AP does not have a 60 GHz radio)

    - STA 6:

      - Laptop

      - location:  (x = 1.55 m, y = 1.20 m, z fixed at 1m)

      - Traffic type:  web browsing

    - STA 7:

      - Laptop

      - location:  (x = 1.85 ,y = 3.10, z fixed at 1m)

      - Traffic type:

        - local file transfer to STA 8 (LOS link)

        - Local file transfer from AP (if the AP has 60 GHz radio) or file transfer from STA 5 (if AP does not have a 60 GHz radio)

    - STA 8:

      - mobile device

      - location: (x = 1.60, y = 3.25, , z fixed at 1m)

      - Traffic type: Local file transfer from STA 7 (LOS link)

      - Antenna capability: >= 60 degree HPBW pointed towards STA 7

- PHY channel impulse and pathloss model

- office conference room

- All links to the AP are LOS links that may be blocked by people. Model of the human blockage is in [3]. Type of links (LOS or NLOS) between two STAs is specified above.

- The following non-communicating pairs have NLOS channels: STA2  STA7, STA2  STA8, STA6  STA7, STA1 STA8. All other pairs have LOS channels, except due to human blockage.

- Interference modeling

    - Generate a set of channel realizations with inter-cluster parameters for all the links defined in the scenario with ray-tracing.  Store the generated channel realizations as a channel realization table in the simulator.

    - Total number of the channel realizations to be generated before simulations: 
2 STAs out of 9 STAs =92=36channel realizations

    - Signal power calculation procedure over the n-th link for a MAC simulator:

      - A packet received over the n-th link (link-n)

      - Read the link-n’s channel realization from the channel realization table

      - Block part of clusters and apply human blockage if necessary

      - Generate intra-cluster

      - Apply antenna model and beamforming

      - Calculate received signal power

Figure 4: Signal power calculation procedure over the n-th link

Enterprise cubicle

The enterprise cubicle floor plan is shown in Figure 5.

Figure 5: Enterprise cubicle floor plan

Figure 6: Locations of the STAs within a cube

Mix of uses:  Laptop transmitting lightly compressed video to monitor.  Laptop connected to AP.  Laptop connected to hard drive.

- configuration

  - cubicle layout

    - single cubicle (length, width) in meters: 2.5m x 1.8 m

    - 8 cubicles in 4 rows, 2 columns

    - ceiling height 3m

    - Populate three cubes at Cubicle 1, Cubicle 2, and Cubicle 5 in the fixed locations as shown in Figure 5 using same frequency channel

  - Floor dimension: 25m x 25m

  - AP: AP located in the ceiling in the middle of the group of cubicles as indicated in Figure 5, at a height of 2.9m)

  - Devices in each of the populated cubes

    - STA 1:

      - monitor

      - location: (x=0.5 m,y=0.5 m) from the reference point of each cube shown in Figure 6, z fixed at 1m

      - Traffic type: receiving lightly compressed video from STA 2

    - STA 2:

      - Laptop

      - location: (x=1 m,y=0.25 m) from the reference point of each cube shown in Figure 6, z fixed at 1m

      - Traffic type:

        - transmitting lightly compressed video to STA 1 with target bit rate (p) equal to 600 Mbps

        - Local file transfer to/from AP

        - Local file transfer to/from STA 3

        - web browsing to/from AP

    - STA 3:

      - hard drive

      - location: (x=0.25m,y=1m)from the reference point of each cube shown in Figure 6, z fixed at 1m

      - Traffic type: Hard disk file transfer to/from STA 2

- PHY channel and pathloss model

- Enterprise cubicle

- TBD mix of LOS and NLOS in cubicle, NLOS between cubicles, TBD mix of LOS/NLOS to AP

- Interference modeling

    - Use the same procedure as described in Figure 4.

    - Total number of the channel realizations to be generated before simulations: 
2 STAs out of 9 STAs + 3 links between the laptops to the AP =92+3=39channel realizations

## Comparison Criteria

- goodput (aggregate and per flow)

  - average

- delay (per flow)

  - average

  - # of packets that exceed delay requirement

- packet loss rate (per flow)

- Provide description of PHY abstraction & antenna model

- Provide description of scheduling algorithm

# References

- 11-08-0806-07-0vht-60-ghz-par-nescom-form-plus-5Cs.doc

- 11-07-2988-04-0000-liaison-from-wi-fi-alliance-to-802-11-regarding-wfa-vht-study-group-consolidation-of-usage-models.ppt

- 11-09-0334-YY-00ad-draft-channel-models-for-60-ghz-wlan-systems.doc

- 11-09-0583-00-00ad-tgad-usage-model.ppt

- 11-09-0709-02-00ad-lightly-compressed-video-traffic-modeling.ppt

- 11-09-1216-01-00ad-internet-traffic-modeling.ppt

- 11-09-1213-01-00ad-60ghz-impairments-modeling.ppt

- 11-09-1222-01-00ad-harddrive-traffic-model.ppt

- 11-09-1229-01-00ad-Application-of-60-GHz-Channel-Models-for Comparison-of-TGad-Proposals.ppt

- 11-09-1317-00-00ad-internet-traffic-modeling.ppt

- 11-09-0228-05-00ad-functional-requirements.doc

- 11-10-0067-00-00ad-tgad-interference-modeling-for-mac-simulations.ppt

- 11-10-0046-00-00ad-proposed-additions-to-evaluation-methodology.pptx

- 11-10-0149-00-00ad-proposed-evaluation-methodology-addition.pptx