IEEE P802.11
Wireless LANs

| Vendor Specific Public Action Frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-3-9 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chris Hartman | Apple | 1 Infinite Loop Cupertino, CA 95014 | 408-862-9317 | chartman@apple.com |
| Henry Ptasinski | Broadcom | 190 Mathilda Place, Sunnyvale CA | 408-543-3316 | henryp@broadcom.com |
| Emily Qi | Intel Corporation | 2111 NE 25th Ave, Hillsboro OR | 5032647799 | Emily.h.qi@intel.com |

- Abstract

The Vendor Specific Action frame was introduced in TGma, and Public Action frame was introduced in TGk, which didn’t consider the vendor specific action frame use cases. This document provides a normative text proposal for Vendor Specific Public Action Frame that allows vendor specific action frame to be transmitted prior to the association.

7.4.7 Public Action details

7.4.7.1 Public Action frames

Insert the following new row into Table 7-57e and update the reserved value as shown:

Table 	7-57e—Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Vendor Specific |

Insert the following at the end of clause 7.4.7:

7.4.7.x Vendor Specific Public Action frame format

The Vendor Specific Public Action frame is defined for vendor-specific signaling between unassociated STAs. The format of the Vendor Specific Public Action frame is shown in Figure mx1.

|  | Category | Action | OUI | Vendor Specific Content |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | variable |

Figure mx1 Vendor Specific Public Action Frame format

The Category field is set to the value indicating the Public category, as specified in Table 7-24 in 7.3.1.11.

The Action field is set to the value indicating Vendor Specific Public, as specified in Table 7-57e in 7.4.7.1.

The OUI field is a public OUI assigned by the IEEE. It is 3 octets in length. It contains the OUI of the entity that has defined the content of the particular Vendor Specific Public Action.

The Vendor Specific Content contains the vendor specific fields and may include Information Elements defined in the standard. The length of the Vendor Specific Content in a Vendor Specific Public Action frame is limited by the maximum allowed MMPDU size.

Insert the following at the end of clause 10.3:

    - Vendor Specific Public Action

This set of primitives supports the signaling of Vendor Specific Public Action frames between peer SMEs.

      - MLME-PVSPECIFIC.request

        - Function

This primitive requests transmission of a Vendor Specific Public Action frame to a peer entity.

        - Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-PVSPECIFIC.request	(
Peer MAC Address,
OUI,
Vendor Specific Content
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Vendor Specific Public Action frame is sent. |
| OUI | 3 octets | 00-00-00 – FF-FF-FF | A public value assigned by the IEEE to identify the entity that has defined the content of the particular Vendor Specific Public Action. |
| Vendor Specific Content | Set of information elements and vendor specific fields | Set of information elements and vendor specific fields | A set of information elements as defined in 7.3.2. A set of vendor specific fields specifying the required fields for the Vendor Specific Public Action frame. |

        - When Generated

This primitive is generated by the SME to request that a Vendor Specific Public Action frame be sent to a peer entity.

        - Effect of Receipt

On receipt of this primitive, the MLME constructs a Vendor Specific Public Action frame containing the set of information elements and vendor specific fields. The STA then attempts to transmit the frame to the peer entity.

      - MLME-PVSPECIFIC.confirm

        - Function

This primitive reports the result of a request to send a Vendor Specific Public Action frame to the peer entity.

        - Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-PVSPECIFIC.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME- PVSPECIFIC.request. |

        - When Generated

This primitive is generated by the MLME when the request to transmit a Vendor Specific Public Action frame completes and indicates the results of the request.

        - Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

      - MLME-PVSPECIFIC.indication

        - Function

This primitive indicates that a Vendor Specific Public Action frame has been received from a peer entity.

        - Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-PVSPECIFIC.indication	(
Peer MAC Address,
OUI,
Vendor Specific Content
)

| Name | Type | Valid Range |  | Description |
| --- | --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address |  | The address of the peer MAC entity from which a Vendor Specific Public Action frame was received. |
| OUI | 3 octets |  | 00-00-00 – FF-FF-FF | A public value assigned by the IEEE to identify the entity that has defined the content of the particular Vendor Specific Public Action. |
| Vendor Specific Content | Set of information elements and vendor specific fields | Set of information elements and vendor specific fields |  | A set of information elements as defined in 7.3.2. A set of vendor specific fields specifying the required fields for the Vendor Specific Public Action frame. |

        - When Generated

This primitive is generated by the MLME when a valid Vendor Specific Public Action frame is received.

        - Effect of Receipt

On receipt of this primitive, the Vendor Specific Content can be made available for SME processes.