IEEE P802.11
Wireless LANs

| TGac Channel Model Addendum |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Greg Breit | Qualcomm Incorporated | 5775 Morehouse Drive, San Diego, CA, 92121 USA | +18586513809 | gbreit@qualcomm.com |
| Hemanth Sampath | Qualcomm Incorporated | 5775 Morehouse Drive, San Diego, CA, 92121 USA |  | hsampath@qualcomm.com |
| Sameer Vermani | Qualcomm Incorporated | 5775 Morehouse Drive, San Diego, CA, 92121 USA |  | svverman@qualcomm.com |
| Richard Van Nee | Qualcomm Incorporated | Straatweg 66S, 3621BR Breukelen, The Netherlands |  | rvannee@qualcomm.com |
| Minho Cheong | ETRI | 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea |  | minho@etri.re.kr |
| Byung-Jae Kwak | ETRI | 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea |  | bjkwak@etri.re.kr |
| Myung Don Kim | ETRI | 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea |  | mdkim@etri.re.kr |
| Jae Joon Park | ETRI | 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea |  | jjpark@etri.re.kr |
| Naoki Honma | NTT | 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea |  | honma.naoki@lab.ntt.co.jp |
| Takatori Yasushi | NTT | 1-1 Hirkarinooka, Yokosuka-shi, Kanagawa, Japan, 239-0847 |  | takatori.yasushi@lab.ntt.co.jp |
| Yongho Seok | LG | LG R&D Complex, Anyang-Shi, 431-749, Korea |  | yhseok@lge.com |
| Seyeong Choi | LG | LG R&D Complex, Anyang-Shi, 431-749, Korea |  | seyeong.choi@lge.com |
| Phillipe Chambelin | Thomson | 1 Av Belle Fontaine – CS 17616 35576 Cesson Sévigné, France |  | philippe.chambelin@thomson.net |
| John Benko | Orange | 801 Gateway Blvd. Suite 500 South San Francisco, CA 94114 USA |  | john.benko@orange-ftgroup.com |
| Laurent Cariou | Orange | 4 rue du Clos Courtel 35510 Cesson Sévigné, France |  | laurent.cariou@orange-ftgroup.com |
| VK Jones | Qualcomm Incorporated | 3105 Kifer Road, Santa Clara, CA, 95051 USA |  | vkjones@qualcomm.com |
| Allert Van Zelst | Qualcomm Incorporated | Straatweg 66S, 3621BR Breukelen, The Netherlands |  | allert@qualcomm.com |
| Lin Yang | Qualcomm Incorporated | 5775 Morehouse Drive, San Diego, CA, 92121 USA |  | linyang@qualcomm.com |
| Thomas Kenney | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97229 USA |  | thomas.j.kenney@intel.com |
| Eldad Perahia | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97229 USA |  | eldad.perahia@intel.com |
| Vinko Erceg | Broadcom | 16340 W Bernardo Dr; San Diego CA 92127 USA |  | verceg@broadcom.com |

Abstract

This document provides the addendum to the TGn channel model document to be used for the Very High Throughput Task Group (TGac).

Revision History

| Date | Version | Description of changes |
| --- | --- | --- |
| 03/09/09 | 1.0 | First Draft of TGac Channel Modem Addendum Document |
| 04/22/09 | 2.0 | Updates to Sections 2, 3 and 5. |
| 05/11/09 | 3.0 | Updates to all sections for May 2009 TGac meeting. Supporting material extracted to [3] |
| 05/11/09 | 4.0 | Revision to author list |
| 05/12/09 | 5.0 | Revision to Doppler section. Motion passed on 12 May 2009 to accept this as baseline document for TGac channel models. |
| 07/15/09 | 6.0 | Implemented all document changes described in IEEE 802.11-09/0779r0, with the exception of the Doppler model recommendation. Doppler section text is unchanged from r5. |

# Introduction

The TGn task group has developed a comprehensive MIMO broadband channel model, with support for 40 MHz channelization and 4 antennas [1]. The TGac task group is targeting >1 Gbps MAC SAP throughput using one or more of the following technologies:

- Higher order MIMO (> 4x4)

- Higher Bandwidth (> 40 MHz)

- Multi-User MIMO with > 4 AP antennas

- OFDMA

This document describes modifications to TGn channel models to enable their use for TGac. The reader is referred to [3] for supporting data and justification.

The scope of this document is limited to extensions of the existing TGn channel model definitions (i.e., Models A-F). The introduction of a new model definition covering indoor corridor propagation is TBD.

# Modifications for Larger System Bandwidth

The TGn channel models assume minimum tap spacing of 10 ns and were employed for system bandwidth of up to 40 MHz. TGac systems may have much larger bandwidth. For TGac systems with larger overall system bandwidth, the channel sampling rate shall be increased by reducing the power delay profile (PDP) tap spacing by a factor of 2ceilinglog2W40, where W is the new system bandwidth in MHz. These scaling factors are summarized in Table 1, for up to 1.28 GHz system bandwidth

Table 1: Channel sampling rate expansion (tap spacing reduction) factors

| System Bandwidth W | Channel Sampling Rate Expansion Factor | PDP Tap Spacing |
| --- | --- | --- |
| W ≤ 40 MHz | 1 | 10 ns |
| 40 MHz < W ≤ 80 MHz | 2 | 5 ns |
| 80 MHz < W ≤ 160 MHz | 4 | 2.5 ns |
| 160 MHz < W ≤ 320 MHz | 8 | 1.25 ns |
| 320 MHz < W ≤ 640 MHz | 16 | 0.625ns |
| 640 MHz < W ≤ 1.28 GHz | 32 | 0.3125 ns |

The PDP tap spacing shall be reduced by generating new PDP taps based on linear interpolation of the TGn-defined PDP tap powers on a cluster-by-cluster basis using the following approach:

For each cluster in the TGn-defined model, and assuming a channel sampling rate expansion factor k (new sampling rate = k*100MHz), a sequence of k-1 new PDP taps, spaced 10/k ns apart, shall be appended after each TGn-defined PDP tap. The first PDP tap in the sequence shall occur 10/k ns after the TGn-defined PDP tap. The power (in dB) assigned to each new tap shall be determined by linear interpolation of the TGn-defined PDP tap powers (in dB) immediately before and after the new PDP tap, in proportion to its position in time relative to the two TGn PDP taps. No new PDP taps shall be added after the final TGn PDP tap for each cluster.

Figure 1 illustrates this procedure for the example of k=4 (new channel sampling rate = 400MHz), for a hypothetical pair of TGn-defined PDP taps spaced 20ns apart. The TGn PDP taps are denoted by the thick grey arrows, and the new interpolated PDP taps are denoted by the thin black arrows. In this case, 3 new taps are added 2.5ns, 5.0ns, and 7.5ns after TGn Tap i. Power for each of the new PDP taps is derived from the line connecting the power of the TGn PDP Taps i and i+1, which accomplishes the dB-proportional power interpolation described above. This procedure is performed for all TGn PDP Taps i for i=1 to (n_taps-1), where n_taps is the number of PDP taps in the cluster being interpolated.

Figure 1: Illustration of PDP tap interpolation scheme for channel bandwidth expansion

Tap interpolation must be performed on the TGn cluster definitions themselves in order to allow for appropriate normalization of the NLOS PDP and ensure conservation of energy in the final modeled channel. This process is validated in [3], which shows that Ricean K is preserved after interpolation.

Note that since the interpolated PDP taps result in independent channel tap realizations, the newly generated TGac channel, post-interpolation, is a fundamentally different channel compared to TGn base channels. Such a TGac channel will have larger frequency diversity compared to TGn base channel. Hence, for development work going forward, it is recommended to state explicitly whether simulations are based on TGn or TGac versions of the channel models.

# Higher Order MIMO

The TGn channel models were originally conceived for systems with 4x4 MIMO, and are based on the Kronecker channel correlation model assumption [2].

The TGac channel models shall use the identical Kronecker correlation model for simulation of higher-order MIMO channels. See [3] for supporting measurements.

# Modifications to AoA and AoD for Multi-User MIMO

TGac requires specification of channels to multiple users as simultaneous communication will take place to multiple STAs in technologies like multi-user MIMO. The TGn channel model document specifies the cluster AoAs and AoDs for point-to-point single user transmissions. Extensions of these AoDs and AoAs to the multi-user case are needed.

Multi-user channels shall be modeled with the following modifications to the AoA and AoD for each client:

- Assume TGn-defined cluster AoDs and AoAs as baseline.

- For each client:

  - Apply single random offset uniformly distributed over ±180° to the LOS tap AoA

  - Apply single random offset uniformly distributed over ±180° to the LOS tap AoD

  - Apply single random offset uniformly distributed over ±180° to the NLOS cluster AoAs

  - Apply single random offset uniformly distributed over ±180° to the NLOS cluster AoDs

See [3], [7], and [8] for supporting data

Note: The random offsets specified above shall be generated using the method described in Appendix A.

# Modifications to Doppler Components

Recent indoor channel measurements [4, 9, 10, 11] indicate that the magnitude of Doppler assumed in the TGn channel model is too high. TGac shall use the Doppler model specified in the TGn channel model document, with the following modifications:

In Section 4.7.1 of TGn channel model document, the environmental speed, vo, may be reduced to TBD km/h.

# Incorporating Dual-Polarized Antennas

By exploiting polarization diversity in the channel, dual-polarized antennas may provide significant improvement in MIMO channel capacity, especially in LOS scenarios. Furthermore, co-located dual-polarized antennas can minimize real estate in devices with large number of antennas, making them likely to be employed in TGac devices.

The TGac channel model may (TBD) use the TGn-defined polarization model with the following parameters:

- XPD value of 10 dB for channel elements representing transmission between orthogonally-polarized antennas in the steering matrix HF

- XPD value of 3 dB for for channel elements representing transmission between orthogonally-polarized antennas in the variable matrix Hv

- Correlation of 0 for orthogonally-polarized antenna elements

The channels incorporating XPD shall be normalized only to the norm of the co-polarized elements of the channel matrix. This is because normalization to the Frobenius norm of the entire channel matrix will fail to account for the additional path loss due to transmitting and receiving on orthogonal polarizations.

Appendix A – Generation of Pseudorandom Per-User AoA and AoD Offsets for MU-MIMO Channel Model

Section 4 describes extensions to the TGn channel model to support multi-user (MU) MIMO scenarios, where multiple spatially-separated clients are served by a single AP. For each client, pseudorandom offsets shall be added to TGn-defined cluster AoAs and AoDs to simulate diversity of user position and orientation with respect to the AP. For each user, a different pseudorandom angle offset shall be applied to the LOS (steering matrix) AoD, NLOS cluster AoD, LOS AoA, and NLOS AoA. This appendix specifies the method to generate pseudorandom angle offsets to allow consistent implementation across development groups performing MU-MIMO channel simulation.

For each TGn angle parameter (e.g. LOS AoA), per-user angle offsets shall take the form of:

where Δφi is the angle offset in degrees for User i, xi is a pseudorandom floating point value for User i, uniformly distributed on [0,1], and range is the total range of uniformly distributed random angle offsets specified in Section 4 for the TGn angle parameter being modeled (e.g. 360° for LOS AoA).

The pseudorandom values shall be produced using a simple multiplicative congruential random number generator [5]. Although this method is unsuitable for large-scale Monte Carlo simulations, it is adequate for producing the limited number of pseudorandom values required for this application and has the following advantages over more sophisticated generators:

  - This is the default method for the rand() function in MATLAB Version 4 and is still supported by that tool for legacy compatibility.

  - This method is well established and easily coded by users wishing to implement the TGac channel model without relying on MATLAB.

For each angle parameter, a different seed value shall be used to initialize the random number generator. A sequence of values is generated for each seed, one value per MU-MIMO client. For LOS AoA and AoD, the offset is applied to the steering matrix. For NLOS AoA and AoD, the offset is applied to all cluster angles.

A.1 – MATLAB Implementation

The following seed values (selected randomly from a list of primes between 2 and 50000) shall be used to generate angle offsets by the MATLAB multiplicative congruential generator:

| Angle parameter (Downlink) | Angle parameter (Uplink) | MATLAB seed value |
| --- | --- | --- |
| LOS AoD (steering matrix) | LOS AoA (steering matrix) | 39161 |
| NLOS AoD (cluster angles) | NLOS AoA (cluster angles) | 2803 |
| LOS AoA (steering matrix) | LOS AoD (steering matrix) | 45191 |
| NLOS AoA (cluster angles) | NLOS AoD (cluster angles) | 13367 |

The following example MATLAB code generates a column vector of user offsets for downlink LOS AoD:

n_users = 6;              % number of clients

seed_AoD_LOS = 39161;     % seed value for DL LOS AoD values

range_AoD_LOS = 360;      % specified range of LOS AoD diversity (+/-180deg)

% NOTE:’seed’ method specifies MATLAB multiplicative congruential generator

rand('seed',seed_AoD_LOS);   % initialize random number generator

offsets_AoD_LOS = (rand(n_users,1)-0.5)*range_AoD_LOS; % generate offsets

For reference, this example code generates the following angle offsets:

| User | LOS AoD Offset (degrees) |
| --- | --- |
| 1 | -78.0189 |
| 2 | -142.9707 |
| 3 | 91.0158 |
| 4 | 62.9668 |
| 5 | -116.7050 |
| 6 | 178.2852 |

As of MATLAB Version 7.7.0, a new object-oriented syntax for random number generation has been adopted although the legacy method is still supported for backwards compatibility. The following example code uses the new syntax to produce the identical random sequence:

n_users = 6;              % number of clients

seed_AoD_LOS = 39161;     % seed value for DL LOS AoD values

range_AoD_LOS = 360;      % specified range of LOS AoD diversity (+/-180deg)

% NOTE: 'mcg16807' method specifies MATLAB multiplicative congruential generator.

% ’Seed’ in this case refers to a property of the random stream s1.

s1 = RandStream('mcg16807','Seed',seed_AoD_LOS); % init random number generator

offsets_AoD_LOS = (rand(s1,n_users,1)-0.5)*range_AoD_LOS;  % generate offsets

A.2 – MATLAB-Independent Implementation

The general expression for a linear congruential random number generator is:

where a, c, and m are constants, and I0, I1, I2,… is a sequence of pseudorandom integers between 0 and (m-1). The MATLAB implementation uses 32-bit integers and assumes a = (75), c = 0, and m = (231-1) [6]. A uniform deviate between 0 and 1 is produced by dividing the sequence I by the modulus m. The method by which MATLAB seeds the generator from user-supplied seed values is undocumented, but the following seed values (used as I0) will produce angle offset sequences equivalent to the MATLAB implementation described above:

| Angle parameter (Downlink) | Angle parameter (Uplink) | Seed value (I0) |
| --- | --- | --- |
| LOS AoD (steering matrix) | LOS AoA (steering matrix) | 608341199 |
| NLOS AoD (cluster angles) | NLOS AoA (cluster angles) | 1468335517 |
| LOS AoA (steering matrix) | LOS AoD (steering matrix) | 266639588 |
| NLOS AoA (cluster angles) | NLOS AoD (cluster angles) | 115415752 |

The following example C code will produce a sequence of angle offsets equivalent to the MATLAB examples above:

#include <stdio.h>

main() {

int i;

int n_users = 6; /* Number of clients */

int range_AoD_LOS = 360; /* specified range of LOS AoD diversity (+/-180deg) */

unsigned long seed_AoD_LOS = 608341199; /* seed value for DL LOS AoD values */

unsigned long a = 16807;      /* (7^5) */

unsigned long m = 2147483647; /* (2^31-1) */

unsigned long rand_val[16];

float rand_variant;

rand_val[0] = seed_AoD_LOS; /* Seed sequence */

/* Generate sequence */

for (i=1; i<n_users; i++) {

rand_val[i] = (a*rand_val[i-1])%m;

}

/* Print sequence */

for (i=0; i<n_users; i++) {

rand_variant = (double)rand_val[i]/(double)m; /* normalize to modulus */

printf("%d  %10.4f\n", i, (rand_variant-0.5)*range_AoD_LOS);

}

}

References

- Erceg, V. et al. “TGn Channel Models.”  Doc. IEEE802.11-03/940r4.

- Schumacher, L.; Pedersen, K.I.; Mogensen, P.E., "From antenna spacings to theoretical capacities - guidelines for simulating MIMO systems," Personal, Indoor and Mobile Radio Communications, 2002. The 13th IEEE International Symposium on, vol.2, no., pp. 587-592 vol.2, 15-18 Sept. 2002.

- Breit, G. et al., “TGac Channel Model Addendum Supporting Material,” Doc. IEEE802.11-09/0569r0.

- Nishimori, K. et al., “Measured Doppler Frequency in Indoor Office Environment,” Doc. IEEE802.11-09/537r0.

- W. H. Press, S. A. Teukolsky, W. T. Vetterling, B. P. Flannery, Numerical Recipes: The Art of Scientific Computing, Third Edition. Cambridge University Press, 2007.

- C. Moler, “10435 Years is a Long Time,” MATLAB News & Notes, Fall 1995. http://www.mathworks.com/company/newsletters/news_notes/pdf/Cleve.pdf

- Breit, G. et al., “Multi-User AoD Diversity Measurements.” Doc. IEEE802.11-09/0699r5.

- Kwak, B.J. et al., “AoD and AoA Estimation for TGac.” Doc. IEEE 802.11-09/0822r0.

- Perahia, E., “Investigation into the 802.11n Doppler Model.” Doc. IEEE 802.11-09/0538r0.

- Perahia, E., “Channel Coherence Time.” Doc. IEEE 802.11-09/0784r0.

- Yamada, W. et al., “Coherence Time Measurement in NTT Lab.” Doc. IEEE 802.11-09/0828r0.