IEEE P802.11
Wireless LANs

| LB-142 CID 5350 update to Emergency Alert functionality |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion (RIM) UK Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This proposal addresses CIDs 5350, 5371

This document is based on IEEE 802.11u-D5.0

Change all occurances of the MIB value “dot11EASNEnabled” to “dot11EASEnabled” throughout the whole draft.

Change the following definition in clause 3

3.u.5 homogenous ESS: A collection of BSSes, within the same extended service set (ESS), in which the SSPN or other external network reachable at one BSS, is reachable at all of them. All BSSs identified by a homogeneous ESS identifier (HESSID) are in the same mobility domain, if one is defined.

Modify the following clauses:

- 7.3.2. Information elements

- Insert the following to the contents of Table 7-26 as shown below:

| - lE IDs |  |  |  |
| --- | --- | --- | --- |
| Information Element | Element ID | Length (in octets) | Extensible |
| Interworking (see See Interworking information element.) | <ANA> | 3, 4, 5, 6, 9, 10, 11, 12 |  |
| Advertisement Protocol (see See Advertisement Protocol information element.) | <ANA> | 4 to 257 |  |
| Emergency Services Public Credential (See Emergency Services Public Credential information element.) | <ANA> | 10 to 257 | Yes |
| Expedited bandwidth request (see See Expedited Bandwidth Request information element.) | <ANA> | 3 |  |
| QoS Map Set (see See QoS Map Set information element .) | <ANA> | 18 to 60 | Yes |
| Roaming Consortium (see See Roaming Consortium information element.) | <ANA> | 6, 9, 12 | Yes |
| Emergency Alert (see 7.3.2.89) | <ANA> | variable |  |

- The Element IDs will be assigned by ANA prior to sponsor ballot.

- 7.3.2.27 Extended Capabilities information element

- Insert the following additional rows at the end of Table 7-35a

| - Table 7-35a - Capabilities field |  |  |
| --- | --- | --- |
| Bit(s) | Information | Notes |
| <ANA> | Interworking | When dot11InterworkingServiceEnabled is set to TRUE, the Interworking field is set to 1 to indicate the STA supports Interworking Service as described in 11.21. When dot11InterworkingServiceEnabled is set to FALSE, the Interworking field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | QoS Map | When dot11QosMapEnabled is set to TRUE, the QoS Map field is set to 1 to indicate the STA supports QoS Map service as described in 11.21.6. When dot11QosMapEnabled is set to FALSE, the QoS Map field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | EBR | When dot11EBREnabled is set to TRUE, the EBR field is set to 1 to indicate the STA supports EBR as described in7.3.2.86. When dot11EBREnabled is set to FALSE, the EBR field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | SSPN Interface | When dot11SSPNInterfaceEnabled is set to TRUE, the SSPN Interface field is set to 1 to indicate the AP supports SSPN Interface service as described in 11.21.4. When dot11SSPNInterfaceEnabled is set to FALSE, the SSPN Interface is set to 0 to indicate the AP does not support this capability. Non-AP STAs shall set this field to 0. |
| <ANA> | EAS | When dot11EASEnabled is set to TRUE, the EASN field is set to 1 to indicate the STA supports the EAS mechanism as described in 11.18.5. When dot11EASEnabled is set to FALSE, the EASN field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | MSGCF Capability | When dot11MSGCFEnabled is set to TRUE, the MSGCF Capability field is set to 1 to indicate the non-AP STA supports the MSGCF in 11B. When dot11MSGCFEnabled is set to FALSE, the MSGCF Capability is set to 0 to indicate the AP does not support this capability. APs shall set this bit to 0. |

- Insert the following new subclauses:

- 7.3.2.83 Interworking information element

The Interworking information element contains information about the Interworking Service capabilities of a STA as shown in Figure 7-95o108.

|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN | Emergency Service Accessible | Venue Info (optional) | HESSID (optional) |
| Octets: | 1 | 1 | 1 |  |  |  |  | 0 or 2 | 0 or 6 |

The Length is a one octet field whose value is 1 if no optional fields are present, 3 if only Venue Info field is present, 7 if only HESSID field is present, or 9 if both optional fields are present.

The Network Types are shown in Figure 7-43bt. It is set by AP to advertise its Network Type to non-AP STAs. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See Annex V.1 for informative text on usage of fields contained within the Interworking element.

| - Figure 7-43bt - Network Type codes |  |  |
| --- | --- | --- |
| Network Type Codes | Meaning | Description |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network type are home networks and enterprise networks, which may employ user accounts. These networks may or may not employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts are available. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http:/https redirect or DNS redirection). |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. |
| 4 | Personal Device Network | A small network of personal devices such as camera and computer. |
| 5 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network type |

Bit 4 is the Internet field. The AP sets this field to 1 if the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 by the AP to indicate that the network requires a further step for access; otherwise it is set to 0. For more information, refer to Network Authentication Type Information in 7.3.4.5.

Bit 6 is the Emergency Alert System Notification (EASN) field. It is set to 1 by the AP to indicate that a higher layer EASN is available at the AP for download.  Either a native GAS query element (Clause 7.3.4.16) or an EA IE (Clause xxx) is then used to determine whether this is a new Emergency Alert which has not been already downloaded.  For more information, refer to 11.21.5.

Bit 7 is the Emergency Service Accessible field. When the AP sets it to 0, this field indicates that no emergency services are directly configured to be reachable through the SSID. When set to 1, this field indicates that higher layer emergency services are reachable through the SSID, in addition to any other services offered. An AP advertises this capability if at least one of the two methods is supported, as described in Annex V.4.4 and Annex V.4.5.

For a network to be identified as an Emergency Service (ES) network, location capability is enabled on an AP if the AP is located in a regulatory domain that requires location capabilities. Location capability is enabled when the ES CIVIC Location or ES Geo Location field in the Extended Capabilities Element is set to 1 in a Beacon or probe response frame. In addition, this field is only advertised if all of the following are true (see Annex V.4.2. for further information):

- dot11QosOptionImplemented is true.

- dot11EBREnabled is true.

The Venue Info is a 2-octet field. It contains Venue Group and Venue Type duple. The format of Venue Info is shown in Figure 7-95o109.

| - Venue Info format |  |  |
| --- | --- | --- |
|  | Venue Group | Venue Type |
| Octets: | 1 | 1 |

The Venue Group and Venue Type fields are both one octet values selected from See Venue Group codes and descriptions. and See Venue Type assignments. respectively. The entries in See Venue Group codes and descriptions. and See Venue Type assignments. are drawn from the International Building Code's Use and Occupancy Classifications See International Code Council, Inc., "International Building Code 2006", November 2006, ISBN-13: 978-1-58001-251-5...

| - Venue Group codes and descriptions |  |
| --- | --- |
| Venue Group Code | Venue Group Description |
| 0 | Unspecified |
| 1 | Assembly |
| 2 | Business |
| 3 | Educational |
| 4 | Factory and Industrial |
| 5 | Institutional |
| 6 | Mercantile |
| 7 | Residential |
| 8 | Storage |
| 9 | Utility and Miscellaneous |
| 10 | Vehicular |
| 11 | Outdoor |
| 12 | Personal Network |
| 13 - 254 | Reserved |
| 255 | Wildcard |

| - Venue Type assignments |  |  |
| --- | --- | --- |
| Venue Group | Venue Type Code | Venue Description |
| 0 | 0 | Unspecified |
| 0 | 1 - 255 | Reserved |
| 1 | 0 | Unspecified Assembly |
| 1 | 1 | Arena |
| 1 | 2 | Stadium |
| 1 | 3 | Passenger Terminal (e.g., airport, bus, ferry, train station) |
| 1 | 4 | Amphitheater |
| 1 | 5 | Amusement Park |
| 1 | 6 | Place of Worship |
| 1 | 7 | Convention Center |
| 1 | 8 | Library |
| 1 | 9 | Museum |
| 1 | 10 | Restaurant |
| 1 | 11 | Theater |
| 1 | 12 | Bar |
| 1 | 13 | Coffee Shop |
| 1 | 14 | Zoo or Aquarium |
| 1 | 15 | Emergency Coordination Center |
| 1 | 16 - 255 | Reserved |
| 2 | 0 | Unspecified Business |
| 2 | 1 | Doctor or Dentist office |
| 2 | 2 | Bank |
| 2 | 3 | Fire Station |
| 2 | 4 | Police Station |
| 2 | 6 | Post Office |
| 2 | 7 | Professional Office |
| 2 | 8 | Research and Development Facility |
| 2 | 9 | Attorney Office |
| 2 | 10 - 255 | Reserved |
| 3 | 0 | Unspecified Educational |
| 3 | 1 | School, Primary |
| 3 | 2 | School, Secondary |
| 3 | 3 | University or College |
| 3 | 4-255 | Reserved |
| 4 | 0 | Unspecified Factory and Industrial |
| 4 | 1 | Factory |
| 4 | 2 - 255 | Reserved |
| 5 | 0 | Unspecified Institutional |
| 5 | 1 | Hospital |
| 5 | 2 | Long-Term Care Facility (e.g., Nursing home, Hospice, etc.) |
| 5 | 3 | Alcohol and Drug Re-habilitation Center |
| 5 | 4 | Group Home |
| 5 | 5 | Prison or Jail |
| 5 | 6 - 255 | Reserved |
| 6 | 0 | Unspecified Mercantile |
| 6 | 1 | Retail Store |
| 6 | 2 | Grocery Market |
| 6 | 3 | Automotive Service Station |
| 6 | 4 | Shopping Mall |
| 6 | 5 - 255 | Reserved |
| 7 | 0 | Unspecified Residential |
| 7 | 1 | Hotel or Motel |
| 7 | 2 | Dormitory |
| 7 | 3 | Boarding House |
| 7 | 4 - 255 | Reserved |
| 8 | 0 - 255 | Reserved |
| 9 | 0 - 255 | Reserved |
| 10 | 0 | Unspecified Vehicular |
| 10 | 1 | Automobile or Truck |
| 10 | 2 | Airplane |
| 10 | 3 | Bus |
| 10 | 4 | Ferry |
| 10 | 5 | Ship or Boat |
| 10 | 6 | Train |
| 10 | 7 | Motor Bike |
| 10 | 8 - 255 | Reserved |
| 11 | 0 | Unspecified Outdoor |
| 11 | 1 | Muni-mesh Network |
| 11 | 2 | City Park |
| 11 | 3 - 255 | Reserved |
| 12 | 0 | Reserved |

The HESSID field specifies the value of HESSID, see 11.21.1.  A non-AP STA uses this field to indicate the desired HESSID in an active scan. The HESSID field shall be the value of dot11HESSID.

- 7.3.4.1 Native Info Capability List

The Native Info Capability List element provides a list of information/capabilities that has been configured on a STA. The native query protocol elements that may be configured are shown in See Native Query Protocol info ID definitions. along with the element's usage rules. This capability element may be returned in response to a Native GAS Query Request.

| - Native Query Protocol info ID definitions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  | IBSS |
| Info Name | Info ID | Native Info Element (clause) | AP | Non-AP STA | STA |
| Reserved | 0-255 | n/a | n/a | n/a | n/a |
| Capability List | 256 | See Native Info Capability List. | T, R | T, R | T, R |
| Venue Name information | 257 | See Venue Name information . | T | R | ---- |
| Emergency Call Number information | 258 | See Emergency Call Number information . | T | R | ---- |
| Emergency Public Network Access information | 259 | See Emergency Public Network Access information. | T | R | ---- |
| Network Authentication Type information | 260 | See Network Authentication Type information . | T | R | ---- |
| Roaming Consortium List | 261 | See Roaming Consortium List information. | T | R | ---- |
| AP Date, Time and Time Zone information | 262 | See AP Date, Time and Time Zone Information. | T | R | ---- |
| IP Address Type Availability information | 263 | See IP Address Type Availability Information. | T, R | T, R | T, R |
| NAI Realm List | 264 | See NAI Realm List. | T | R | T, R |
| 3GPP Cellular Network information | 265 | See 3GPP Cellular Network information. | T | R | ---- |
| AP Geospatial Location | 266 | See AP Geospatial Location. | T | R | ---- |
| AP Civic Location | 267 | See AP Civic Location. | T | R | ---- |
| Domain Name List | 268 | See Domain Name List. | T | R | ---- |
| Emergency Alert | 269 | See 7.3.4.16 | T | R | T,R |
| Reserved | 270- 56796 | n/a | n/a | n/a | n/a |
| Native Query Protocol vendor-specific query | 56797 | See Native Query Protocol vendor specific query. | T, R | T, R | T, R |
| Native Query Protocol vendor-specific response | 56798 | See Native Query Protocol vendor specific response. | T, R | T, R | T, R |
| Reserved | 56799 - 65535 | n/a | n/a | n/a | n/a |
| Symbols T Native query protocol element may be transmitted by MAC entity R Native query protocol element may be received by MAC entity --- Native query protocol element is neither transmitted nor received by MAC entity |  |  |  |  |  |

Add the following new Clause

- 7.3.2.89 Emergency Alert information element

The Emergency Alert information element provides identification details of the emergency alert which is currently stored (or available) in the network.  This detail allows the non-AP STA to then access whether this is a emergency alert which has not been previously seen and is therefore necessary to download from the network.  The format of the Emergency Alert information element is provided in Table 7-95o118a

| - Table 7-95o118a - Emergency Alert information element format |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | EICI | CAP identifier | CAP sender |  |
| Octets: | 1 | 2 | 1 | variable | variable |  |

The Length is a 2 - octet field whose value is equal to 1 octet plus the total length of the CAP identifier and CAP sender fields.

The Emergency Information Change Indication (EICI) field is used to indicate the number of a new EAS message. The value of the EICI field is incremented by 1 whenever a new EAS message arrives to the AP.  The absolute value of this field is not important, only the fact that it changes for each new message.

The CAP identifier field contains the <identifier> field from the CAP XML formatted message of the emergency alert.

The CAP sender field contains the <sender> field from the CAP XML formatted message of the emergency alert.

Add the following new clause

- 7.3.4.16 Emergency Alert Information

The Emergency Alert information provides identification details of the emergency alert which is currently stored (or available) in the network.  This detail allows the non-AP STA to then access whether this is an emergency alert which has not been previously seen and is therefore necessary to download from the network.  The format of the Emergency Alert information is provided in Table 7-95o142.

| - Table 7-95o142 - Emergency Alert information format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Info ID | Length | EICI | CAP identifier | CAP sender |
| Octets: | 2 | 2 | 1 | variable | variable |

The Length is the 2-octet field whose value is equal to 1 octet plus the total length of the CAP identifier and CAP sender fields.

The Emergency Information Change Indication (EICI) field is used to indicate the number of a new EAS message. The value of the EICI field is incremented by 1 whenever a new EAS message arrives to the AP.  The absolute value of this field is not important, only the fact that it changes for each new message.

The CAP identifier field contains the <identifier> field from the CAP XML formatted message of the emergency alert.

The CAP sender field contains the <sender> field from the CAP XML formatted message of the emergency alert.

Add the following text to the penultimate paragraph of clause 11.21.1

NOTE--It is required by this standard that the HESSID field in the Interworking element is administered consistently across all the BSSs in a homogeneous ESS.

Modify the following clause

11.21.5 Interworking Procedures: Emergency Alert System (EAS) Support

The Emergency Alert System Notification (EASN) provides alerts, typically issued by authorities, to be transmitted, from APs to non-AP STAs upon request from non-AP STAs. The mechanism uses the EASN Available  field in the Interworking element in Figure 7-95ao together with:

- for an un-associated STA, the Emergency Alert information element as described in clause 7.3.4.16

- for an associated STA, the Emergency Alert IE as described in clause 7.3.2.89.

When the Emergency Alert System Notification (EASN) is supported, dot11EASEnabled shall be set to TRUE. When EASN is not supported, dot11EASEnabled shall be set to FALSE.

Each time a new EAS alert is received by the IEEE 802.11 infrastructure, each AP shall advertise the availability of the new message by setting the EASN field in the interworking element to 1, and incrementing the last value of EICI field by 1.  The AP also allows transmission of the identifier and sender fields from the emergency alert system message.  When there is no EASN message in the AP, the EASN field in the interworking element shall be set to 0.

Once the non-AP STA receives the EASN (due to EASN being set to 1), it will retrive the EA information (through either the native GAS query or the EA IE).  The combination of the EICI value, the identifier and the sender information allows the non-AP STA to determine whether this is a new emergency alert which it has not seen before, by comparing these fields with the previous values.

If the values are different, then the non-AP STA will retrieve the EAS message from AP using the non-Native GAS procedures.

If the values are the same (as the same as the previously stored values) the non-AP STA will then ignore the EASN, since the EASN is for the same message which the non-AP STA has already retrieved.

Note: It is assumed that the network EAS messages are formatted using the Common Alert Protocol (CAP). The CAP allows the collection and distribution of “all-hazard” safety notifications and emergency warnings across information.