IEEE P802.11
Wireless LANs

| LB-142 Network Usage Codes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses a candidate remedy to CID 5009.  The commenter has expressed that the term “Network Type” is confusing and conflicts/collides with the same term which is used for a different purpose in IEEE 802.11-2007 clause 8.7.2.1.  This submission replaces “Network Type” with “Network Usage”.

This document is based on 802.11u-d5.0.

3	Definitions

Insert the new definition alphabetically re-numbering as necessary

3.u.x network usage:  the primary conditions under which a user or subscriber will access an IEEE 802.11 network.  For example, an enterprise network has a condition of access of private network and users, which are employees of the enterprise, would expect to have user accounts to access the network and that other users will also be employed by the enterprise.

5	General Description

5.4	Overview of the services

5.4.8	Interworking with External Networks

Change the text below as shown

An overview of the interworking functions addressed in this standard is provided below:

- Network Discovery and Selection

  - Discovery of suitable networks through the advertisement of network usage, roaming consortium and venue information

  - Selection of a suitable IEEE 802.11 infrastructure using advertisement services in the BSS or in the ESS

  - Selection of an SSPN or External Network with its corresponding IEEE 802.11 infrastructure

- Emergency Services

  - Emergency Call and Network Alert support at the link level

- QoS Map distribution

- SSPN Interface service between the AP and the SSPN

7	Frame formats

7.3	Management frame body components

7.3.2	Information elements

7.3.2.83	Interworking information element

Change the text as shown below

The Network Usage are shown in Table 7-43bt. It is set by AP to advertise its Network Usage to non-AP STAs. A non-AP STA uses this field to indicate the desired Network Usage in an active scan. See Annex V.1 for informative text on usage of fields contained within the Interworking element.

Table 7-43bt—Network Usage codes

| Network Usage Codes | Meaning | Description |
| --- | --- | --- |
| 0 | Private network | Non-authorized users are not permitted on this network. Examples of this network usage are home networks and enterprise networks, which may employ user accounts. These networks may or may not employ encryption. |
| 1 | Private network with guest access | Private network but guest accounts are available. Examples of this network usage are enterprise networks offering access to guest users. |
| 2 | Chargeable public network | The network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (e.g., 802.21, http:/https redirect or DNS redirection). Examples of this network usage is a hotspot in a coffee shop offering internet access on a subscription basis or a hotel offering in-room internet access service for a fee. |
| 3 | Free public network | The network is accessible to anyone and no charges apply for the network use. An example of this network usage is an airport hotspot or municipal network providing free service. |
| 4 | Personal Device Network | A small network of personal devices such as camera and computer. An example of this type of network usage is a camera attaching to a printer, thereby forming a network for the purpose of printing pictures. |
| 5 to 13 | Reserved | Reserved |
| 14 | Test or experimental | The network is used for test or experimental purposes only. |
| 15 | Wildcard | Wildcard network usage |

10	Layer management

10.3	MLME SAP interface

10.3.2	Scan

10.3.2.1	MLME-Scan.request

10.3.2.1.2	Semantics of the service primitive

In the parameter list,change the term “NetworkType” to”NetworkUsage”

In the parameter table, in the Name column change the term “NetworkType” to”NetworkUsage” and in the Description column change the term “network type” to “network usage” (in 2 places)

11	MLME

11.1	Synchronization

11.1.3	Acquiring synchronization, scanning

11.1.3.2	Active scanning

11.1.3.2.1	Sending a probe response

Change the text as shown below

STAs having dot11InterworkingServicesEnabled set to TRUE, subject to criteria below, receiving Probe Request frames containing an Interworking field in the Extended capabilities element set to 1 shall respond with a Probe Response only if:

EDITORIAL NOTE--802.11v also makes changes to this bullet. It needs to be aligned before the sponsor ballot.

- the SSID in the Probe Request is the wildcard SSID or the specific SSID of the STA,

- the BSSID field in the Probe Request is the wildcard BSSID or the BSSID of the STA,

- the DA field in the Probe Request is the broadcast address or the specific MAC address of the STA,

- the HESSID field, if present in the Interworking element, is the wildcard HESSID or the HESSID of the STA.

- the Network Usage field in the Interworking element is the wildcard Network Usage or the Network Usage of the STA.

11.21	WLAN Interworking with External Networks Procedures

11.21.1	Interworking Capabilities and Information

Change the text as shown below

The Interworking information element also provides a Network Usage value in Beacon and Probe Response frames to assist the non-AP STA with network discovery and selection.

11B	MAC State Generic Convergence Function

11B.5	MAC State Generic Convergence SAP

11B.5.1	ESS Status Reporting

11B.5.1.6	MSGCF-ESS-Link-Scan.request

11B.5.1.6.2	Semantics of the service primitive

In the parameter list,change the term “NetworkType” to”NetworkUsage”

In the parameter table, in the Name column change the term “NetworkType” to “NetworkUsage”

Change the sentence in the parameter table in the Description column as shown below

This may be a specific value to match one usage of networks, or all 1's to match all network usages.

Annex A

(normative)

Protocol Implementation Conformance Statement (PICS) Proforma

A.4.22	Interworking (IW) with External Networks extensions

In the PICS tablet, in the IW1.2 row, change the Protocol Capability from “NetworkType” to”NetworkUsage”

Annex D

Change the text as shown below

dot11ESNetwork OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The Emergency Services Network Usage set to TRUE for this HESSID set Indicates that higher layer emergency call services are reachable via this SSID.“

::= {dot11StationConfigEntry 120}

Annex V

(informative)

Interworking with External Networks

On P152L40, change the change the term “NetworkType” to “NetworkUsage”

On P152L44, change the change the term “NetworkType” to “NetworkUsage”

On P153L18, change the change the term “NetworkType” to “NetworkUsage”

On P153L23, change the change the term “NetworkType” to “NetworkUsage”

On P153L58, change the change the term “NetworkType” to “NetworkUsage”

On P153L62, change the change the term “NetworkType” to “NetworkUsage”