IEEE P802.11
Wireless LANs

| Corrected resolution for Clause 7.3.2.98.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | Philips Electronics Nederland BV Research High Tech Campus 37 5656 AE Eindhoven The Netherlands | +31-402-749-743 | dee.denteneer@philips.com |
| Guido R. Hiertz | Philips | Riedel Communications Riedel Technologie Park Uellendahler Straße 353 42109 Wuppertal Federal Republic of Germany | +49-202-292-9987 | hiertz@ieee.org |

Abstract

This document resolves an issue of submission 11-09-0320-00-000s where the resolution of Clause 7.3.2.98.2 was incorrect.

The following instructions describe changes to Clause 7.3.2.98.2 of draft D2.07 of the IEEE 802.11s amendment.

Instructions to editor:

In Clause

- MCCA Information field

change the text as follows

The MCCA Information field is a field of length 2 Octets that is used to provide information on the MCCAOP Reservations. The field consists of seven six subfields and its format is illustrated in Figure s37.

| B0-B7 | B4-B7B8-B11 | B8B12 | B9B13 | B10B14 | B15 | B11-15 |
| --- | --- | --- | --- | --- | --- | --- |
| MCCA Access Fraction | MCCA Access Fraction Limit | TX-RX Report Present | Broadcast Report Present | Interfering Report Present | Partial Report | Reserved |
| Bits: 48 | 4 | 1 | 1 | 1 | 41 | 4 |
| Figure s37 – MCCA Information field |  |  |  |  |  |  |

MCCA Access Fraction and MCCA Access Fraction Limit fields are both 4 bit unsigned number fields. The MCCA Access Fraction is an eight bit unsigned number field and the MCCA Access Fraction Limit field is a 4 bit unsigned number field. They denote a positive fraction of the mesh DTIM interval length expressed in units of (1/16). The MCCA Access Fraction denotes a positive fraction of the MCCA Access Fraction Limit expressed in units of 1/255. The MCCA Access Fraction Limit denotes a positive fraction of the mesh DTIM interval length expressed in units of (1/16). The MCCA Access Fraction field represents the current value of MCCA Access Fraction at the mesh STA rounded down (floor) to the nearest multiple of (1/161/255) of the MCCA Access Fraction Limit. The MCCA Access Fraction Limit field represents the maximum MCCA access fraction allowed at the mesh STA. This number is always a multiple of (1/16).

The TX-RX Report Present is a one bit field. It is set to 1 if a TX-RX Report is present in the MCCAOP Advertisement and set to 0 if no TX-RX Report is present. The Broadcast Report Present is a one bit field. It is set to 1 if a Broadcast TX-RX Report is present in the MCCAOP Advertisement and set to 0 if no Broadcast TX-RX Report is present. The Interfering Report Present is a one bit field. It is set to 1 if an Interfering Times Report is present in the MCCAOP Advertisement and set to 0 if no Interfering Times Report is present. The Partial Report field is a one bit field. It is set to 0 if the Advertisement element includes all known reservations and it is set to 1 if the Advertisement element does not include all known reservations.

References:

Word conversion of version D2.07 of the IEEE 802.11s draft amendment