IEEE P802.11
Wireless LANs

| LB-142 CID 5195 Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission addresses the CID 5195.  It conditions the AP behavior mandated in clause 11.7.1.2 on the proper MIB variable.

This document is based on 802.11u-d5.0.

11	MLME

11.7	DLS operation

11.7.1	DLS procedures

11.7.1.2	Setup procedure at the AP

Change the text as shown below:

Upon receipt of the DLS Request frame (step 1a in Figure 11-15), the AP  shall

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not Allowed in the BSS, if direct links are not allowed in the BSS, or for the AP with dot11SSPNInterfaceEnabled set to TRUE, with a result code of Not Allowed by SSP if the dot11NonAPStationAuthDls MIB variable in either of the non-AP STA's dot11InterworkingTable, if present, is true (step 2b in Figure 11-15).

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not Allowed in the BSS, if direct links are not allowed in the BSS (step 2b in Figure 11-15).

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not Present, if the destination STA is not present in the BSS (step 2b in Figure 11-15).

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not a STA, if the destination STA does not have QoS facility (step 2b in Figure 11-15).

- Send the DLS Request frame, with all fields having the same value as the DLS Request frame received by the AP, to the destination STA (step 1b in Figure 11-15).