IEEE P802.11
Wireless LANs

| Root Announcement Cleanup |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32771 | +1 407-562-4050 | Guenael.Strutt@motorola.com |
| Michael Bahr | Siemens Corporate Technology | Otto-Hahn-Ring 6 80200 München, Germany | +49 89/636-49926 | bahr et siemens dod com |

Abstract

The submission is a resolution of comment 1700.

Baseline: D2.07

Instructions to the TGs editor are shown in red.  When the instruction “modify (the text) as shown” is used, red indicates a deletion and blue indicates an insertion. Some changes are made just with formatting the text accordingly (colour & strikethrough/underline)!

| CID | Comment / Explanation | Recommended Change | Resolution Notes | What I did |
| --- | --- | --- | --- | --- |
| 1700 | The clause on the Root Announcement needs some overhaul. | Review and improve text. | What is "dot11MeshHWMPperrMinInterval" doing there? | I did some cleanup |

Modify 11B.10.9 as follows:

- Root Announcement (RANN)

This subclause describes the function, generation and processing of the Root Announcement element.

- Function

The RANN element, described in Error: Reference source not found, is used for announcing the presence of a mesh STA configured asRroot mesh STA using the proactive RANN mechanism. RANN elements are sent out periodically by the root mesh STA.

The RANN element propagates path metric information across the network so that each mesh STA can select a best metric path to the announced root mesh STA. This mechanism allows bidirectional trees to be built, using a robust procedure based on individually addressed frames initiated by the mesh STAs. This ensures that the root mesh STA is aware of all mesh STAs in the mesh.

Receiving mesh STAs shall propagate the RANN as described below.

- Conditions for generating and sending a RANN

A mesh STA sends out a RANN in the following cases:

Case A: Original transmission

All of the following conditions applyapplies:

- The mesh STA is configured as a root mesh STA using the proactive RANN mechanism (dot11MeshHWMProotMode = 4).

- The root mesh STA sent its previous RANN dot11MeshHWMPrannInterval TUs ago

The contents of a RANN element in Case A shall be as shown in Contents of a RANN element in Case A.

| - Contents of a RANN element in Case A |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the RANN element |
| Length | 17 |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | 0 |
| HWMP TTL | Maximum number of hops allowed for this element |
| Root mesh STA Address | MAC address of the root mesh STA |
| HWMP Sequence Number | Last used HWMP sequence number of the root mesh STA + 1 |
| Metric | iInitial value of active path selection metric |

Case B:	Propagation

All of the following conditions applyapplies:

- tThe mesh STA has valid forwarding information to a root mesh STA using the proactive RANN mechanism (dot11MeshHWMProotMode = 4)

- The mesh STA sent its previous RANN dot11MeshHWMPrannInterval TUs ago

- dot11MeshForwarding is set to 1

- tThe Time To Live field is greater than 1 – See Error: Reference source not found HWMP propagation

The contents of a RANN element in Case B shall be as shown in Contents of a RANN element in Case B.

| - Contents of a RANN element in Case B |  |
| --- | --- |
| Field | Value |
| ID | Value given in Error: Reference source not found for the RANN element |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| HWMP TTL | As received – 1 |
| Root mesh STA Address | As received |
| HWMP Sequence Number | As received |
| Metric | As receivedown link metric toward the transmitting mesh STA |

- RANN Reception

Received RANN elements are subject to certain acceptance criteria. Processing and actions taken depend on the content of the RANN and the forwarding information maintained by the receiving mesh STA.

See also Error: Reference source not found: General rules for message processing.

- Acceptance criteria

The RANN element shall not be accepted (and shall not be processed as described in Effect of receipt) if any of the following is true:

- The HWMP sequence number < previous HWMP sequence number from this originating root mesh STA

- (HWMP sequence number = previous HWMP sequence number) AND (updated path metric is worse than previous path metric)

Otherwise, the RANN element shall be accepted.

- Effect of receipt

The following applies only to a RANN element that was accepted according to the acceptance criteria in Acceptance criteria.

- The receiving mesh STA may initiate a PREQ/PREP exchange with the root mesh STA to set up or update a path to the root mesh STA. See Error: Reference source not found, case C.

- The receiving mesh STA may record the root mesh STA Aaddress, together with the HWMP sequence number, hopcount, and metric in order to assist in executing 11B.10.6.2  Case C.

The receiving mesh STA shall transmit a RANN as if the conditions defined in Conditions for generating and sending a RANN, Case B, are true.