IEEE P802.11
Wireless LANs

| LB142 QoS Map Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

This document resolves CIDs 5129, 5130, 5131, 5132, 5134, 5135, 5136, and 5137 from LB142.  These comments are against clause 7.3.2.87, the definition of the QoS Map information element.

Editorial Instructions: Change clause 7.3.2.87 as follows

- 7.3.2.87 QoS Map Set information element

The QoS Map Set information element is transmitted from an AP to a non-AP STA and provides the  mapping of higher-layer quality of service constructs to User Priorities defined by transmission of Data frames in this standard.  This IE maps the higher-layer priority from the DSCP field used with the Internet Protocol to User Priority as defined by this standard.  The QoS Map Set element is show n in See QoS Map Set element description..

| - QoS Map Set element description |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | DSCP Exception #1 (optional) | ... | DSCP Exception #n (optional) | UP 0 DSCP Range | UP 1 DSCP Range | UP 2 DSCP Range | ... | UP 7 DSCP Range |
| Octets: | 1 | 1 | 2 |  | 2 | 2 | 2 | 2 |  | 2 |

The Length field is set to 16+2×n, where n is the number of Exception elements in the QoS Map set.

DSCP Exception fields are optionally included in the QoS Map Set. If included, the QoS Map Set has a maximum of 21 exception fields. The format of the exception field is shown in See DSCP Exception format..

| - DSCP Exception format |  |  |
| --- | --- | --- |
|  | DSCP Value | User Priority |
| Octets: | 1 | 1 |

The DSCP value in the DSCP Exception field is in the range 0 to 63 inclusive, or 255; the User Priority value is between 0 and 7, inclusive.

When a non-AP STA begins transmission of a Data frame containing the Internet Protocol, it matches the DSCP field in the IP header to the corresponding DSCP value contained in this element.  The non-AP STA will first attempt to match the DSCP value to a DSCP exception field and uses the UP from the corresponding UP in the same DSCP exception field if successful; if no match is found then the non-AP STA attempts to match the DSCP field to a UP n DSCP Range field, and uses the n as the UP if successful; and otherwise uses a UP of 0.

Each DSCP Exception field has a different value of DSCP Value.

| - DSCP Range description |  |  |
| --- | --- | --- |
|  | DSCP Low Value | DSCP High Value |
| Octets: | 1 | 1 |

The QoS Map Set has a DSCP Range field corresponding to each of the 8 user priorities. The format of the range field is shown in See DSCP Range description.. The DSCP Range value is between 0 and 63 inclusive, or 255. For any IP packet to be transmitted by a non-AP STA whose DSCP is contained within the range (inclusive of the high and low values) for a given UP, then the packet is transmitted with the corresponding UP. IPv4 and IPv6 both have a DSCP field for communicating priority to lower protocol layers such as the one defined by this standard.

- The DSCP range for each user priority is non-overlapping.

- The DSCP High Value is greater than or equal to the DSCP Low Value.

- If the DSCP Range high value and low value are both equal to 255, then the corresponding UP is not used.

Any packet to be transmitted by a non-AP STA whose DSCP does not match any of the DSCP Ranges or DSCP Exception field is transmitted with UP=0.