IEEE P802.11
Wireless LANs

| Draft Reply Liaison to 3GPP CT1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| George Bumiller | Research In Motion | 39 Lakeview Terrace Ramsey, NJ 07446 USA | 1 201 327 1136 | g.bumiller@ieee.org |

Abstract

This reply liaison to 3GPP CT1 thanks them for the inclusion of the “generic container” in TS 24.234.

In reply to their questions to IEEE 802.11, three items are identified for their ANDSF (access network discovery and selection function) work:

Device Supported Services

HESSID

Regulatory Class

Reply Liaison to “LS on ANDSF Management Object”. C1-085530 (IEEE 802.11-09/0204r0)

To : CT1

Cc: CT, SA1, SA2

- Overall Description

IEEE 802.11 kindly thanks CT1 (C1-085530) for their liaison about activities related to ANDSF Management Object.

The ballot on IEEE 802.11u draft 5.0, “Interworking with External Networks”, recently passed the 75% approval rate.  It currently remains a draft, with the goal of final approval in 2010 as part of IEEE 802.11.  The draft will be available for purchase within the next several weeks from the online IEEE-SA store1.

As part of the draft, the “generic container” that will be available for UEs (non-Access point Stations - non-AP STAs) to obtain information on 3GPP networks, will use native GAS (Generic Advertising Service), which provides the most rapid response to inquiries from the UE.

IEEE 802.11 kindly thanks CT1 for their definition of the generic container for 3GPP use (TS 24.234 Annex A).

IEEE 802.11 kindly wishes to bring to the attention of CT1,  IEEE 802.11u elements which may be applicable to the ANDSF work that CT1 is engaged in. These two IEEE 802.11u elements are:

HESSID (homogenous ESS identifier): The Identifier for a collection of BSSes, within the same extended service set (ESS), in which the SSPN (Subscription Service Provider Network) or other external network reachable at one BSS, is reachable at all them. All BSSs identified by an HESSID are in the same mobility domain, if one is defined.

The HESSID provides a more useful identifier than SSID (Service Set Identifier) normally used to identify an access point (AP). It is a globally unique identifier, which in conjunction with the SSID, may be used to provide network identification for an SSPN.

[IEEE P802.11u/D5.0, February 2009]

Regulatory Class (for the OFDM PHY) is an index into a set of values for radio equipment sets of rules. It provides a means of identifying the requirements of national regulatory agencies which must be adhered to within countries or regions.

[IEEE Std 802.11-2007, Annex I and Annex J]

2. Actions

Device Supported Services: IEEE 802.11 kindly suggests that ANDSF capability be extended to include device supported services, so that the UE would be able to quickly determine those services that the network is able to provide.

HESSID: IEEE 802.11 kindly suggests that CT1 consider use of HESSID in appropriate areas within the several ANDSF-related technical specifications. It should be noted that the HESSID will only be available from access points (APs) that are updated to conform to IEEE 802.11u, after that amendment is issued.

Regulatory Class: IEEE 802.11 kindly suggests that CT1 may wish to make use of Regulatory Class, which provides a means of identifying those radio equipment values for the regulatory rules governing operation in specific countries and regions.

3. Date of Next IEEE 802.11 Meetings:

Interim		May 11-15, 2009	Montreal, PQ, Canada

Plenary	July 13-17, 2009 	San Francisco, CA, USA

Interim		Sept 21-25, 2009 	Waikoloa Village, HI, USA

1 http://standards.ieee.org, menu option “shop”

References: