IEEE P802.11
Wireless LANs

| TGac Functional Requirements and Evaluation Methodology Rev. 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Peter Loc | Ralink Technology | 20833 Stevens Creek Blvd, Suite 200., Cupertino CA 95014 | 408 807-0868 | peterloc@gmail.com |
| Minho Cheong | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5635 | minho@etri.re.kr |

ABSTRACT

This document describes the functional requirements and the evaluation methodology for TGac. These requirements are derived from the document “11-08-0807-04-0vht-below-6-ghz-par-nescom-form-plus-5cs”. All proposals submitted in response to the IEEE802.11 TGac call for proposal must address the functional requirements that are shown as mandatory in this document.

# Contributors

This will grow to reflect those providing explicit contributions / review comments of this document. (Please let Peter and Minho know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |
| Robert Stacey | Intel Corporation |  |  | robert.j.stacey@intel.com |
| Vinko Erceg | Broadcom |  |  | verceg@broadcom.com |
| Santosh Abraham | Qualcomm Inc. |  |  | sabraham@qualcomm.com |
| Allan Zhu | Samsung |  |  | c.zhu@samsung.com |
| Kai Shi | Atheros Comm. Inc. |  |  | kai.shi@atheros.com |
| Yashusi Takadori | NTT |  |  | takatori,yasushi@lab.ntt.co.jp |
| Darwin Engwer | Self |  |  | evoles_2001@yahoo.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| 0304r0 | Peter and Minho initial contribution on Functional Requirements | 13 March 2009 |
| R0 | Evaluation Methodology and Simulation Scenario are included | 10 April 2009 |
| R1 | Revised by conference call discussions on April 9 and April 23. | 09 May 2009 |

# 1. Overview

This document specifies the functional requirements for TGac as stated in the VHT below 6 GHz PAR Plus 5C’s. The emphasis is on the following aspects of TGac devices

- System performance

- Backward compatibility with 802.11a/n devices operating in 5 GHz

- Coexistence with 802.11a/n devices operating in 5 GHz

- Support of mobile devices

- Enhanced power saving

- OBSS operation

- Compliance to PAR

This document also specifies the evaluation methodology for TGac devices.

# 2. Functional Requirements

# 2.1 System Performance

2.1.1 Supporting band

TGac R1 - TGac devices shall operate in 5GHz frequency band.

2.1.2 Multi-STA throughput measured at the MAC SAP to be at least 1 Gbps.

TGac R2 – The TGac amendment shall demonstrate a mode of operation that supports an aggregate throughput of at least 1 Gbps at the top of the MAC data service access points (MAC SAPs) utilizing no more than 80 MHz of channel bandwidth in 5GHz band.

A typical test scenario may include multiple STAs that are simultaneously and actively communicating with an AP in 5 GHz band and the measured throughput at the MAC SAP of the AP exceeds 1 Gbps. Note that the PAR gives an example of an AP simultaneously actively-communicating with 3 STAs in the explanatory note. However, the number of STAs may actually be 2 or more in a test set up.

2.1.3 Single-STA throughput measured at the MAC SAP to be at least 500 Mbps.

TGac R3 – The TGac amendment shall demonstrate a mode of operation that supports a throughput of at least 500 Mbps at the top of the MAC data service access point (MAC SAP) utilizing no more than 80 MHz of channel bandwidth in 5GHz band.

A typical test scenario includes a single STA actively communicating with an AP in 5 GHz band and the measured throughput at the MAC SAP of either the AP or STA exceeds 500 Mbps .

2.1.4 Spectrum efficiency

TGac R4 - The TGac amendment shall demonstrate a mode of operation that achieves a spectral efficiency of at least 12 bps/Hz for the PSDU.

2.1.5 802.11e QoS support

TGac R5 - The proposal shall support the QoS enhancements of the IEEE802.11-2007 within a TGac STA.

2.1.6 Control of support for 802.11a/n STA from TGac AP

TGac R6 – TGac AP can be configured to reject or accept associations from 802.11a/n STAs.

## 2.2 Backward Compatibility

2.2.1 802.11a backward compatibility

TGac R7- TGac devices shall ensure backward compatibility with IEEE802.11a devices operating in the 5 GHz frequency band.

2.2.2 802.11n backward compatibility

TGac R8- TGac devices shall ensure backward compatibility with IEEE802.11n devices operating in the 5 GHz frequency band.

2.3 Coexistence

TGac R9 – The TGac amendment shall provide a mechanism to enable coexistence and spectrum sharing with IEEE802.11a/n devices operating in the same frequency band.

# 2.4 Mobile Devices

TGac R10 – TGac mobile devices are not required to support requirement R3 (500 Mbps). However, they shall demonstrate a mode of operation that achieves a minimum throughput of 100 Mbps as measured at their MAC SAPs.

# 2.5 Enhanced Power Saving

TGac R11 – The TGac amendment shall provide an enhanced power saving mechanism that results in a lower power consumption than typical 802.11n devices.

# 2.6 OBSS Operation

TGac R12 – TGac devices shall provide a mechanism to enable fair channel access and bandwidth sharing with other devices operating in OBSS. For example, when TGac devices are operating with 802.11a/n and 802.11ac devices in the same, adjacent or overlapping channels, they shall not severely affect the throughput of those devices.

# 2.7 Compliance to PAR

TGac R13 - The proposal complies with the PAR and 5 Criteria [1].

# 3. Evaluation Methodology

The evaluation methodology defines PHY performance, conditions for PAR compliance and a limited set of simulation scenarios and comparison criteria for TGac evaluatation.

As TGac agreed on the approach outlined in the 802.11/09/0376r1, the evaluation methodology for TGac can be build up based on 802.11n one by some modifications.

# 3.1 PHY Performance

3.1.1 PHY channel model

Channel models defined in 802.11n channel model document [8] shall be used. Channel model G for corridor environments will be newly included and AoA and AoD diversity are also considered for TGac [11].

3.1.2 PHY impairments

PHY impairments are updated from ones desribed in 802.11n comparison criteria document [7].

| Number | Name | Definition | Comments |
| --- | --- | --- | --- |
| IM1 | PA non-linearity | Simulation should be run at an oversampling rate of at least 4x. To perform convolution of the 4x oversampled transmit waveform with the channel, the channel may be resampled by rounding each channel tap time value to the nearest integer multiple of a sample interval of the oversampled transmit waveform. Use RAPP power amplifier model as specified in document 00/294 with p = 3. Calculate backoff as the output power backoff from full saturation: PA Backoff = 10 log10(Average TX Power/Psat). Total TX power shall be limited to no more than 17 dBm. Disclose: (a) EIRP and how it was calculated, (b) PA Backoff, and (c) Psat per PA. Note: the intent of this IM is to allow different proposals to choose different output power operating points. Note: the value Psat = 25dBm is recommended. | Added comments for higher sampling rate for channel |
| IM2 | Carrier frequency offset | Single-user simulations for all comparisons except Offset Compensation shall be run using a fixed carrier frequency offset of –13.675 ppm at the receiver, relative to the transmitter. The symbol clock shall have the same relative offset as the carrier frequency offset. Simulations shall include timing acquisition on a per-packet basis. Multi-user simulations for all comparisons except offset compensation shall be run using a fixed carrier frequency offset selected from the array [N(1) ,N(2),……,N(16) ], relative to the transmitter, where N(j) corresponds to the frequency offset of the j-th client and is randomly chosen from [-20,20] ppm with a uniform distribution. | Added a set of possible offsets to be used for several STAs. 802.11n specified a single offset of -13.67 ppm |
| IM3 | Phase noise | The phase noise will be specified with a pole-zero model. PSD(f)=PSD(0)[1+(ffz)2][1+(ffp)2] PSD(0) = -100 dBc/Hz pole frequency fp = 250 kHz zero frequency fz = 7905.7 kHz Note, this model results in PSD(infinity) = -130 dBc/Hz Note, this impairment is modeled at both transmitter and receiver. | Unchanged from 802.11n |
| IM4 | Noise figure | Input referred total noise figure from antenna to output of the A/D will be 10dB. | Unchanged from 802.11n |
| IM5 | Antenna Configuration | The TGn antenna configuration at both ends of the radio link shall be a uniform linear array of isotropic antennas with separation of one-half wavelength, with an antenna coupling coefficient of zero. The TGac antennas can be assumed to either be all vertically polarized or a mix of vertical and horizontal polarizations or dual polarization at ±45 degree, as specified in the TGac channel model addendum document [11] | Mix of vertically and horizontally polarized antennas or dual polarization at ±45 degree is also considered for TGac devices |
| IM6 | Fluoroscent Light Effects | The fluoroscent light effects specifed in the TGn Channel model shall not be considered for the simulation scenarios. |  |

3.1.3 Comparison criteria

- PER vs. SNR curves

- all MCS’s

- Simulate all of channel models

- Simulation must include:

- updated PHY impairments

- timing acquisition on a per-packet basis

- preamble detection on a per-packet basis

# 3.2 Traffic Models

TGac evaluation shall consider traffic models defined 802.11n usage model documents [3] including high-quality videos for VHT defined in [12] and high-speed file transfer.

| Num. | Application | Offered Load (Mbps) | Protocol | MSDU Size (B) | Max. PLR | Max. Delay (ms) | Source [ref] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Lightly-compressed video | 150 | UDP | TBD | 10^-7 | 10 | Motion JPEG2000 |
| 2 | Lightly-compressed video | 200 | UDP | TBD | 10^-7 / 8 | 20 | H.264 |
| 3 | Compressed video | 50Mbps | UDP | TBD | 10^-7 | 20 | Blu-rayTM |
| 4 | Compressed video | 20Mbps | UDP | 1500 | 3x10^-7 | 20 | HD-MPEG2 |
| 5 | VoD control channel | 0.06 | UDP | 64 | 10^-2 | 100 | Guess |
| 6 | Video Conf | 0.128 - 2 | UDP | 512 | 10^-4 | 100 | 1 |
| 7 | Internet Streaming video/audio | 0.1 – 4 | UDP | 512 | 10^-4 | 200 | 1 |
| 8 | Internet Streaming audio | 0.064~0.256 | UDP | 418 | 10^-4 | 200 | Group guess |
| 9 | VoIP | 0.096 | UDP | 120 | 5% | 30 | ITU-T G.114 300ms round-trip delay G.711 Codec |
| 10 | Reserved |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |
| 12 | MP3 Audio Other formats are taking over (AAC/MPEG-4, OggVorbis, etc) | 0.064 – 0.32 | UDP | 418 | 10^-4 | 200 | 1 |
| 13 | Reserved |  |  |  |  |  |  |
| 14 | Content download (photo camera) | minimum 10 | TCP | 1500 | N/A |  | Corresponds to USB and flash speed |
| 15 | Internet File transfer (email, web, chat) | Minimum 1 | TCP | 300 | N/A |  |  |
| 16 | Local File transfer, printing | minimum 30 | TCP | 1500 | N/A |  | Aps guess |
| 17 | Reserved |  |  |  |  |  |  |
| 18 | Reserved |  |  |  |  |  |  |
| 19 | Reserved |  |  |  |  |  |  |
| 20 | Video phone | 0.5 | UDP | 512 | 10^-2 | 100 | Aps guess |
| 21 | Remote user interface (X11, Terminal Server Client) (remote display/keyboard/mouse) | 0.5-1.5 (peak) | UDP | 700 | N/A | 100 | 11-03-0696r0 |
| 22 | Clicking on web link | 0.256 | TCP | 64 | N/A |  | desribed in 11-03-0802r23 (pp. 27) |

Ref.) High-quality video service requirements for VHT [12]

| Video Compression |  | Description | Rate, Mbps | Packet Error Rate | Jitter, ms | Delay,ms |
| --- | --- | --- | --- | --- | --- | --- |
| Uncompressed | 720p | (RGB): 1280x720 pixels, 24bits/pixels,60frames/s | 1300 | 10^-8 | 5 | 5 |
|  | 1080i | (RGB): 1920x1080/2pixels, 24bits/pixels,60frames/s | 1300 | 10^-8 |  |  |
|  | 1080p | (YCrCb): 1920x1080 pixels, 12bits/pixels,60frames/s | 1500 | 10^-8 |  |  |
|  | 1080p | (RGB): 1920x1080 pixels, 24bits/pixels,60frames/s | 3000 | 10^-8 |  |  |
| Lightly Compressed |  | Motion JPEG2000 H.264 | 150 70 - 200 | 10^-7 10^-7 / 8 | 10 20 | 10 20 |
| Compressed |  | Blu-ray™ | 50 | 10^-7 | 20 | 20 |
|  |  | HD MPEG2 | 20 | 3x10^-7 | 20 | 20 |

# 4. Simulation Scenarios

Simulation scenarios for TGac evaluation are summarized as:

| Scenario Number | Purpose | Note |
| --- | --- | --- |
| 1 | Test compliance to PAR. | Single STA 500Mbps throughput at the MAC SAP |
| 2 | Test compliance to PAR. | Multi STA 1Gbps throughput at the MAC SAP |
| 3 | In-home entertainment application. | Multiple flows with varied QoS requirements Includes several lightly-compressed video flows. Aligns with Category 1 and Category 2 applications. |
| 4 | Enterprise network | Stress test for TGac operation. Scenario with large number of flows. Aligns with Category 2 and Category 3 applications. |

# 4.1 Test for Compliance to PAR

4.1.1 Point-to-point link test (scenario #1)

Synthetic test case to demonstrate single STA 500Mbps throughput at the MAC SAP.

This scenario is derived from scenario #19 defind in 802.11n usage model document.

Two stations

One TGac AP is source.

One TGac STA is sink.

Traffic from AP to STA

Protocol: UDP

Offered load : infinite

MSDU size: 1500

PHY channel model

Model B

Locations of stations

Fixed locations: (0,0) meters for AP and (0,5) meters for STA

Meet requirements in [functional requirements Sections 2.1.3]

4.1.2 Point-to-multi-point link test (scenario #2)

Synthetic test case to demonstrate multi STA aggregated 1Gbps throughput at the MAC SAP.

This scenario is also derived from scenario #19 defind in 802.11n usage model document.

Number of stations (AP + STAs): at least 3

One TGac AP is source

Number of TGac STAs which are sinks : at least 2

Traffic from AP to STA

Protocol: UDP

Offered load : infinite

MSDU size: 1500

PHY channel model

Model D

Locations of stations

Fixed locations

Meet requirements in [functional requirements Sections 2.1.2]

| Flow No. | Source | Source Location (meters) | Sink | Channel Model | Sink Location (meters) | Application | Bit Rate (Mbps) | Rate Distribution | MSDU Size (B) | Max Delay (ms) | PLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Downlink Flows |  |  |  |  |  |  |  |  |  |  |
| 1 | AP | (0,0) | STA1 | D | (0,10) | Data | minimum 1000/N Mbps | Infinite Backlog , UDP | 1500 | N/A | 10^-7 |
| 2 | AP | (0,0) | STA2 | D | (10,0) | Data | minimum 1000/N Mbps | Infinite Backlog , UDP | 1500 | N/A | 10^-7 |
|  | : : | : : |  | : : | : : | : : | : : | : : | : : | : : | : : |
| N | AP | (0,0) | STAN | D | (-6,-8) | Data | minimum 1000/N Mbps | Infinite Backlog , UDP | 1500 | N/A | 10^-7 |

# 4.2 In-Home Entertainment Application (scenario #3)

Test case to demonstrate in-home entertainment application with multiple flows with varied QoS requirements including lightly-compressed video.

This scenario is derived from scenario #1 defind in 802.11n usage model document and modified in order to consider usage model 2a and 2b (PVR’s in residential) defined in [12] as well.

11 stations

One TGac AP is source and sink.

10 STAs are souces and sinks. (STA2 is reserved)

Traffic from AP to STAs

Lightly-compressed video, compressed video, Internet file, Internet streaming video, MP3 audio

Traffic from STAs to AP

Compressed video, VoD control channel, video console, Internet entertainment

Traffic STAs to STAs

Local file transfer, video phone, controller to console

PHY channel for each link

Channel model type applied : Model C

Channel model break point between LOS and NLOS: unchanged from 802.11n

Shadowing term is 0dB

In 802.11n channel model document [8], shadow fading std. dev. is specified for each channel model between 3dB and 6dB at page 7.

But,

in 802.11n usage model document [3], shadowing term applied to all the 802.11n simulation scenarios is set to 0dB for generating a channel realization at page 22.

Locations of stations

Fixed locations (unchagend from 802.11n scenario #1)

Meet requirements specified in PLR and Max. Delay per each flow

Total throughput condition

Total throughput (about minimum 386Mbps) is not a heavy burden compared to that of scenario #1 defined in 802.11n usage model document (about 64Mbps).

| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) | Channel Model | Application | Bit Rate (Mbps) | Rate Distribution | MSDU Size (B) | Max. Delay (ms) | PLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Downlink Flows |  |  |  |  |  |  |  |  |  |  |
| 1 | AP | (0,0) | STA1 | (0,5) |  | LC Video | 150.00 | Constant, UDP | 1500 | 10 | 10^-7 |
| 2 | AP | (0,0) | STA3 | (5,0) |  | Blu-rayTM | 50.00 | Constant, UDP | 1500 | 20 | 10^-7 |
| 3 | AP | (0,0) | STA4 | (-7,7) |  | Blu-rayTM | 50.00 | Constant, UDP | 1500 | 20 | 10^-7 |
| 4 | AP | (0,0) | STA4 | (-7,7) |  | Internet file | Minimum 1.00 | TCP | 300 | Inf. | N/A |
| 5 | AP | (0,0) | STA7 | (20,0) |  | VoD control channel | 0.06 | Constant, UDP | 64 | 100 | 10^-2 |
| 6 | AP | (0,0) | STA8 | (0,20) |  | VoD control channel | 0.06 | Constant, UDP | 64 | 100 | 10^-2 |
| 7 | AP | (0,0) | STA9 | (0,-20) |  | VoD control channel | 0.06 | Constant, UDP | 64 | 100 | 10^-2 |
| 8 | AP | (0,0) | STA10 | (10,10) |  | Internet Streaming video | 2.00 | UDP | 512 | 200 | 10^-4 |
| 9 | AP | (0,0) | STA11 | (10,5) |  | MP3 audio | 0.13 | UDP | 418 | 200 | 10^-4 |
|  | Uplink Flows |  |  |  |  |  |  |  |  |  |  |
| 10 | STA1 | (0,5) | AP | (0,0) |  | VoD control channel | 0.06 | Constant, UDP | 64 | 100 | 10^-2 |
| 11 | STA3 | (5,0) | AP | (0,0) |  | VoD control channel | 0.06 | Constant, UDP | 64 | 100 | 10^-2 |
| 12 | STA7 | (20,0) | AP | (0,0) |  | Blu-rayTM | 50.00 | Constant, UDP | 1500 | 20 | 10^-7 |
| 13 | STA8 | (0,20) | AP | (0,0) |  | Blu-rayTM | 50.00 | Constant, UDP | 1500 | 20 | 10^-7 |
| 14 | STA9 | (0,-20) | AP | (0,0) |  | HD MPEG2 | 20.00 | Constant, UDP | 1500 | 20 | 3x10^-7 |
| 15 | STA10 | (10,10) | AP | (0,0) |  | Video console + Internet entertainment | 1.00 | Constant, UDP | 512 | 50 | 10^-4 |
|  | STA to STA Flows |  |  |  |  |  |  |  |  |  |  |
| 16 | STA4 | (-7,7) | STA10 | (10,10) |  | Local file transfer | Minimum 10.00 | TCP | 300 | Inf. | N/A |
| 17 | STA5 | (-15,0) | STA6 | (0,-15) |  | Video Phone | 0.50 | Constant, UDP | 512 | 100 | 10^-2 |
| 18 | STA6 | (0,-15) | STA5 | (-15,0) |  | Video Phone | 0.50 | Constant, UDP | 512 | 100 | 10^-2 |
| 19 | STA11 | (10,5) | STA10 | (10,10) |  | Controller to Console | 0.50 | Constant, UDP | 50 | 16 | 10^-4 |
|  |  |  |  |  |  | Total Thruput | Minimum 385.93 |  |  |  |  |

Note) STA2 is reserved.

# 4.3 Enterprise Network (scenario #4)

Test case to demonstrate enterprise network application with large number of flows (over 40).

This scenario is derived from scenario #4 defind in 802.11n usage model document.

31 stations

One TGac AP is source and sink

30 STAs are souces and sinks.

Traffic from AP to STAs

Internet file, video conferencing, Internet streaming video + MP3, local file transfer

VoIP

Traffic from STAs to AP

Clicking on web link, file upload, video conferencing, VoIP

PHY channel for each link

Channel model type applied : Model D

Channel model break point between LOS and NLOS : unchanged from 802.11n

Shadowing term is 0dB (unchanged from 802.11n scenarios)

In 802.11n channel model document [8], shadow fading std. dev. is specified for each channel model between 3dB and 6dB at page 7.

But in 802.11n usage model document [3], shadowing term applied to all the 802.11n simulation scenarios is set to 0dB for generating a channel realization at page 22.

Locations of stations

Fixed locations (unchagend from 802.11n scenario #4)

Meet requirements specified in PLR and Max. Delay per each flow

Total throughput condition

Total throughput (minimum about 460Mbps) is not a heavy burden compared to that of scenario #4 defined in 802.11n usage model document (about 460Mbps).

| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) | Channel | Application | Bit Rate (Mbps) | Rate Distribution | MSDU Size (B) | Max. Delay (ms) | PLR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Downlink Flows |  |  |  |  |  |  |  |  |  |  |
| 1 | AP | (0,0) | STA1 | (5,-9.5) |  | Internet file | Minimum 1.00 | TCP | 300 | Inf. | N/A |
| 2 | AP | (0,0) | STA2 | (3.5,7.5) |  | Internet file | Minimum 1.00 | TCP | 300 | Inf. | N/A |
| 3 | AP | (0,0) | STA3 | (7.5,-9.5) |  | Internet file | Minimum 1.00 | TCP | 300 | Inf. | N/A |
| 4 | AP | (0,0) | STA4 | (-4,0.5) |  | Internet file | Minimum 1.00 | TCP | 300 | Inf. | N/A |
| 5 | AP | (0,0) | STA5 | (-1.5,6) |  | Internet file | Minimum 1.00 | TCP | 300 | Inf. | N/A |
| 6 | AP | (0,0) | STA6 | (-5.5,4.5) |  | Internet file, downloading large email attachments | Minimum 10.00 | TCP | 300 | Inf. | N/A |
| 7 | AP | (0,0) | STA7 | (-9,-5) |  | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 8 | AP | (0,0) | STA8 | (-8.5,8.5) |  | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 9 | AP | (0,0) | STA9 | (7,-7.5) |  | Internet Streaming video + MP3 audio | 2.00 | UDP | 512 | 200 | 10^-4 |
| 10 | AP | (0,0) | STA10 | (-3,0.5) |  | Internet Streaming video + MP3 audio | 2.00 | UDP | 512 | 200 | 10^-4 |
| 11 | AP | (0,0) | STA11 | (-0.5,8) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 12 | AP | (0,0) | STA12 | (7,7) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 13 | AP | (0,0) | STA13 | (-4,-4) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 14 | AP | (0,0) | STA14 | (7.5,-1) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 15 | AP | (0,0) | STA15 | (3,-0.5) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 16 | AP | (0,0) | STA16 | (8,-6) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 17 | AP | (0,0) | STA17 | (0,-7.5) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 18 | AP | (0,0) | STA18 | (10,0.5) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 19 | AP | (0,0) | STA19 | (-2.5,-4.5) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 20 | AP | (0,0) | STA20 | (0.5,-2) |  | Local File transfer | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 21 | AP | (0,0) | STA25 | (3.5,-5) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 22 | AP | (0,0) | STA26 | (9,9.5) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 23 | AP | (0,0) | STA27 | (-6,2.5) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 24 | AP | (0,0) | STA28 | (-8,-5.5) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 25 | AP | (0,0) | STA29 | (1.5,3.5) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 26 | AP | (0,0) | STA30 | (9.5,3.5) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| Flow No. | Source | Source Location (meters) | Sink | Sink Location (meters) |  | Application | Bit Rate (Mbps) | Rate Distribution | MSDU Size (B) | Max Delay (ms) | PLR |
|  | Uplink Flows |  |  |  |  |  |  |  |  |  |  |
| 27 | STA1 | (5,-9.5) | AP | (0,0) |  | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 28 | STA2 | (3.5,7.5) | AP | (0,0) |  | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 29 | STA3 | (7.5,-9.5) | AP | (0,0) |  | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 30 | STA4 | (-4,5,0.5) | AP | (0,0) |  | File Upload | Minimum 5 | TCP | 1000 | Inf. | N/A |
| 31 | STA5 | (-1.5,6) | AP | (0,0) |  | File Upload | Minimum 10 | TCP | 1500 | Inf. | N/A |
| 32 | STA6 | (-5.5,4.5) | AP | (0,0) |  | Clicking on web link | 0.256 | TCP | 64 | Inf. | N/A |
| 33 | STA7 | (-9,-5) | AP | (0,0) |  | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 34 | STA8 | (-8.5,8.5) | AP | (0,0) |  | Video conferencing | 1.00 | Constant, UDP | 512 | 100 | 10^-4 |
| 35 | STA21 | (-6.5,-3) | AP | (0,0) |  | File Upload | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 36 | STA22 | (0,-4.5) | AP | (0,0) |  | File Upload | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 37 | STA23 | (-1.5,7) | AP | (0,0) |  | File Upload | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 38 | STA24 | (3,2.5) | AP | (0,0) |  | File Upload | Minimum 30Mbps | TCP | 1500 | Inf. | N/A |
| 39 | STA25 | (3.5,-5) | AP | (0,0) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 40 | STA26 | (9,9.5) | AP | (0,0) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 41 | STA27 | (-6,2.5) | AP | (0,0) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 42 | STA28 | (-8,-5.5) | AP | (0,0) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 43 | STA29 | (1.5,3.5) | AP | (0,0) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
| 44 | STA30 | (9.5,3.5) | AP | (0,0) |  | VoIP | 0.10 | Constant, UDP | 120 | 30 | 5% |
|  |  |  |  |  |  | Total Measured Thruput | Minimum 460.22Mbps |  |  |  |  |

# 4.4 Comparison Criteria

MAC SAP throughput (goodput)

Aggregated throughput (downlink + uplink + STA-to-STA)

Throughput of each flow in the simulation scenario

Packet Loss Rate for each flow in the simulation scenario

Lost packets include

Packets that exceed maximum latency requirement

Packets that are dropped after exceeding maximum number of re-tries

Note)   TGac proposals shall explain how it meets requirement R12 (OBSS operation).

# 4.5 Results Presentation Format

e.g.) for scenario #3

| Flow No. | Source | Sink | Application | Bit Rate (Mbps) | Rate Distribution | Max Delay (ms) | PLR | Pass/Fail |  | Latency Compliant (%) | Measured Throughput (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Downlink Flows |  |  |  |  |  |  |  |  |  |  |
| 1 | AP | STA1 | LC Video | 150.00 | Constant, UDP | 10 |  |  |  |  |  |
| 2 | AP | STA3 | Blu-rayTM | 50.00 | Constant, UDP | 20 |  |  |  |  |  |
| 3 | AP | STA4 | Blu-rayTM | 50.00 | Constant, UDP | 20 |  |  |  |  |  |
| 4 | AP | STA4 | Internet file | Minimum 1.00 | TCP | Inf. |  |  |  |  |  |
| 5 | AP | STA7 | VoD control channel | 0.06 | Constant, UDP | 100 |  |  |  |  |  |
| 6 | AP | STA8 | VoD control channel | 0.06 | Constant, UDP | 100 |  |  |  |  |  |
| 7 | AP | STA9 | VoD control channel | 0.06 | Constant, UDP | 100 |  |  |  |  |  |
| 8 | AP | STA10 | Internet Streaming video | 2.00 | UDP | 200 |  |  |  |  |  |
| 9 | AP | STA11 | MP3 audio | 0.13 | UDP | 200 |  |  |  |  |  |
|  | Uplink Flows |  |  |  |  |  |  |  |  |  |  |
| 10 | STA1 | AP | VoD control channel | 0.06 | Constant, UDP | 100 |  |  |  |  |  |
| 11 | STA3 | AP | VoD control channel | 0.06 | Constant, UDP | 100 |  |  |  |  |  |
| 12 | STA7 | AP | Blu-rayTM | 50.00 | Constant, UDP | 20 |  |  |  |  |  |
| 13 | STA8 | AP | Blu-rayTM | 50.00 | Constant, UDP | 20 |  |  |  |  |  |
| 14 | STA9 | AP | HD MPEG2 | 20.00 | Constant, UDP | 20 |  |  |  |  |  |
| 15 | STA10 | AP | Video console + Internet entertainment | 1.00 | Constant, UDP | 50 |  |  |  |  |  |
|  | STA to STA Flows |  |  |  |  |  |  |  |  |  |  |
| 16 | STA4 | STA10 | Local file transfer | Minimum 10.00 | TCP | Inf. |  |  |  |  |  |
| 17 | STA5 | STA6 | Video Phone | 0.50 | Constant, UDP | 100 |  |  |  |  |  |
| 18 | STA6 | STA5 | Video Phone | 0.50 | Constant, UDP | 100 |  |  |  |  |  |
| 19 | STA11 | STA10 | Controller to Console | 0.50 | Constant, UDP | 16 |  |  |  |  |  |
|  |  |  | Total Given Thruput | Minimum 385.93 |  |  |  |  |  | Total Measured Thruput | 0.00 |

# 5. Summary of Functional Requirements

| Requirement Number | Description | Requirement Statement | Status of Requirement | Notes (informative) |
| --- | --- | --- | --- | --- |
| R1 | Operational in 5 GHz | TGac devices shall operate in 5GHz frequency band. |  |  |
| R2 | Maximum multi-STA throughput | Support at least 1 Gbps at the top of the MAC SAP. |  |  |
| R3 | Maximum single link throughput | Support 500 Mbps throughput at the top of the MAC SAP |  |  |
| R4 | Spectrum efficiency | The TGac amendment shall demonstrate a mode of operation that achieves a spectral efficiency of at least 12 bps/Hz for the PSDU. |  |  |
| R5 | Support 802.11 QoS Enhancements support | The proposal shall support the QoS enhancements of the IEEE802.11-2007 within a TGac STA. |  |  |
| R6 | Control of support for legacy STA from TGac AP | A TGac AP can be configured to reject or accept associations from 802.11a/n STAs |  |  |
| R7 | 802.11a backward compatibility | TGac devices shall ensure backward compatibility with IEEE802.11a devices operating in the 5 GHz frequency band. . |  |  |
| R8 | 802.11n backward compatibility | TGac devices shall ensure backward compatibility with IEEE802.11n devices operating in the 5 GHz frequency band |  |  |
| R9 | Coexistence with 802.11a/n devices | TGac devices provide mechanisms to enable coexistence and spectrum sharing with IEEE802.11a/n devices operating in the same frequency band. |  |  |
| R10 | Support of mobile devices | TGac mobile devices are not required to support requirement R3 (500 Mbps). However, they shall demonstrate a mode of operation that achieves a minimum throughput of 100 Mbps as measured at their MAC SAPs |  |  |
| R11 | Enhanced power saving | The TGac amendment shall provide an enhanced power saving mechanism that results in a lower power consumption than typical 802.11n devices. |  |  |
| R12 | OBSS operation | TGac devices shall provide a mechanism to enable fair channel access and bandwidth sharing with other devices operating in OBSS. |  |  |
| R13 | Compliance to PAR | The proposal complies with the PAR and 5 Criteria [1]. |  |  |

# 6. References

- 11-08-0807-04-0vht-below-6-ghz-par-nescom-form-plus-5cs

- 11-03-0813-13-000n-functional-requirements

- 11-03-0802-23-000n-usage-models

- 11-08-0307-01-0vht-on-the-feasibility-of-1gbps-for-various-mac-phy-architectures

- 11-08-0535-00-0vht-phy-and-mac-throughput-analysis-with-80-mhz-for-vht-below-6-ghz

- 11-09-0071-01-00ac-discussion-on-functional-requirements

- 11-03-0814-31-000n-comparison-criteria

- 11-03-0940-04-000n-tgn-channel-models

- 11-09-0059-04-00ac-tgac-802.11ac-proposed-selection-procedure

- 11-09-0376-01-00ac-proposal-for-tgac-evaluation-methodology

- 11-09-0308-05-00ac-tgac-channel-model-addendum-document

- 11-09-0161-02-00ac-802.11ac-usage-model-document

Note, this corresponds to a loss of a 1024B MSDU per hour.   The TS PDU PLR is higher than this.  It is not known what is the effect to the decoder of giving it burst packet losses.

Note, a PLR of 10^-7 will not be measurable in our simulation technologies.